Agency Name: Department of Health and Environmental Control

Statutory Authority: 48-1-10 et seq.

Document Number: 4160

Proposed in State Register Volume and Issue: 34/11

Final in State Register Volume and Issue: 35/2

Status: Final

Subject: Prevention of Significant Deterioration, Nonattainment New Source Review (NSR), and Air Pollution Control Regulations and Standards

History: 4160

By	Date	Action Description	Jt. Res. No.	Expiration Date

-	11/26/2010	Proposed Reg Published in SR

-	02/25/2011	Final to comply with Federal

Law, exempt GA review

-	02/25/2011	Effective Date unless otherwise

provided for in the Regulation

Document No. 4160

DEPARTMENT OF HEALTH AND ENVIRONMENTAL CONTROL

CHAPTER 61

Statutory Authority: 1976 Code Sections 48-1-10 et seq.

61-62. Air Pollution Control Regulations and Standards

61-62.5. Standard No. 7, Prevention of Significant Deterioration

61-62.5. Standard No. 7.1, Nonattainment New Source Review (NSR)

Synopsis:

The Department has amended R. 61-62.5, Standard No. 7, Prevention of Significant Deterioration; R. 61-62.5, Standard No. 7.1, Nonattainment New Source Review (NSR); and the SIP to incorporate the provisions of the federal rule entitled “Final Rule To Implement the 8-Hour Ozone National Ambient Air Quality Standard” (70 FR 71612) also known as the “Phase 2 Ozone Rule,” and the federal rule entitled “Implementation of the New Source Review (NSR) Program for Particulate Matter Less Than 2.5 Micrometers (PM2.5)” (73 FR 28321), also known as the “NSR PM2.5 Implementation Rule NSR PM2.5 Implementation Rule.” The Department also added a nonsubstantive clarification to correct a scrivener’s error.

These regulations were promulgated to comply with federal law. A fiscal impact statement, assessment report, or legislative review are not required.

A Notice of Drafting for these proposed regulations was published in the State Register on July 23, 2010.

Discussion of Revisions:

SECTION CITATION/EXPLANATION OF CHANGE:

Regulation, 61-62.5, Standard No. 7, Prevention of Significant Deterioration

Regulation, 61-62.5, Standard No. 7, Prevention of Significant Deterioration, (b) Definitions, (44) “Regulated NSR Pollutant”:

The definition of “Regulated NSR Pollutant” was revised to include elaboration on which pollutants are precursors for criteria pollutants under the National Ambient Air Quality Standards, and specific instructions on how to calculate PM emissions when determining if emissions fall within the scope of this definition.

Regulation, 61-62.5, Standard No. 7, Prevention of Significant Deterioration, (b) Definitions, (49)(i) “Significant”:

The definition of “Significant” was revised to include new thresholds for PM2.5.

Regulation, 61-62.5, Standard No. 7, Prevention of Significant Deterioration, (i) Exemptions, (1)(xi):

This item was added to clarify what provisions apply to PSD applications received before July 15, 2008.

Regulation, 61-62.5, Standard No. 7, Prevention of Significant Deterioration, (i) Exemptions, (5)(i):

This item was modified by adding “or nitrogen oxides” to the footnote for Ozone in the list of de minimis air quality levels, so that a facility with a net increase of 100 tons per year or more of nitrogen oxides subject to PSD would be required to perform an ambient impact analysis including the gathering of ambient air quality data.

Regulation, 61-62.5, Standard No. 7.1, Nonattainment New Source Review (NSR)

Regulation, 61-62.5, Standard No. 7.1, Nonattainment New Source Review (NSR), (c) Definitions, (13) “Regulated NSR pollutant”:

The definition of “Regulated NSR pollutant” was modified in the introductory sentence by adding the words “of this regulation” for clarification to correct a nonsubstantive scrivener’s error. The definition is also modified by adding a list of precursors and the requirements for considering PM, including timing requirements for considering condensable emissions.

Regulation, 61-62.5, Standard No. 7.1, Nonattainment New Source Review (NSR), (c) Definitions, (14) “Significant”:

The definition of “Significant” was revised to include new thresholds for PM2.5.

Instructions:

Amend Regulation 61-62, Air Pollution Control Regulations and Standards, pursuant to each instruction provided below with the text of the amendments.

Text:

Regulation, 61-62.5, Standard No. 7, Prevention of Significant Deterioration

(b) Definitions, (44) “Regulated NSR Pollutant” shall be revised as follows:

(44) “Regulated NSR pollutant,” for purposes of this regulation, means the following:

(i) Any pollutant for which a national ambient air quality standard has been promulgated and any constituents or precursors for such pollutants identified by the Administrator. Precursors identified by the Administrator for purposes of NSR are the following:

(a) Volatile organic compounds and nitrogen oxides are precursors to ozone in all attainment and unclassifiable areas.

(b) Sulfur dioxide is a precursor to PM2.5 in all attainment and unclassifiable areas.

(c) Nitrogen oxides are presumed to be precursors to PM2.5 in all attainment and unclassifiable areas, unless the State demonstrates to the Administrator’s satisfaction or the EPA demonstrates that emissions of nitrogen oxides from sources in a specific area are not a significant contributor to that area’s ambient PM2.5 concentrations.

(d) Volatile organic compounds are presumed not to be precursors to PM 2.5 in any attainment or unclassifiable area, unless the State demonstrates to the Administrator’s satisfaction or the EPA demonstrates that emissions of volatile organic compounds from sources in a specific area are a significant contributor to that area’s ambient PM2.5 concentrations.

(ii) Any pollutant that is subject to any standard promulgated under section 111 of the Clean Air Act;

(iii) Any Class I or II substance subject to a standard promulgated under or established by title VI of the Clean Air Act;

(iv) Any pollutant that otherwise is subject to regulation under the Clean Air Act; except that any or all hazardous air pollutants either listed in section 112 of the Clean Air Act or added to the list pursuant to section 112(b)(2) of the Clean Air Act, which have not been delisted pursuant to section 112(b)(3) of the Clean Air Act, are not regulated NSR pollutants unless the listed hazardous air pollutant is also regulated as a constituent or precursor of a general pollutant listed under section 108 of the Clean Air Act;

(v) Reserved;

(vi) Particulate matter (PM) emissions, PM2.5 emissions and PM10 emissions shall include gaseous emissions from a source or activity which condense to form particulate matter at ambient temperatures. On or after January 1, 2011 (or any earlier date established in the upcoming rulemaking codifying test methods), such condensable particulate matter shall be accounted for in applicability determinations and in establishing emissions limitations for PM, PM2.5 and PM10 in PSD permits. Compliance with emissions limitations for PM, PM2.5 and PM10 issued prior to this date shall not be based on condensable particular matter unless required by the terms and conditions of the permit or the applicable implementation plan. Applicability determinations made prior to this date without accounting for condensable particular matter shall not be considered in violation of this section unless the applicable implementation plan required condensable particular matter to be included.

(b) Definitions, (49)(i) “Significant” shall be revised as follows:

(49)(i) “Significant” means, in reference to a net emissions increase or the potential of a source to emit any of the following pollutants, a rate of emissions that would equal or exceed any of the following rates:

Pollutant and Emissions Rate

Carbon monoxide: 100 tons per year (tpy)

Nitrogen oxides: 40 tpy

Sulfur dioxide: 40 tpy

Particulate matter:

25 tpy of particulate matter emissions;

15 tpy of PM10 emissions;

10 tpy of direct PM2.5 ;

40 tpy of sulfur dioxide emissions;

40 tpy of nitrogen oxide emissions unless demonstrated not to be a PM2.5 precursor under (b)44 of this section

Ozone: 40 tpy of volatile organic compounds or oxides of nitrogen

Lead: 0.6 tpy

Fluorides: 3 tpy

Sulfuric acid mist: 7 tpy

Hydrogen sulfide (H2S): 10 tpy

Total reduced sulfur (including H2S): 10 tpy

Reduced sulfur compounds (including H2S): 10 tpy

Municipal waste combustor organics (measured as total tetra- through octa-chlorinated dibenzo-p-dioxins and dibenzofurans): 3.2 x 10-6 megagrams per year (3.5 x 10-6 tons per year)

Municipal waste combustor metals (measured as particulate matter): 14 megagrams per year (15 tons per year)

Municipal waste combustor acid gases (measured as sulfur dioxide and hydrogen chloride): 36 megagrams per year (40 tons per year)

Municipal solid waste landfills emissions (measured as nonmethane organic compounds): 45 megagrams per year (50 tons per year)

(i) Exemptions, (5)(i) shall be revised as follows:

(i) The emissions increase of the pollutant from the new source or the net emissions increase of the pollutant from the modification would cause, in any area, air quality impacts less than the following amounts:

Carbon monoxide--575 µg/m3, 8-hour average;

Nitrogen dioxide--14 µg/m3, annual average;

Particulate matter--10 µg/m3 of PM10, 24-hour average;

Sulfur dioxide--13 µg/m3, 24-hour average;

Ozone;1

Lead--0.1 µg/m3, 3-month average;

Fluorides--0.25 µg/m3, 24-hour average;

Total reduced sulfur--10 µg/m3, 1-hour average;

Hydrogen sulfide--0.2 µg/m3, 1-hour average;

Reduced sulfur compounds--10 µg/m3, 1-hour average; or

1 No de minimis air quality level is provided for ozone. However, any net increase of 100 tons per year or more of volatile organic compounds or nitrogen oxides subject to PSD would be required to perform an ambient impact analysis including the gathering of ambient air quality data.

(i) Exemptions, (5)(ii) shall be revised as follows:

(ii) The concentrations of the pollutant in the area that the source or modification would affect are less than the concentrations listed in paragraph (i)(5)(i), or the pollutant is not listed in paragraph (i)(5)(i).

Regulation, 61-62.5, Standard No. 7.1, Nonattainment New Source Review (NSR)

(c) Definitions, (13) “Regulated NSR pollutant” shall be revised as follows:

(13) “Regulated NSR pollutant,” for purposes of this regulation, means the following:

(A) Oxides of nitrogen or any volatile organic compounds;

(B) Any pollutant for which a national ambient air quality standard has been promulgated;

(C) Any pollutant that is a constituent or precursor of a general pollutant listed under paragraphs (c)(13)(A) or (B), provided that a constituent or precursor pollutant may only be regulated under NSR as part of regulation of the general pollutant. Precursors identified by the Administrator for purposes of NSR are the following:

(a) Volatile organic compounds and nitrogen oxides are precursors to ozone in all ozone nonattainment areas;

(b) Sulfur dioxide is a precursor to PM2.5 in all PM2.5 nonattainment areas;

(c) Nitrogen oxides are presumed to be precursors to PM2.5 in all PM2.5 nonattainment areas, unless the State demonstrates to the Administrator’s satisfaction or EPA demonstrates that emissions of nitrogen oxides from sources in a specific area are not a significant contributor to that area’s ambient PM 2.5 concentrations;

(d) Volatile organic compounds and ammonia are presumed not to be precursors to PM2.5 in any PM2.5 nonattainment area, unless the State demonstrates to the Administrator’s satisfaction or EPA demonstrates that emissions of volatile organic compounds or ammonia from sources in a specific area are a significant contributor to that area’s ambient PM2.5 concentrations; or

(D) PM2.5 emissions and PM10 emissions shall include gaseous emissions from a source or activity which condense to form particulate matter at ambient temperatures. On or after January 1, 2011 (or any earlier date established in the upcoming rulemaking codifying test methods), such condensable particulate matter shall be accounted for in applicability determinations and in establishing emissions limitations for PM2.5 and PM10 in permits issued under this ruling. Compliance with emissions limitations for PM2.5 and PM10 issued prior to this date shall not be based on condensable particulate matter unless required by the terms and conditions of the permit or the applicable implementation plan. Applicability determinations made prior to this date without accounting for condensable particulate matter shall not be considered in violation of this section unless the applicable implementation plan required condensable particulate matter to be included.

(c) Definitions, (14) “Significant” shall be revised as follows:

(14) “Significant” means, in reference to a net emissions increase or the potential of a source to emit any of the following pollutants, as rate of emissions that would equal or exceed any of the following rates:

Pollutant Emission Rate

Carbon monoxide: 100 tons per year (tpy)

Nitrogen oxides: 40 tpy

Particulate matter:

15 tpy of PM10 emissions;

10 tpy of direct PM2.5 ;

40 tpy of sulfur dioxide emissions;

40 tpy of nitrogen oxide emissions unless demonstrated not to be a PM2.5 precursor under paragraph 13 of this section

Sulfur dioxide: 40 tpy

Ozone: 40 tpy of volatile organic compounds or oxides of nitrogen

Lead: 0.6 tpy

Statement of Need and Reasonableness:

This statement of need and reasonableness was determined by staff analysis pursuant to S.C. Code Section 1-23-115(C)(1)-(3) and (9)-(11).

DESCRIPTION OF REGULATION: Amendments to R. 61-62.5, Standard No. 7, Prevention of Significant Deterioration, R. 61-62.5, Standard No. 7.1, Nonattainment New Source Review (NSR).

Purpose: The Department has amended R. 61-62.5, Standard No. 7, Prevention of Significant Deterioration; R. 61-62.5, Standard No. 7.1, Nonattainment New Source Review (NSR); and the SIP to incorporate the provisions of the Phase 2 Ozone Rule and the NSR PM2.5 Implementation Rule, and to add a nonsubstantive clarification to correct a scrivener’s error.

Legal Authority: The legal authority for R. 61-62, Air Pollution Control Regulations and Standards, is S.C. Code Sections 48-1-10, et seq.

Plan for Implementation: The amendments took effect upon approval by the Board of Health and Environmental Control and publication in the State Register. The amendments will be implemented by providing the regulated community with copies of the regulation, sending an email to stakeholders, and publishing the Notice of Final Regulation and updated regulations on the Department’s website.

DETERMINATION OF NEED AND REASONABLENESS OF THE REGULATION BASED ON ALL FACTORS HEREIN AND EXPECTED BENEFITS:

The EPA regularly promulgates amendments to federal regulations to include clarification, guidance, technical amendments, and strengthen air quality provisions. States are mandated by law to adopt these federal amendments. These amendments are reasonable as they promote consistency and ensure conformity of state regulations to federal requirements.

DETERMINATION OF COSTS AND BENEFITS:

There will be no increased cost to the state or its political subdivisions resulting from this revision. The standards to be adopted are already effective and applicable to the regulated community as a matter of federal law, thus regulated community has already incurred the cost of these regulations. The amendments will benefit the regulated community by clarifying the regulations and increasing their ease of use.

UNCERTAINTIES OF ESTIMATES:

There are no uncertainties of estimates relative to the costs to the state or its political subdivisions.

EFFECT ON ENVIRONMENT AND PUBLIC HEALTH:

Adoption of the recent changes in federal regulations through the amendments to R. 61-62, Air Pollution Control Regulations and Standards, and the SIP will provide continued protection of the environment and public health.

DETRIMENTAL EFFECT ON THE ENVIRONMENT AND PUBLIC HEALTH IF THE REGULATIONS ARE NOT IMPLEMENTED:

The state’s authority to implement federal requirements, which are beneficial to the public health and environment, would be compromised if these amendments were not adopted in South Carolina.