AGY:	Wildlife and Marine Resources Department 123

PRD:	19970108

EFD:	19970627

EXD:	19970709

REG:	2165

FII:	6

FIV:	21

PRI:	1

PRV:	21

COM:	Fish, Game and Forestry Committee 07 SFGF

Agriculture, Natural Resources and Environmental Affairs Committee 20 HANR

RES:	751

STA:	Final

AUT:	50-5-110

SUB:	Turtle Excluder Devices in Channel (Set) Nets

HST: 2165

BY	DATE	ACTION DESCRIPTION	COM	ISS/VOL	EXP DATE	R. NUM

__	________	_____________________________	_______	________	__________	_______

-	19970108	Proposed Reg Published in SR		1/21

-	19970311	Received by Lt.Gov. & Speaker			19970709

H	19970311	Referred to Committee	HANR 20

S	19970319	Referred to Committee	SFGF 07

S	19970513	Resolution Intro to Approve	SFGF 07			S751

S	19970604	Approved by: Rat # R149		6/21

TXT:

Document No. 2165

DEPARTMENT OF NATURAL RESOURCES

CHAPTER 123

Statutory Authority: 1976 Code Section 50-5-110

Synopsis:

The proposed regulation would require the use of approved turtle excluder devices (TEDs) in all channel (set) nets used in State waters.  Specific areas, including parts of Winyah Bay and all of North Santee Bay, would be exempted by the Department of Natural Resources from the TED requirement unless a significant threat to sea turtles was determined to exist.  The regulation contains gear specifications for the types of TEDs which would be required.  The proposed regulation would also require any sea turtles accidentally taken in channel nets to be immediately released unless resuscitation of the turtle was required, in which case the turtle would have to be retained until it has sufficiently recovered and is capable of swimming before being released.

Instructions: R.123-35 is a new regulation to be added to Chapter 123.

Text:

123.35 Turtle Excluder Devices Required in Channel Nets.

A. The following regulations apply to fishing gears known as channel (set) nets as defined in Sections 50-17-130, 50-17-421, and 50-17-450, and regulation 123-20, SC Code of Laws:

(1) Functional “soft” or “hard” Turtle Excluder Devices (TEDs) must be correctly installed in all channel (set) nets used in the state waters of South Carolina, except in areas specifically exempted by these regulations.

(2) “Soft” TEDs must conform to the following specifications:

(a)	Webbing must be polypropylene or polyethylene

(b) 	Stretched mesh size may not exceed 6 inches

(c) 	Horizontal, taut length of the panel may not exceed 120 inches (10 feet)

(d) 	Panel width at the leading edge may not exceed 229 inches (19 feet)

(e) 	The escape opening may not be less than 72 inches (6 feet)

(f)	The TED panel must form a complete barrier inside the net forward of the codend

(g)	The TED must slope upwards with the escape opening being on the top of the net just forward of the panel’s attachment to the top of the net

(h)	 An optional, rectangular flap may be installed over the escape opening, provided it is attached only on its forward edge, does not extend more than 4 inches aft of the escape opening, is no wider than 36 inches (3 feet), and the mesh size is no larger than 2 inches stretch.

(3) Hard” TEDs must conform to the following specifications:

(a) The TED must be a single, rigid, oval deflector grid which is made of steel rod or pipe not less than 1/3 inch diameter, aluminum rod or pipe not less than ½ inch diameter, or fiberglass rod of comparable strength

(b) The TED must be installed such that it is at a 30 degree to 45 degree angle from the horizontal when in use

(c) The minimum inside diameter may not be less than 30 inches

(d) The minimum (slit) escape opening is 35 inches

(e) The maximum spacing between the bars is 4 inches; there is no minimum spacing

(f) All “hard” TEDs must be top exiting only

(g) An optional, rectangular flap may be installed over the escape opening, provided it is attached only on its forward edge, does not extend more than 4 inches aft of the escape opening, is no wider than 36 inches (3 feet), and the mesh size is no larger than 2 inches stretch.

(4) Channel (set) nets will not require Turtle Excluder Devices if they are fished east of a line in Winyah Bay from the front range on Big Marsh Island, running southeast to Day Marker No. 18, thence running south southeast to red Nun Buoy No.16.  All channel nets west of this line must use approved Turtle Excluder Devices.  Channel nets in North Santee Bay will not be required to have Turtle Excluder Devices.  However, the Department may require TEDs in additional or all open areas if a significant threat to sea turtles is determined.

(5) Any sea turtle accidentally taken must be immediately released unless it is apparent that resuscitation is required.  If resuscitation is required, the fisherman must retain the turtle until such time that the turtle has sufficiently recovered and is capable of swimming.  At that time, the turtle must be released.

Fiscal Impact Statement:

The South Carolina Department of Natural Resources has determined that no additional state funding is required.  Existing staff and resources will be utilized to implement the proposed regulation.

Summary of Assessment Report:

The proposed regulation does not require an assessment report.