AGY:	Department of Labor, Licensing and Regulation 71

FII:	4

FIV:	24

PRD:	19991022

EFD:	20000428

EXD:	20000509

REG:	2464

PRI:	10

PRV:	23

COM:	Labor, Commerce and Industry Committee 	12 SLCI

Labor, Commerce and Industry Committee 26 HLCI

RES:	4487

STA:	Final

AUT:	42-16-40

SUB:	New and Existing Facilities-Platform and Stairway Chairlifts; Sump Pumps or Drains in Elevator Pits

HST:	2464

BY	DATE	ACTION DESCRIPTION	COM	VOL/ISSUE	EXP DATE	R. NUM

___	________	__________________________	_______	___________	__________	_______

-	19991022	Proposed Reg Published in SR		23/10

-	19991222	Received by Lt. Gov & Speaker			20000509

H	20000112	Referred to Committee 	HLCI 26

S	20000112	Referred to Committee 	SLCI 12

H	20000127	Resolution Intro to Approv	HLCI 26			H4487

-	20000323	Approved by:  Rat # R256		24/4

TXT:

Document No. 2464

SOUTH CAROLINA DEPARTMENT OF LABOR, LICENSING AND REGULATION

OFFICE OF ELEVATOR AND AMUSEMENT RIDE SAFETY

CHAPTER 71

Statutory Authority: 1976 Code Section 41-16-40, et seq.

Synopsis:

This regulation amendment revises existing regulations concerning new and existing facilities.  The changes add a new regulation to recognize national safety codes and delete unnecessary state specific requirements for the installation of platform and stairway chairlifts, sump pumps in pits, and installation and use of flexible electrical conduit.

Instructions:  Amend current regulations, by amending Regulation 71-5100.1., and by adding Regulation 71-5100.4.D. as they appear in the text below.  Further amend current regulations by deleting Regulation 71-5200.4.J. in its entirety and relettering the remainder of section.

Text:

71-5100.1.  All facilities installed after July 1, 1986, shall comply with the officially adopted editions of the ASME A17.1 Elevator Code and all supplements thereto, at the time the permit is issued.  In the alternative,  manlifts may comply with the 1992 editions of the ANSI A90.1 Safety Standards for Manlifts and all supplements thereto.  In the alternative platform and stairway chairlifts may comply with ANSI A18.1 and all supplements thereto.  Compliance with any later edition of the required safety codes shall be accepted by the director as compliance with the section.

71-5100.4.D.  A17.1, Rule 106.1(b)(3) is repealed.  Sump pumps or drains are not required in elevator pits by these regulations.  Where indicated by design consideration, sump pumps or drains shall comply with ANSI A17.1, Rule 106.1(b)(3).

Amend 71-5200.4.  by deleting subsection J which currently reads:

71-5200.4.J.  Flexible electrical conduit in lengths in excess of twenty (20) inches shall not be added in machine rooms or machinery spaces.  P.V.C. conduit shall not be added in facilities or electrical feeders supplying facilities except for concrete stub-ups not to exceed six(6) inches.  Any existing P.V.C. or flexible conduit shall be supported or anchored to prevent accidental damage.  Any damaged conduit shall be replaced.

Fiscal Impact Statement:  There will be no additional cost incurred by the State or any political subdivision.