Click here to go directly to text of regulation.

AGY:	Department of Health and Environmental Control

FII:	6

FIV:	25

PRD:	20001027

EFD:	20010622

EXD:	20010518

REG:	2572

PRI:	10

PRV:	24

COM:	Agriculture, Natural Resources and Environmental Affairs Committee 	20 HANR

Medical Affairs Committee 13 SMA

RES:	698

STA:	Final

AUT:	48-1-10 et seq.

SUB:	Water Classifications and Standards

HST:	2572

BY	DATE	ACTION DESCRIPTION	COM	VOL/ISSUE	EXP DATE	R. NUM

___	________	__________________________	_______	___________	__________	_______

-	20001027	Proposed Reg Published in SR		24/10

-	20010116	Received by Lt. Gov & Speaker			20010516

S	20010118	Referred to Committee 	SMA 13

H	20010117	Referred to Committee 	HANR 20

H	20010418	Committee Request Withdrawal	HANR20

120 Day Period Tolled

-	20010420	Withdrawn and Resubmitted			20010518

S	20010522	Resolution Intro to Approv	SMA 13			S698

-	20010518	Approved by:  Expiration Date		25/6

TXT:

Resubmitted April 20, 2001

Document No. 2572

DEPARTMENT OF HEALTH AND ENVIRONMENTAL CONTROL

CHAPTER 61

Statutory Authority: 1976 Code Section 48-1-10 et seq.

R.61-68.   Water Classifications and Standards

Synopsis:

The Department proposes amendment of R.61-68 to clarify, strengthen and improve the existing regulation, and make appropriate revisions of the State's water quality standards in accordance with Section 303(c)(2)(B) of the Federal Clean Water Act.  Section 303(c)(2)(B) requires that South Carolinas water quality standards be reviewed and revised, where necessary, at least every three years for the purposes of considering the Environmental Protection Agencys (EPA) most recent numeric and narrative criteria and to comply with recent Federal regulatory revisions and recommendations.  The Department has also included several revisions that will clarify and improve the regulation.  R.61-68 will be replaced in its entirety because of recodification and stylistic changes made throughout the existing regulation.  See also the Statement of Need and Reasonableness herein.

Table of Changes I:

The table below reflects changes made to the regulation by the Department that were submitted to the General Assembly January 16, 2001.   See also Table II.

(1):	Adoption of federal toxics criteria to reflect the most current final published criteria according to Sections 304(a) and 307(a) of the Clean Water Act.

SECTION					REVISION

R.61-68.E.11				Modified language for clarity and consistency throughout the document.

R.61-68.E.12				Modified language to indicate that all numeric criteria would now be listed in the regulation instead of using references to federal criteria documents.  Also includes modified language for consistency throughout the document.

R.61-68.E.12.a.(1)			Modified language for clarity and consistency throughout the document.

R.61-68.E.12.a.(2)			Modified language for clarity and consistency throughout the document.

R.61-68.E.12.a.(4)			Added language to clarify that the most sensitive resident species in tidal waters must be protected.  Also modified language for clarity and consistency throughout the document.

R.61-68.E.12.a.(5)			Modified language for clarity and consistency throughout the document.  Deleted references to numeric criteria for aquatic life since the numeric criteria are now included in the appendix.

R.61-68.E.12.b.(1)			Added language for use of federal numeric human health criteria for water and organism.  Also modified language for clarity and consistency throughout the document

R.61-68.E.12.c.(1)-(4)		Modified language for clarity and consistency throughout the document.

R.61-68.E.12.c.(5)			Added language to allow the exclusion for the use of federal numeric human health criteria for water and organism when deriving permit effluent limitations.  This will allow a determination that no existing or proposed drinking water source will be affected should the Department not utilize the criteria in deriving permit limitations.

.61-68.E.12.c.(7)-(8)		Added language to reflect that an acute effect on human health (where the public health may be jeopardized to the point that an immediate effect is experienced such as an illness associated with swimming and ingesting fecal material through shellfish) is not allowed by the water quality standards.  The revised language clarifies this point and states how the permit effluent limitations will be calculated to ensure that human health is protected.  Modified language for clarity and consistency throughout the document.

R.61-68.E.12.d.(1)-(2)		Modified language for clarity and consistency throughout the document.

R.61-68.E.12.d.(3)			Added language to allow the Department to use federally-approved methodology for conversion of total metals criteria to dissolved metals for ambient monitoring.

Appendix					The original appendices have been deleted and a new single chart is replacing the previous three.  The chart now contains the values for the numeric criteria for the protection of aquatic life along with human health criteria and organoleptic effects.  This is the first time that the regulation has included these  values for aquatic life numeric criteria since the existing version of the regulation only references the federal documents.  There is also an inclusion of the federal human health numeric criteria for water and organism consumption.  The existing regulation does not include these values.  The changes will make the regulation a better and more convenient source for use by Department staff and the public.  Since the Department no longer references the federal criteria documents for aquatic life, the addition of footnotes and three attachments that contain specifics detailing how the numeric criteria are to be used are also necessarily included.

(2):	Adoption of numeric criteria for phosphorus, nitrogen, and chlorophyll a.

SECTION					REVISION

R.61-68.E.9.a.-e.			Additional language to include numeric criteria for nutrients in order to protect the State’s lakes from nutrient over-enrichment.  Some language was modified for clarification.

(3):	Selection of a more appropriate hardness value.

SECTION					REVISION

R.61-68.E.12.a.(3)			Revised language to reflect a change in the hardness values to be used for calculations for permit effluent limitations and an inclusion of an upper limit for the allowable ambient hardness level.  These hardness values were also required by the EPA in order to reflect the most recently revised federal criteria.

(4):	Adoption of statewide turbidity standard.

SECTION					REVISION

R.61-68.G.9.j.				Addition of numeric criteria for turbidity for Trout waters.

R.61-68.G.10.i.				Addition of numeric criteria for turbidity for Freshwaters including one specific for lakes.

R.61-68.G.11.i.				Addition of numeric criteria for turbidity for Shellfish Harvesting waters.

R.61-68.G.12.i.				Addition of numeric criteria for turbidity for Class SA waters.

R.61-68.G.13.i.				Addition of numeric criteria for turbidity for Class SB waters.

(5):	Inclusion of language to clarify the components of water quality standards.

SECTION					REVISION

R.61-68.A.1.a.-c.			Additional language to clarify the components of water quality standards.

(6):	Inclusion of language to strengthen the existing narrative standards.

SECTION					REVISION

R.61-68.E.3.					Additional language to state that waters of the State cannot be used for waste treatment.

R.61-68.E.12.c.(9)			Additional language to state toxicity testing should be conducted in conformance with federal procedures and when an exception is granted by the Department on the test species, which species should be used.

R.61-68.E.13.a.-b.			Additional language that would allow the Department to require, when determined necessary,  additional monitoring to ascertain the bioaccumulative effects of pollutants.

R.61-68.E.14.a.(1)-(2)		Additional language that requires the Department to use a sensitivity factor when using other scientifically defensible published data for a pollutant for which EPA has not developed a national criterion.  This will ensure that any value used will be appropriate to protect and maintain a balanced indigenous aquatic community as required by the regulation.

R.61-68.E.15.d.				Additional language to state that toxicity testing should be conducted in conformance with federal procedures and when an exception is granted by the Department on the test species, which species should be used.

R.61-68.F.1.a.-e.			Additional language that better describes narrative biocriteria, their objectives, and how they are assessed for purposes of determining compliance with water quality standards.

(7):	Inclusion of a description for the Outstanding National Resource Waters (ONRW) class.

SECTION					REVISION

R.61-68.D.1.a.				Additional language to include the ONRW class.

R.61-68.D.3.					Additional language to include the ONRW class.

R.61-68.G.4.-5.				Additional language to include the ONRW class.

(8):	Clarification of surface water mixing zone allowances.

SECTION					REVISION

R.61-68.C.10.a.-d.			Additional language was included to restrict the size of the acute and chronic mixing zones and to prohibit their use in certain situations in order to protect aquatic life and human health.  Modified language for clarity and consistency throughout the document.

R.61-68.E.10.c.				Modified language to reflect prohibitions on mixing zones for lakes.

(9):	Clarification of language dealing with contaminated groundwater.

SECTION					REVISION

R.61-68.H.4.					Additional language to require that responsible parties be liable for remediation.  Also modified language for clarity and consistency throughout the document.

(10):	Inclusion of a reporting requirement for unauthorized discharges.

SECTION					REVISION

R.61-68.E.4.b.1.-2.			Additional language to require that unauthorized discharges to waters of the State that may cause a violation of a water quality standard be reported to the Department by the responsible party in a timely manner.

(11):	Clarification of the applicability of flow conditions.

SECTION					REVISION

R.61-68.C.4.a.-c.			Additional and revised language to clarify the applicable flow requirements for aquatic life and human health numeric criteria.  This necessitated an inclusion of language for when other flow regimes are allowed, restricted, or prohibited.

(12):	Clarification of existing language to ensure that downstream uses are protected.

SECTION					REVISION

R.61-68.C.7.				Additional language that states downstream uses are protected and maintained.

(13):	Review and revision of language of antidegradation.

SECTION					REVISION

R.61-68.D.2.a.-b.			Additional language that includes requirements for an alternatives analysis consistent with existing federal and state regulatory requirements, more specific determinations that the need for lowering of water quality is necessary to important economic and social development, and public participation requirements.  Clarification that the Department does not have the authority to remove or impair an existing use.

R.61-68.D.4.					Added a S.C. Pollution Control Act citation which contains specific requirements applicable to this section of the regulation.

(14):	Inclusions of new definitions as needed.

Each of the following definitions were added or amended for clarification or consistency with other federal or state regulations, statutes, or scientific documents.  No definitions were added or altered if already defined in the S.C. Pollution Control Act.

SECTION					REVISION

R.61-68.B.2.					Added a definition for 7Q10 flow condition.

R.61-68.B.3.					Added a definition for acute.

R.61-68.B.4.					Added a definition for acute-to-chronic ratio (ACR).

R.61-68.B.6.					Added a definition for annual average flow condition.

R.61-68.B.7.					Added a new definition for aquaculture to be consistent with other state regulation.

R.61-68.B.8.					Amended the definition for aquaculture to aquatic farm.

R.61-68.B.9.					Added a definition for aquatic toxicity test.

R.61-68.B.13.				Added a definition for bioaccumulation.

R.61-68.B.14.				Added a definition for bioaccumulative chemicals of concern.

R.61-68.B.15.				Added a definition for bioavailability.

R.61-68.B.16.				Added a definition for bioconcentration.

R.61-68.B.17.				Added a definition for bioconcentration factor (BCF).

R.61-68.B.18.				Added a definition for biological assessment.

R.61-68.B.19.				Added a definition for biological criteria or biocriteria.

R.61-68.B.20.				Added a definition for biological monitoring or biomonitoring.

R.61-68.B.21.				Added a definition for chlorophyll a.

R.61-68.B.22.				Added a definition for chronic.

R.61-68.B.26.				Added a definition for criterion continuous concentration (CCC).

R.61-68.B.27.				Added a definition for criterion maximum concentration (CMC).

R.61-68.B.30.				Added a definition for ecoregions.

R.61-68.B.31.				Modified definition of ephemeral streams for clarification.

R.61-68.B.34.				Added a definition for fool pool elevation.

R.61-68.B.36.				Added a definition for hydrograph controlled release (HCR).

R.61-68.B.38.				Added a definition for lake.

R.61-68.B.39.				Added a definition for LC50 and deleted the definition for median tolerance limits since they are the same for the purposes of this regulation.

R.61-68.B.41.				Amended the definition for natural conditions to clarify that they do not include conditions attributed to anthropogenic sources.

R.61-68.B.43.				Added a definition for no observed effect concentration (NOEC).

R.61-68.B.44.				Added a definition for nutrients.

R.61-68.B.45.				Added a definition for organoleptic effects.

R.61-68.B.53.				Added a definition for shellfish.

R.61-68.B.56.				Added a definition for tidal conditions.

R.61-68.Be.					Amended the definition for underground source of drinking water to include individual residential wells.

R.61-68.B.62.				Added a definition for whole effluent toxicity (WET).

(15):	Stylistic changes which may include corrections for: clarity and readability, grammar, punctuation, typography, codification, references, language style, and overall improvement of the text of the regulation.

The entire regulation includes revisions due to recodification of additional or deleted language from the proposed text changes so that every section, subsection, subitem, and item could be cited correctly.

SECTION					REVISION

R.61-68.A.1.					Additional language for clarification and consistency throughout the document.

R.61-68.A.2.					Modified language for clarification and consistency throughout the document.

R.61-68.C.1.					Additional language for clarification and consistency throughout the document.

R.61-68.C.2.					Additional language for clarification and consistency throughout the document.

R.61-68.C.3.					Modified language for clarification and consistency throughout the document.

R.61-68.C.5.					Additional language for clarification and consistency throughout the document. Also included additional language to clarify that site-specific standards may be developed for intermittent and ephemeral streams.

R.61-68.C.8.					Additional language for clarification and consistency throughout the document.

R.61-68.C.11.				Additional language for clarification and consistency throughout the document.

R.61-68.C.11.a.				Modified language for clarity and consistency throughout the document.

R.61-68.C.11.a.3.			Modified language for clarity and consistency throughout the document.

R.61-68.C.12.				Additional language for clarification and consistency throughout the document.

R.61-68.E.4.a.				Additional language for clarification and consistency throughout the document.

R.61-68.E.8.					Additional language for consistency throughout the document.

R.61-68.E.10.a.-c.			Additional language for clarification and consistency throughout the document.

R.61-68.E.12.				Additional language for clarification and consistency throughout the document.

R.61-68.E.12.a.(1)-(5)		Additional language for clarification and consistency throughout the document.

R.61-68.E.12.c.(1)-(6)		Additional language for clarification and consistency throughout the document.

R.61-68.E.12.d.(1)-(2)		Additional language for clarification and consistency throughout the document.

R.61-68.E.12.d.(4)			Additional language to clarify how the Department assesses Shellfish Harvesting Waters.

R.61-68.E.15.				Modified language for consistency throughout the document.

R.61-68.E.15.b.				Additional language for clarification and consistency throughout the document.

R.61-68.G.3.					Modified language for clarification and consistency throughout the document.

R.61-68.G.6.a.				Additional language for clarification and consistency throughout the document.

R.61-68.G.7.a.-b.			Correction due to new definitions.

R.61-68.G.7.c.				Correction.

R.61-68.G.8.a.				Additional language for clarification and consistency throughout the document.

R.61-68.G.8.b.				Additional language for clarification and consistency throughout the document.

R.61-68.G.8.c.				Additional language for clarification and consistency throughout the document.

R.61-68.G.9.c.				Correction.

R.61-68.G.10.a.				Correction.

R.61-68.G.11.				Additional language for clarification and consistency throughout the document.

R.61-68.G.11.c.				Correction.

R.61-68.G.12.				Additional language for clarification and consistency throughout the document.

R.61-68.G.12.a.				Correction.

R.61-68.G.12.c.				Correction.

R.61-68.G.13.a.				Correction.

R.61-68.G.13.b.				Correction.

R.61-68.G.13.c.				Correction.

Table of Changes (2):

The table below reflects changes made to the regulation by the Department as recommended by the the House Agriculture, Natural Resources, and Environmental Affairs Committee pursuant to S.C. Code Section 1-23-125(B) of the Administrative Procedures Act.

SECTION					REVISION

R.61-68.A.1.b. 				Delete some language which described narrative criteria.

R.61-68.B.14.				Delete definition of Bioaccumulative chemicals of concern (BCCs) that would no longer be used in the regulation.

R.61-68.B.15.-63.			Renumber after deletion of BCCs definition.

R.61-68.C.10.a.				Delete prohibition of mixing zones for BCCs and carcinogens.

R.61-68.C.10.b.				Revise language for prohibition of mixing zones which attract biota to exclude thermal components.

R.61-68.C.10.c.				Delete prohibition of mixing zones for discharges to lakes.  Remove and revise language that clarified when mixing zones would be prohibited based on poor mixing conditions.  Remove language that clarified when mixing zones would be prohibited for a pollutant when the waterbody was impaired for that pollutant.  Revise language that may limit or prohibit mixing zones in significant nursery habitat.

R.61-68.E.9.a.				Revise language to clarify that all sources of nutrients include both point and nonpoint sources.

R.61-68.E.10.c.				Revise language to allow thermal mixing zones in lakes.

R.61-68.E.12.c.(2)			Revise language to allow the use of practical quantitation limits for compliance with permit limitations.

R.61-68.E.12.c.(3)			Revise language to allow the use of practical quantitation limits for compliance with permit limitations.

R.61-68.E.12.c.(6)			Add language for an exclusion for compliance with the numeric turbidity criteria for NPDES permitted discharges that do not have reasonable potential to cause or contribute to an excursion of the water quality standard for turbidity.

R.61-68.E.12.c.(8-10)		Renumber sections after the addition of the exclusion noted in the previous amendment.

R.61-68.G.3.					Add language to clarify the sections that refer to consideration of natural conditions.  Add language for a specified exclusion for compliance with the numeric turbidity criteria for land disturbing activities that are in complete compliance with their permit requirements and do not cause impairment to aquatic life.

Appendix.					Revise 2,3,7,8-TCDD (dioxin) numeric criteria for protection of human health to reflect the State-derived value in lieu of the EPA-derived value.

Text of Regulation

61-68.  WATER CLASSIFICATIONS AND STANDARDS

TABLE OF CONTENTS

SECTION A.	PURPOSE AND SCOPE

SECTION B.	DEFINITIONS

SECTION C.	APPLICABILITY OF STANDARDS

SECTION D.	ANTIDEGRADATIONRULES

SECTION E.	GENERAL RULES AND STANDARDS APPLICABLE TO ALL WATERS

SECTION F.	NARRATIVE BIOLOGICAL CRITERIA

SECTION G.	CLASS DESCRIPTIONS, DESIGNATIONS, AND SPECIFIC STANDARDS FOR SURFACE WATERS

SECTION H.	CLASS DESCRIPTIONS AND SPECIFIC STANDARDS FOR GROUNDWATERS

SECTION I.		SEVERABILITY

APPENDIX.		WATER QUALITY CRITERIA FOR PROTECTION OF AQUATIC LIFE, HUMAN HEALTH, AND BASED ON ORGANOLEPTIC DATA

A.  PURPOSE AND SCOPE.

1.	These regulations, promulgated pursuant to authority in the S. C. Pollution Control Act, Section 48-1-10 et seq., 1976 Code of Laws, establish a system and rules for managing and protecting the quality of South Carolina's surface and ground water.  They establish the State's official classified water uses for all waters of the State, establish general rules and specific numeric and narrative criteria for protecting classified and existing water uses, and establish procedures for classifying waters of the State.  The water quality standards include the uses of the waters, the numeric and narrative criteria, and the antidegradation rules contained in this regulation.

a.	 The uses of the waters of the State are defined and described in Sections B, C, E, F, G, and H of this regulation.

b.	Numeric criteria for aquatic life and human health are numeric values for specific parameters and pollutants or water quality levels which have been assigned for the protection of the existing and classified uses for each of the classifications in South Carolina and are listed in Section D, E, G, H, and the Appendix.  Narrative criteria for aquatic life and human health are general goals and statements of attainable or attained conditions of biological integrity and water quality of the waterbody.  These narrative criteria rely upon the use of standardized measures and data analyses to make qualitative determinations of the water quality and use attainment.  The Department uses scientifically sound and, where applicable, EPA-approved methods in making these determinations.  Narrative criteria are listed in Sections C, D, E, F, G, and H.

c.	Antidegradation rules provide a minimum level of protection to all waters of the State and also include provisions and requirements necessary to determine when and if water quality degradation is allowed.  Antidegradation rules are described in Section D of this regulation.

2.	Waters which meet standards shall be maintained.  Waters which do not meet standards shall be improved, wherever attainable, to achieve those standards.  However, the Department cannot assure that classified waters shall at all times meet the numeric water quality standards for such uses.

3.	Recognizing the technical and economic difficulty in restoring water quality, the Department shall emphasize a preventive approach in protecting waters of the State.

4.	It is a goal of the Department to maintain and improve all surface waters to a level to provide for the survival and propagation of a balanced indigenous aquatic community of flora and fauna and to provide for recreation in and on the water.  It is also a goal to provide, where appropriate and desirable, for drinking water after conventional treatment, shellfish harvesting, and industrial and agricultural uses.

5.	It is a goal of the Department to maintain or restore ground water quality so it is suitable as a drinking water source without any treatment.

B.	DEFINITIONS.

1.	The definition of any word or phrase employed in this regulation shall be the same as given in the South Carolina Pollution Control Act, 48-1-10, et seq, S.C. Code of Laws, 1976, hereafter referred to as the Act.  Words or phrases which are not defined in the Act are defined as follows:

2.	7Q10 means the annual minimum seven day average flow rate that occurs with an average frequency of once in ten years as published or verified by the U. S. Geological Survey (USGS) or an estimate extrapolated from published or verified USGS data.

3.	Acute means a stimulus severe enough to rapidly induce an effect; in aquatic toxicity tests, an effect observed in 96 hours or less typically is considered acute.  When referring to aquatic toxicology or human health, an acute effect is not always measured in terms of lethality.

4.	Acute-to-chronic ratio (ACR) means the ratio of the acute toxicity of an effluent or a toxicant to its chronic toxicity.  It is used as a factor for estimating chronic toxicity on the basis of acute toxicity data, or for estimating acute toxicity on the basis of chronic toxicity data.

5.	Agricultural means the use of water for stock watering, irrigation, and other farm purposes.

6.	Annual average flow means the annual mean flow rate of a stream at a specific point as published or verified by the U. S. Geological Survey (USGS) or an estimated annual mean flow rate extrapolated from published or verified USGS data.

7.	Aquaculture means a defined managed water area which uses discharges of pollutants into that designated area for the maintenance or production of harvestable freshwater, estuarine, or marine plants or animals.

8.	Aquatic farm means the cultivation, production, or marketing of domestic aquatic organisms which are any fish, aquatic invertebrates, or aquatic plants that are spawned, produced, or marketed as a cultivated crop in the waters of the State.

9.	Aquatic toxicity test mean laboratory experiments that measure the biological effect (e.g., growth, survival, and reproduction) of effluents or receiving waters on aquatic organisms.

10.	Aquifer means a geologic formation, group of formations, or part of a formation that contains sufficient saturated permeable material to yield significant quantities of ground water to wells or springs.

11.	Balanced indigenous aquatic community means a natural, diverse biotic community characterized by the capacity to sustain itself through cyclic seasonal changes, presence of necessary food chain species and by a lack of domination by pollutant tolerant species.

12.	Best management practice (BMP) means a practice or combination of practices that are the most effective, practical ways of controlling or abating pollution from widespread or localized sources.

13.	Bioaccumulation means the process by which a compound is taken up and retained by an aquatic organism, both from water and through food.

14. Bioavailability means a measure of the physiochemical access that a toxicant has to the biological processes of an organism.  The less the bioavailability of a toxicant, the less its toxic effect on an organism.

15. Bioconcentration means the process by which a compound is absorbed from water through gills or epithelial tissues and is concentrated in the body.

16.	Bioconcentration factor (BCF) means the ratio of a substances concentration in tissue versus its concentration in water, in situations where the food chain is not exposed or represents equilibrium partitioning between water and organisms.

17.	Biological assessment means an evaluation of the biological condition of a waterbody using biological surveys and other direct measurements of resident biota in surface waters and sediments.

18.	Biological criteria, also known as biocriteria, mean narrative expressions or numeric values of the biological characteristics of aquatic communities based on appropriate reference conditions.  Biological criteria serve as an index of aquatic community health.

19. Biological monitoring, also known as biomonitoring, means a description of the living organisms in water quality surveillance used to indicate compliance with water quality standards or permit effluent limits and to document water quality trends.  Methods of biological monitoring may include, but are not limited to, toxicity testing such as ambient toxicity testing, whole effluent toxicity testing, and ambient assessment of the resident biological community.

20.	Chlorophyll a means a photosynthetic pigment present in all types of green plants.  It is used as a measure of algal biomass and is an indicator of nutrient enrichment.

21.	Chronic means a stimulus that lingers or continues for a relatively long period of time, often one-tenth of the life span or more.  Chronic should be considered a relative term depending on the life span of an organism.  The measurement of a chronic effect can be reduced growth, reduced reproduction, etc., in addition to lethality.

22.	Classified uses means those uses specified in Section G for surface waters and Section H for ground waters, whether or not those uses are being attained.

23.	Concentrated aquatic animal production facility means a hatchery, fish farm, or other facility related to aquatic animal production which is not located in waters of the State and is subject to a National Pollutant Discharge Elimination System (NPDES) permit.

24.	Conventional treatment as applying to potable water supplies means treatment including at least flocculation, sedimentation, filtration, and disinfection.

25.	Criterion continuous concentration (CCC) means the highest instream concentration of a toxicant or an effluent to which the organisms can be exposed to protect against chronic (long-term) effects.  EPA derives chronic criteria from longer term (often greater than 28 days) tests that measure survival, growth, reproduction, and in some cases bioconcentration.

26.	Criterion maximum concentration (CMC) means the highest instream concentration of a toxicant or an effluent to which the organisms can be exposed for a brief period of time without causing an acute effect.  EPA derives acute criteria from 48 to 96 hour tests of lethality or immobilization.

27.	Daily average means the average of all samples taken during any 24 hour period.

28.	Deleterious substances mean those substances which in sufficient concentrations or levels have a harmful effect on classified or existing water uses.

29.	Ecoregions mean areas of general similarity in ecosystems and in the type, quality, and quantity of environmental resources and are designed to serve as a spatial framework for the research, assessment, management, and monitoring of ecosystems and ecosystem components. The EPA has published a document that outlines the Level III ecoregions (please refer to U.S.Environmental Protection Agency. 1999.  Level III ecoregions of the continental United States (revision of Omernik, 1987).  Corvallis, Oregon, U.S. E.P.A.-National Health and Environmental Effects Research Laboratory, Map M-1.)  The following are South Carolina Level III ecoregions: Blue Ridge Mountains, Piedmont, Southeastern Plains, and Middle Atlantic Coastal Plains.

30.	Ephemeral streams mean streams that generally have defined natural watercourses that flow only in direct response to rainfall or snowmelt and in which discrete periods of flow persist no more than 29 consecutive days per event.

31.	Existing uses means those uses actually being attained in or on the water, on or after November 28, 1975, regardless of the classified uses.

32.	Fishing means the taking, harvesting, or catching of finfish or crustaceans for human consumption.

33.	 Full pool elevation means the maximum lake level attained before water releases over a fixed weir, spillway, or other discharge structure.  In larger lakes and reservoirs, the full pool elevation is the maximum level established for management.

34.	Groundwater means water below the land surface in a zone of saturation.

35.	Hydrograph Controlled Release (HCRs) means the onsite storage or holding of treated wastewater or the use of an alternative discharge option contained in Section D.2.a. of this regulation, during specified critical streamflow conditions and then discharging the treated wastewater to the stream when streamflow is sufficient to assimilate the wastewater.

36.	Intermittent streams means streams that generally have defined natural watercourses which do not flow year around, but flow beyond periods of rainfall or snowmelt.

37.	Lake means any water of the State that is a freshwater pond, reservoir, impoundment, or similar body of water located wholly or partially within the State.

38.	LC50 means the concentration of a toxicant at which lethality occurs to 50 percent of the test organisms during a specified exposure time period.

39.	Mixing zone means:

a.	for surface waters, an area where a discharge undergoes initial dilution and is extended to cover the secondary mixing in the ambient waterbody.  A mixing zone is an allocated impact zone where water quality criteria can be exceeded as long as acutely toxic conditions are prevented (except as defined within a Zone of initial dilution) and public health and welfare are not endangered.

b.	For ground waters, a hydrogeologically controlled three-dimensional flow path in the subsurface which constitutes the pathway for waste constituents to migrate from a source.

40.	Natural conditions mean those water quality conditions unaffected by anthropogenic sources of pollution.

41.	No Discharge Zone (NDZ) means a waterbody (or a portion of a waterbody) so designated that no discharging Marine Sanitation Devices (MSDs) are allowed on vessels on waterbodies so designated.  All vessels located on such designated waterbodies shall be equipped with MSDs which discharge to a holding tank which shall be pumped out at a designated pump-out location or shall discharge legally outside the boundary of the United States.

42.	No observed effect concentration (NOEC) means the highest tested concentration of an effluent or a toxicant at which no adverse effects are observed on the aquatic test organisms at a specific time of observation and determined using hypothesis testing.

43.	Nutrients mean an element or chemical essential to life including, but not limited to, nitrogen and phosphorus.

44.	Organoleptic effects mean those sensory effects associated with taste and smell.

45.	Outstanding recreational or ecological resource waters means waters which are of exceptional recreational or ecological importance or of unusual value.  Such waters may include, but are not limited to:  waters in national or state parks or wildlife refuges; waters supporting threatened or endangered species; waters under the National Wild and Scenic Rivers Act or South Carolina Scenic Rivers Act; waters known to be significant nursery areas for commercially important species or known to contain significant commercial or public shellfish resources; or waters used for or having significant value for scientific research and study.

46.	Prohibited area means an area adjacent to point source discharges or other sources of potential contamination in shellfish growing waters where the gathering of clams, mussels, or oysters is prohibited to protect public health.

47.	Primary contact recreation means any activity with the intended purpose of direct water contact by the human body to the point of complete submergence, including but not limited to swimming, water skiing, and skin diving.

48.	Propagation means the continuance of species through reproduction and growth in the natural environment, as opposed to the maintenance of species by artificial culture and stocking.

49.	Public water system means any public or privately owned waterworks system which provides drinking water for human consumption, except those serving a single private residence or dwelling.

50.	Recharge area means an area where an underground source of drinking water is poorly confined, is under water table conditions, and has a downward component of flow from the water table into the underground source of drinking water.

51.	Secondary contact recreation means any activity occurring on or near the water which does not have an intended purpose of direct water contact by the human body to the point of complete submergence, including but not limited to fishing, boating, canoeing, and wading.

52.	Shellfish mean bivalve mollusks, specifically clams, mussels, or oysters.

53.	Shellfish harvesting means taking of bivalve mollusks, specifically clams, mussels, or oysters, for direct marketing or human consumption.

54.	Source for drinking water supply means any source of surface water which is used for domestic consumption, or used in connection with the processing of milk, beverages, food or for other purposes which required finished water meeting regulations [40 CFR Part 141 and 40 CFR Part 143] established pursuant to the Safe Drinking Water Act (Public Law 93-523, 95-190) applicable to public water systems.

55.	Tidal conditions mean conditions determined by the Department as appropriate for  tidally influenced waters of the State to be analogous to the 7Q10 or the annual average flow for flowing waters of the State.

56.	Tidal saltwaters means those waters whose elevation is subject to changes due to oceanic tides and which have chloride ion content in excess of 250 milligrams per liter (mg/l) (salinity = 0.48 parts per thousand).

57.	Toxic wastes means those wastes or combinations of wastes including disease-causing agents which, after discharge and upon exposure, ingestion, inhalation, or assimilation into any organism, either directly from the environment or indirectly by ingestion through food chains, may cause death, disease, behavioral abnormalities, cancer, genetic mutations, physiological malfunctions (including malfunctions in reproduction), physical deformations, or restrict or impair growth in such organisms or their offspring.

58.	Underground source of drinking water (USDW) means an aquifer or its portion:

a.	Which supplies any public water system or individual residential well; or

b.	Which contains a sufficient quantity of ground water to supply a public water system or individual residential well; and,

(1) Currently supplies drinking water for human consumption; or

(2) Contains water with less than ten thousand milligrams per liter total dissolved solids.

59.	Water table means that level below the land surface at which all the voids are filled with water at a pressure equal to atmospheric.

60.	Weekly average means the average of all samples taken during any consecutive seven day period.

61.	Whole effluent toxicity (WET) means the aggregate toxic effect of an aqueous sample measured directly by an aquatic toxicity test.

62.	Zone of initial dilution (ZID) means that minimal area of a mixing zone immediately surrounding the outfall where water quality criteria are not met, provided there is no acute toxicity to drifting organisms and public health and welfare are not endangered.

C.	APPLICABILITY OF STANDARDS.

1.	The water quality standards are applicable to both surface waters and ground waters.

2.	Any exception specified in this regulation is to be applied exclusively to the situation for which it was incorporated and not as a general rule applicable to all situations or waters of the State.

3.	Uses in all waters shall be protected, wherever attainable, regardless of flow.

4.	Flow requirements, prohibitions, and exceptions.

a.	Aquatic life numeric criteria

(1)	The applicable critical flow conditions for aquatic life criteria shall be defined as 7Q10 or tidal conditions as determined by the Department.  The numeric  criteria of this regulation are not applicable to waters of the State when the flow rate is less than 7Q10 except as prescribed below.

(2)	The Department shall consider conditions that are comparable to or more stringent than 7Q10 where appropriate to protect classified and existing uses, such as below dams and in tidal situations.  Only those situations where the use of 7Q10 flows are determined to be impracticable, inappropriate, or insufficiently protective of aquatic life uses shall be considered as a situation in which the Department may consider other flow conditions.

(3)	The Department shall use the applicable critical flow conditions for the protection and maintenance of aquatic life for, but not limited to, the following: permit issuance, wasteload allocations, load allocations, and mixing zones.

b.	Human health and organoleptic numeric criteria

(1)	The applicable critical flow conditions for human health shall be defined as annual average flow for carcinogens, 7Q10 (or 30Q5 if provided by the applicant) for noncarcinogens, or tidal conditions as determined by the Department.  The applicable critical flow conditions for organoleptic criteria shall be defined as annual average flow or tidal conditions as determined by the Department.  The numeric criteria of this regulation are not applicable to waters of the State when the flow rate is less than the annual average flow for carcinogens or 7Q10 (or 30Q5 if provided by the applicant) for noncarcinogens, except as prescribed below.

(2)	The Department shall consider conditions that are comparable to or more stringent than annual average flow, 7Q10, or 30Q5 (if provided by the applicant) where appropriate to protect the classified and existing uses, such as below dams and in tidal situations.  Only those situations where the use of annual average flow, or 7Q10, or 30Q5 (if provided by the applicant) are determined to be impracticable, inappropriate, or insufficiently protective of human health uses shall be considered as a situation in which the Department may consider other flow conditions.

(3)	The Department shall use the applicable critical flow conditions for human health and organoleptic effects for, but not limited to, the following: permit issuance, wasteload allocations, load allocations, and mixing zones.

c.	As described below, the Department may also consider conditions other than 7Q10 for use with an HCR.

(1)	After a complete antidegradation review in compliance with Section D.2., an HCR for oxygen-demanding substances may be permitted by the Department for the following situations:

i.	if other flow-related effluent conditions are allowed by federal effluent guidelines as specified in 40 CFR Parts 400-499 (Chapter I, Subchapter N) and when used the numeric criteria shall not be exceeded and all water quality standards are maintained and protected;

ii.	for industrial discharges, after application of advanced wastewater treatment, as determined by the Department, for the type of wastewater discharged;

iii.	for other discharges, after application of advanced wastewater treatment which will be defined, for this purpose, at or below the following permit effluent limitations of BOD5 = 10 mg/l, NH3-N = 1 mg/l, and DO = 6 mg/l.

(2)	In cases where an HCR may be allowed, the permit effluent limitations for toxics will not be variable and will be based on the critical flow conditions (chemical-specific or WET).

(3)	In cases where an HCR may be allowed, new or proposed expansions of existing permits shall require instream biological assessments and existing permits may require instream biological assessments.

5.	Intermittent streams and ephemeral streams shall be considered waters of the State.  The  water quality standards of the class of the stream to which intermittent and ephemeral streams are tributary shall apply, disregarding any site-specific numeric criteria for the named waterbody.  This does not preclude the development of site-specific numeric criteria for intermittent and ephemeral streams.

6.	The standards of adjacent waters must be maintained in basins excavated from high ground and constructed solely for berthing vessels.  The standards of the adjacent waters must also be maintained with regard to impacts from created marina basins.

7.	The existing and classified uses of downstream waters shall be maintained and protected.

8.	Where surface waters are not classified by name (unlisted) in R.61-69, Classified Waters, the water quality standards of the class of the stream to which they are tributary shall apply, disregarding any site specific numericcriteria for the named waterbody.  In tidal areas where an unlisted tributary flows between two differently classified waterbodies, the more stringent numeric criteria of the classified waters apply to the unlisted tributary, disregarding any site-specific numeric criteria for those waterbodies.  This does not preclude the development of site-specific numeric criteria for unlisted tributaries.

9.	Because of natural conditions some surface and ground waters may have characteristics outside the standards established by this regulation.  Such natural conditions do not constitute a violation of the water quality standards; however, degradation of existing water quality is prohibited unless consistent with Section D.4 of this regulation.

10.	A mixing zone for surface waters may be allowed by the Department.  All water quality standards of the classification of the surface waters, including affected downstream waters, are applicable unless a mixing zone, setting forth certain conditions, is granted by the Department.  When the Department grants a mixing zone, the mixing zone shall not be an area of waste treatment nor shall it interfere with or impair the existing uses of the waterbody.  The size of the mixing zone shall be minimized, as determined by the Department, and shall be based upon applicable critical flow conditions.  Since mixing zones are allocated impact zones where human health and aquatic life numeric criteria can be exceeded, the Department shall restrict their use.  The following prohibitions and restrictions are established in order to support these important uses of the waters of the State.

a.	In order to protect human health, mixing zones are not allowed when: they would endanger public health and welfare, any portion of the mixing zone would be in a state-approved source water protection area, the mixing zone would adversely affect shellfish harvesting, or the mixing zone would be for bacteria (e.g. fecal coliform).

b.	In order to protect aquatic life, mixing zones are not allowed when: a pollutant, excluding temperature or thermal, in a discharge would attract biota; the mixing zone would result in undesirable aquatic organisms or a dominance of nuisance species outside of the mixing zone; there is a reasonable expectation that a discharge would adversely affect a federally-listed endangered or threatened aquatic species, its habitat, or a proposed or designated critical habitat; the mixing zone would not allow safe passage of aquatic organisms when passage would otherwise be unobstructed; or the mixing zone would not allow for the protection and propagation of a balanced indigenous aquatic community in and on the waterbody.

c.	In order to protect both human health and aquatic life, mixing zones are not allowed when: a discharge would not be predicted to or does not produce adequate mixing at the point of discharge; or a discharge would be to a waterbody where multiple discharges interact if the combined mixing zone would impair the waterbody outside the mixing zone.  The Department may prohibit or limit mixing zones in waters of the State that may be considered a significant estuarine nursery habitat for resident species.

d.	The size of the mixing zone shall be kept to a minimum and may be determined on an individual project basis considering biological, chemical, engineering, hydrological, and physical factors.

11.	Mixing zones for ground waters may be allowed by the Department.  In order to ensure the maintenance and protection of the uses of the waters of the State and in compliance with Section D of this regulation, any mixing zone granted by the Department shall be determined on an individual basis by the Department as prescribed below.

a.	The numeric standards for Class GB ground water, Section H.9, are applicable unless a mixing zone solely within the bounds of the property, setting forth certain conditions, is granted by the Department.  Such a mixing zone shall be granted upon satisfactory demonstration to the Department that:

(1)	Reasonable measures have been taken or binding commitments are made to minimize the addition of contaminants to ground water and/or control the migration of contaminants in ground water;

(2)	The ground water in question is confined to a shallow geologic unit which has little or no potential of being an Underground Source of Drinking Water, and discharges or will discharge to surface waters without contravening the surface water standards set forth in this regulation;

(3)	The contaminant(s) in question occurs within the bounds of the property, and there is minimum possibility for ground water withdrawals (present or future) to create drawdown such that contaminants would flow off-site; and

(4)	The contaminants or combination of contaminants in question are not dangerously toxic, mobile, or persistent.

12.	Site-specific numeric criteria for surface waters may be established by the Department to replace the numeric criteria of Sections E, G, and the appendix of this regulation or to add new numeric criteria not contained in this regulation.  Establishment of such numeric criteria shall be subject to public participation and administrative procedures for adopting regulations.  In addition, such site-specific numeric criteria shall not apply to tributary or downstream waters unless specifically described in the water classification listing R.61-69, Classified Waters.

13.	In classifying and adopting standards for the waters of the State, the Department considers:

a.	The size, depth, surface area covered, volume, flow direction, rate of flow, stream gradient and temperature of the water;

b.	The character of the district bordering such water and its suitability for the uses and with a view to conserving it and encouraging the most appropriate use of the lands bordering on such water for residential, agricultural, industrial, or recreational purposes;

c.	The uses which have been made, are being made, may be made or are desired to be made of such waters for transportation, domestic, and industrial consumption, irrigation, swimming, fishing, fish culture, fire prevention, sewage disposal or other uses;

d.	The present quality of such waters; and

e.	Information, about the four items above, from government agencies, interested groups, and the public.

D.	ANTIDEGRADATION RULES.

1.	Existing water uses and the level of water quality necessary to protect these existing uses shall be maintained and protected regardless of the water classification and consistent with the policies below.

a.	A new activity or expansion of an existing activity shall not be allowed in Class ONRW, Class ORW, or Shellfish Harvesting Waters if it would exclude, through establishment of a prohibited area, an existing shellfish harvesting or culture use.  A new activity or expansion of an existing activity which will result in a prohibited area may be allowed in Class SA or Class SB waters when determined to be appropriate by the Department and would not remove or impair an existing use.

b.	Existing uses and water quality necessary to protect these uses are presently affected or may be affected by instream modifications or water withdrawals.  The stream flows necessary to protect classified and existing uses and the water quality supporting these uses shall be maintained consistent with riparian rights to reasonable use of water.

c.	Existing or classified ground water uses and the conditions necessary to protect those uses shall be maintained and protected.

2.	Where surface water quality exceeds levels necessary to support propagation of fish, shellfish, and wildlife, and recreation in and on the water, that quality shall be maintained and protected unless the Department finds, after intergovernmental coordination and public participation, that allowing lower water quality is necessary to important economic or social development in the areas where the waters are located.  In allowing such lower water quality, water quality adequate to fully protect existing and classified uses shall be maintained.  The highest statutory and regulatory requirements for all new and existing point sources shall be achieved and all cost-effective and reasonable best management practices for nonpoint source control shall be achieved within the State's statutory authority and otherwise encouraged.  In order to fulfill these goals, the Department shall consider (a) and (b) below when evaluating any proposed expansion or new discharge to waters of the State that will lower water quality to a measurable effect.  This includes, but is not limited to, the new or increased loading of any pollutant or pollutant parameter in the effluent regardless of whether the discharge flow changes.

a.	An alternatives analysis, conducted by the applicant, must demonstrate to the Department that none of the following applicable alternatives that would minimize or eliminate the lowering of water quality are economically and technologically reasonable:

(1)	water recycle or reuse,

(2)	use of other discharge locations,

(3)	connection to other wastewater treatment facilities,

(4)	use of land application,

(5)	product or raw material substitution,

(6)	any other treatment option or alternative.

b.	After the alternatives analysis is completed, the Department shall evaluate whether a proposed discharge that will result in the lowering of water quality of a waterbody, and for which there are no economically or technologically reasonable alternatives, is necessary for important economic or social development.  For this to be accomplished, several economic and social factors must be considered.  If an evaluation of the economic and social factors reveals that affordable treatment options that, combined with any alternatives, would prevent the need for the lowering of water quality, the Department shall deny the request.  Conformance of the proposed discharge with the applicable 208 Areawide Water Quality Management Plans may demonstrate importance to economic and social development as well as intergovernmental coordination and public participation.  Activities requiring permits or certification by the Department shall provide for public participation through the Departments existing public notification processes.  Economic and social factors to be considered may include the following:

(1)	employment (increases, maintenance, or avoidance of reduction),

(2)	increased industrial production,

(3)	improved community tax base,

(4)	improved housing, and/or

(5)	correction of an environmental or public health problem.

3.	The water quality of outstanding resource surface waters designated as Class ONRW or Class ORW shall be maintained and protected through application of the standards for these classifications as described in Section G.

4.	Certain natural conditions may cause a depression of dissolved oxygen in surface waters while existing and classified uses are still maintained.  The Department shall allow a dissolved oxygen depression in these naturally low dissolved oxygen waterbodies as prescribed below pursuant to the Act, Section 48-1-83, et seq., 1976 Code of Laws:

a.	Under these conditions the quality of the surface waters shall not be cumulatively lowered more than 0.1 mg/l for dissolved oxygen from point sources and other activities, or

b.	Where natural conditions alone create dissolved oxygen concentrations less than 110 percent of the applicable water quality standard established for that waterbody, the minimum acceptable concentration is 90 percent of the natural condition.  Under these circumstances, an anthropogenic dissolved oxygen depression greater than 0.1 mg/l shall not be allowed unless it is demonstrated that resident aquatic species shall not be adversely affected.  The Department may modify permit conditions to require appropriate instream biological monitoring.

c.	The dissolved oxygen concentrations shall not be cumulatively lowered more than the deficit described above utilizing a daily average unless it can be demonstrated that resident aquatic species shall not be adversely affected by an alternate averaging period.

E. GENERAL RULES AND STANDARDS APPLICABLE TO ALL WATERS.

1.	The General Assembly of South Carolina in the Act has declared the following policy:  "It is declared to be the public policy of the State to maintain reasonable standards of purity of the air and water resources of the State, consistent with the public health, safety and welfare of its citizens, maximum employment, the industrial development of the State, the propagation and protection of terrestrial and marine fauna and flora, and the protection of physical property and other resources.  It is further declared that to secure these purposes and the enforcement of the provisions of this Act, the Department of Health and Environmental Control shall have authority to abate, control and prevent pollution."

2.	The classes and standards described in Section G and H of this regulation implement the above State policy by protecting the waters of South Carolina.  Consistent with the above policy, the Department adopts the following general standards in items 3-15 for all waters of South Carolina.

3.	No waters of the State shall be used for the sole or principal purpose of transporting or treating wastes.

4.	a. Any discharge into waters of the State must be permitted by the Department and receive a degree of treatment and/or control which shall produce an effluent which is consistent with the Act, the Clean Water Act (P.L. 92-500, 95-217, 97-117, 100-4), this regulation, and related regulations.  No permit issued by the Department shall be interpreted as creating any vested right in any person.  Additionally, any discharge into waters of the State containing sanitary wastes shall be effectively disinfected as necessary to meet the appropriate standards of this regulation.  The Department may require best management practices (BMPs) for control of stormwater runoff as part of the requirements of an NPDES permit, a State construction permit, or a State 401 Water Quality Certification.

b.	When not specifically covered by permit reporting requirements, any unauthorized discharge into waters of the State which may cause or contribute to an excursion of a water quality standard must be reported by the responsible party to the Department orally within 24 hours of becoming aware of such conditions.  Further, written notification must be provided to the Department (Compliance Assurance Division, Bureau of Water) within five (5) days of becoming aware of such conditions and the written notice must include the following:

(1)	A description of the discharge and cause;

(2)	The duration of the discharge, including exact dates and times, and if not corrected, the time that the unauthorized discharge is expected to cease, and what steps are being taken to eliminate, minimize, and prevent recurrence of the discharge.

5.	All ground waters and surface waters of the State shall at all times, regardless of flow, be free from:

a.	Sewage, industrial waste, or other waste that will settle to form sludge deposits that are unsightly, putrescent, or odorous to such degree as to create a nuisance, or interfere with classified water uses or existing water uses;

b.	Floating debris, oil, grease, scum, and other floating material attributable to sewage, industrial waste, or other waste in amounts sufficient to be unsightly to such a degree as to create a nuisance or interfere with classified water uses or existing water uses;

c.	Sewage, industrial, or other waste which produce taste or odor or change the existing color or physical, chemical, or biological conditions in the receiving waters or aquifers to such a degree as to create a nuisance, or interfere with classified water uses (except classified uses within mixing zones as described in this regulation) or existing water uses; and,

d.	High temperature, toxic, corrosive, or deleterious substances attributable to sewage, industrial waste, or other waste in concentrations or combinations which interfere with classified water uses (except classified uses within mixing zones as described in this regulation), existing water uses, or which are harmful to human, animal, plant or aquatic life.

6.	Waters where classified uses are not being attained can be reclassified for protection of an attainable use and standards designated for that use where:

a.	Natural conditions prevent the attainment of the use; or

b.	Natural, ephemeral, intermittent, low flow conditions, or water levels prevent the attainment of the use; or

c.	Human caused conditions or sources prevent the attainment of the use and cannot be remedied or would cause more environmental damage to correct than to leave in place; or

d.	Dams, diversions, or other types of hydrologic modifications preclude the attainment of the use, and it is not feasible to restore the waterbody to its original condition or to operate such modification in a way that would result in the attainment of the use; or

e.	Physical conditions related to the natural features of the water body, such as the lack of a proper substrate, cover, flow, depth, pools, riffles, and the like, preclude attainment of aquatic life protection uses; or

f.	Controls more stringent than those required by Sections 301(b) and 306 of the Clean Water Act would result in substantial and widespread economic and social impact.

7.	Prior to removing any uses, notice and an opportunity for a public hearing shall be provided.

8.	Discharge of fill into waters of the State is not allowed unless the activity is consistent with Department regulations and will result in enhancement of classified uses with no significant degradation to the aquatic ecosystem or water quality.

9.	In order to protect and maintain lakes and other waters of the State, consideration needs to be given to the control of nutrients reaching the waters of the State.  Therefore, the Department shall control nutrients as prescribed below.

a.	Discharges of nutrients from all sources, including point and nonpoint, to waters of the State shall be prohibited or limited if the discharge would result in or if the waters experience growths of microscopic or macroscopic vegetation such that the water quality standards would be violated or the existing or classified uses of the waters would be impaired.  Loading of nutrients shall be addressed on an individual basis as necessary to ensure compliance with the narrative and numeric criteria.

b.	Numeric nutrient criteria for lakes are based on an ecoregional approach which takes into account the geographic location of the lakes within the State and are listed below.  These numeric criteria are applicable to lakes of 40 acres or more.  Lakes of less than 40 acres will continue to be protected by the narrative criteria.

(1)	for the Blue Ridge Mountains ecoregion of the State, total phosphorus shall not exceed 0.02 mg/l, chlorophyll a shall not exceed 10 ug/l, and total nitrogen shall not exceed 0.35 mg/l

(2)	for the Piedmont and Southeastern Plains ecoregions of the State, total phosphorus shall not exceed 0.06 mg/l, chlorophyll a shall not exceed 40 ug/l, and total nitrogen shall not exceed 1.50 mg/l

(3)	for the Middle Atlantic Coastal Plains ecoregion of the State, total phosphorus shall not exceed 0.09 mg/l, chlorophyll a shall not exceed 40 ug/l, and total nitrogen shall not exceed 1.50 mg/l.

c.	In evaluating the effects of nutrients upon the quality of lakes and other waters of the State, the Department may consider, but not be limited to, such factors as the hydrology and morphometry of the waterbody, the existing and projected trophic state, characteristics of the loadings, and other control mechanisms in order to protect the existing and classified uses of the waters.

d.	The Department shall take appropriate action, to include, but not limited to: establishing numeric effluent limitations in permits, establishing Total Maximum Daily Loads, establishing waste load allocations, and establishing load allocations for nutrients to ensure that the lakes attain and maintain the above narrative and numeric criteria and other applicable water quality standards.

e.	The criteria specific to lakes shall be applicable to all portions of the lake.  For this purpose, the Department shall define the applicable area to be that area covered when measured at full pool elevation.

10.		a. The water temperature of all Freshwaters which are free flowing shall not be increased more than 5oF (2.8oC) above natural temperature conditions and shall not exceed a maximum of 90oF (32.2oC) as a result of the discharge of heated liquids unless a different site-specific temperature standard as provided for in C.12 has been established, a mixing zone as provided in C.10 has been established, or a Section 316(a) determination under the Federal Clean Water Act has been completed.

b.	The weekly average water temperature of all Shellfish Harvesting, Class SA and Class SB waters shall not exceed 4oF (2.2oC) above natural conditions during the fall, winter or spring, and shall not exceed 1.5oF (0.8oC) above natural conditions during the summer as a result of the discharge of heated liquids unless a different site-specific temperature standard as provided for in C.12 has been established, a mixing zone as provided for in C.10 has been established, or a Section 316(a) determination under the Federal Clean Water Act has been completed.

c.	The weekly average water temperature of all Freshwaters which are lakes shall not be increased more than 5oF (2.8oC) above natural conditions and shall not exceed 90oF (32.2oC) as a result of the discharge or heated liquids unless a different site-specific temperature standard as provided for in C.12. has been established, a mixing zone as provided in C.10 has been established, or a Section 316(a) determination under the Federal Clean Water Act has been completed.

11.	Numeric criteria based on organoleptic data (prevention of undesirable taste and odor) are adopted herein.  Those substances and their criteria are listed in the appendix.  For those substances which have aquatic life and/or human health numeric criteria and organoleptic numeric criteria, the most stringent of the three shall be used for derivation of permit effluent limitations.

12.	Numeric criteria for the protection and maintenance of all classes of surface waters are adopted herein and are listed in Sections E, G, and the appendix. The numeric criteria developed and published by EPA are hereby incorporated into this regulation  Footnotes that further describe the application of these numeric criteria are included in the appendix.

a.	Application of numeric criteria to protect aquatic life.

(1)	The stated CMC value shall be used as an acute toxicity number for calculating permit effluent limitations.

(2)	The stated CCC value shall be used as a chronic toxicity number for calculating permit effluent limitations.

(3)	If metals concentrations for numeric criteria are hardness-dependent, the CMC and CCC concentrations shall be based on 25 milligrams/liter (mg/l) hardness (as expressed as CaCO3) if the ambient hardness is less than 25 mg/l.  Concentrations of hardness less than 400 mg/l maybe based on the actual mixed stream hardness if it is greater than 25 mg/l and less than 400 mg/l and 400 mg/l if the ambient hardness is greater than 400 mg/l.

.

(4)	If separate numeric criteria are given for fresh and salt waters, they shall be applied as appropriate.  In transitional tidal and estuarine areas, the Department shall apply the more stringent of the criteria to protect the existing and classified uses of the waters of the State.

(5)	The Department shall review new or revised EPA criteria for adoption by South Carolina when published in final form.

(6)	If the State develops site-specific criteria for any substances for which EPA has developed national criteria, the site-specific criteria shall supersede the national criteria.

b.	Application of numeric criteria to protect human health.

(1)	If separate numeric criteria are given for organism consumption, water and organism consumption (W/O), and drinking water Maximum Contaminant Levels (MCLs), they shall be applied as appropriate.  The most stringent of the criteria shall be applied to protect the existing and classified uses of the waters of the State.

(2)	The Department shall review new or revised EPA criteria for adoption by South Carolina when published in final form by EPA.

(3)	If the State develops site-specific criteria for any substances for which EPA has developed national criteria, the site-specific criteria shall supersede the national criteria.

(4)	Adoption of EPA human health criteria does not preclude the Department from considering health effects of other pollutants or from considering new or revised EPA criteria when developing effluent permit conditions.

c.	Application of criteria for the derivation of permit effluent limitations.

(1)	  Numeric criteria for substances listed in Sections E, G, and the appendix shall be used by the Department to derive NPDES permit effluent limitations at the applicable critical flow conditions as determined by the Department unless an exception is provided below.

(2)	When the derived permit effluent limitation based on aquatic life numeric criteria is below the practical quantitation limit for a substance, the derived permit effluent limitation shall include an accompanying statement in the permit that the practical quantitation limit using approved analytical methods shall be considered as being in compliance with the limit.  Appropriate biological monitoring requirements shall be incorporated into the permit to determine compliance with appropriate water quality standards.  Additionally, if naturally occurring instream concentration for a substance is higher than the derived permit effluent limitation, the Department may establish permit effluent limitations at a level higher than the derived limit, but no higher than the natural background concentration.  In such cases, the Department may require biological instream monitoring and/or WET testing.

(3)	When the derived permit effluent limitation based on human health numeric criteria is below the practical quantitation limit for a substance, the derived permit effluent limitation shall include an accompanying statement in the permit that the practical quantitation limit using approved analytical methods shall be considered as being in compliance with the limit.  Additionally, if naturally occurring instream concentration for a substance is higher than the derived permit effluent limitation, the Department may establish permit effluent limitations at a level higher than the derived limit, but no higher than the natural background concentration.

(4)	NPDES permit effluent limitations for metals shall normally be expressed on the permits as total recoverable metals, but the Department may utilize a federally-approved methodology to predict the dissolved fraction, partitioning coefficient, or the bioavailable portion of metals in calculating these limits.

(5)	Except as provided herein, where application of MCLs or W/O numeric criteria using annual average flow for carcinogens, 7Q10 (or 30Q5 if provided by the applicant) for noncarcinogens, or comparable tidal condition as determined by the Department results in permit effluent limitations more stringent than limitations derived from other applicable human health (organism consumption only), aquatic life, or organoleptic numeric values; MCLs or W/O shall be used in establishing permit effluent limitations for human health protection.  The Department may, after Notice of Intent included in a notice of a proposed NPDES permit in accordance with Regulation 61-9.124.10, determine that drinking water MCLs or W/O shall not apply to discharges to those waterbodies where there is: no potential to affect an existing or proposed drinking water source and no state-approved source water protection area.  For purposes of this section, a proposed drinking water source is one for which a complete permit application, including plans and specifications for the intake, is on file with the Department at the time of consideration of an NPDES permit application for a discharge that will affect or has the potential to affect the drinking water source.

(6)	Except as provided herein, where the Department may determine that an NPDES permitted discharge will not cause, have reasonable potential to cause, or contribute to an exceedence of the numeric criterion for turbidity under the following conditions:

i. the facility withdraws its surface intake water containing turbidity from the same body of water into which the discharge is made;

ii. the facility does not significantly concentrate or contribute additional turbidity to the discharged water;

iii. the facility does not alter the turbidity through chemical or physical means that would cause adverse water quality impacts to occur.

(7)	Site specific permit effluent limitations and alternate criteria less stringent than those derived in accordance with the above requirements may be derived where it is demonstrated that such limits and criteria shall maintain the existing and classified uses, adequate opportunity for public participation in such derivation process has occurred, and the effluent shall not cause criteria for human health to be exceeded.  Where a site-specific permit effluent limitation and alternate criterion has been derived, such derivation shall be subject to EPA review as appropriate.  Also, at a minimum, opportunity for input in derivation of a site-specific permit effluent limitation and alternate criterion shall be provided via public notice in NPDES permit notices.

(8)	In order to protect for the consumption use of shellfish, for SFH waters and other waters with approved shellfish harvesting uses, the stated value of 14/100 ml for fecal coliform shall be used as a monthly average number for calculating permit effluent limitations and the stated value of 43/100ml for fecal coliform shall be used as daily maximum number for calculating permit effluent limitations.

(9)	 In order to protect recreational uses for all waters of the State, the stated value of 200/100 ml for fecal coliform shall be used as a monthly average number for calculating permit effluent limitations and the stated value of 400/100ml for fecal coliform shall be used as daily maximum number for calculating permit effluent limitations.

(10)	All effluent permit limitations which include WET will require that the WET tests be conducted using Ceriodaphnia dubia (C. dubia), except as stated.  If the salinity of a discharge to a saline waterbody is high enough to be toxic to C. dubia, Mysidopsis bahia (M. bahia) will be used.  Low salinity discharges to saltwater may be tested using either C. dubia or M. bahia with salinity adjustment, as determined by the Department. The Department may consider an alternative species if it can be demonstrated that the proposed species meets the requirements of 40 CFR.136.4 and 5.  EPA test methods (40 CFR Part 136) for acute and chronic toxicity testing with freshwater organisms or marine and estuarine organisms must be followed.  Any modifications to species selection or the methodology used shall be approved by the EPA.

d.	Evaluation of ambient water quality

(1)	If the numeric criterion for toxic pollutants is lower than the analytical detection limit, the criterion is not considered violated if the ambient concentration is below the detection limit and the instream indigenous biological community is not adversely impacted.

(2)	If the ambient concentration is higher than the numeric criterion for toxic pollutants, the criterion is not considered violated if biological monitoring has demonstrated that the instream indigenous biological community is not adversely impacted.

(3)	In order to appropriately evaluate the ambient water quality for the bioavailability of the dissolved portion of hardness dependent metals, the Department may utilize a federally-approved methodology to predict the dissolved fraction or partitioning coefficient in determining compliance with water quality standards established in this regulation.

(4)	The assessment of fecal coliform for purposes of evaluating the shellfish harvesting use for South Carolinas Shellfish Management Units is conducted in accordance with provisions of S.C. Regulation 61-47, Shellfish.

13.	The Department may require biological or other monitoring in NPDES permits to further ascertain any bioaccumulative effects of pollutants.  Such monitoring may include analyses of fish and shellfish, macroinvertebrates, macrophytes, and/or sediments in order to assess the accumulation of pollutants in tissues or sediments that:

a.	may cause or have the potential to cause adverse impacts to the balanced indigenous aquatic community, and

b.	may cause or have the potential to cause adverse impacts to human health and/or terrestrial flora and fauna.

14.	The Department may consider other scientifically defensible published data which are appropriate for use in developing permit limits and evaluating water quality for constituents for which EPA has not developed national criteria or South Carolina has no standards.

a.	The Department shall apply a sensitivity factor to aquatic toxicity data unless, in the Departments judgement, the data represent a minimum of three appropriately sensitive species representing three taxonomic groups (plant, macroinvertebrate, and fish).

(1)	If only an acute toxicity effect concentration for a number of species for a particular pollutant is given as an LC50, the lowest concentration should be divided by an acute-to-chronic ratio (ACR) of 10 and a sensitivity factor of 3.3, for an acceptable instream concentration in order to protect against chronic toxicity effects.

(2)	If a chronic toxicity effect concentration for a number of species for a particular pollutant is given as a no observed effect concentration (NOEC), the lowest concentration should be divided by a sensitivity factor of 3.3 in order to protect against chronic toxicity to the most sensitive species.

b.	The Department must notify the permittee that other such data were used in developing permit limits and provide justification for their use.

15.	Tests or analytical methods to determine compliance or non-compliance with standards shall be made in accordance with methods and procedures approved by the Department and the EPA).  In making any tests or applying analytical methods to determine compliance or non-compliance with water quality standards, representative samples shall be collected in accordance with methods and procedures approved by the Department and the EPA.  Consideration of representative sample methods shall include the following:

a.	Surface water and ground water samples shall be collected so as to permit a realistic appraisal of quality and actual or potential damage to existing or classified water uses.  For ground waters, consideration shall be given to, but shall not be limited to, depth to water table, flow direction, and velocity.  For surface waters, time of day, flow, surface area, and depth shall be considered.

b.	Biological assessment methods may be employed in appropriate situations to determine abnormal nutrient enrichment, trophic condition, LC50, concentration of toxic substances, acceptable instream concentrations, or acceptable effluent concentrations for maintenance of a balanced indigenous aquatic community.

c.	Temporal distribution of samples in tidally influenced waters shall cover the full range of tidal conditions.

d.	Ambient toxicity tests used for screening purposes shall be conducted using Ceriodaphnia dubia (C. dubia), except as stated.  If salinity of a waterbody is high enough to be toxic to C. dubia, Mysidopsis bahia (M. bahia) will be used.  The Department may consider an alternative species if it can be demonstrated that the proposed species meets the requirements of 40 CFR.136.4 and 5.  EPA test methods (40 CFR Part 136) for acute and chronic toxicity testing with freshwater organisms or marine and estuarine organisms must be followed.  Any modifications to species selection or the methodology used shall be approved by the EPA.

F.  NARRATIVE BIOLOGICAL CRITERIA.

1.	Narrative biological criteria are contained in this regulation and are described throughout the sections where applicable.  The following are general statements regarding these narrative biological criteria.

a.	 Narrative biological criteria in Section A.4. describe the goals of the Department to maintain and improve all surface waters to a level that provides for the survival and propagation of a balanced indigenous aquatic community of fauna and flora.  These narrative criteria are determined by the Department based on the condition of the waters of the State by measurements of physical, chemical, and biological characteristics of the waters according to their classified uses.

b.	 Section C.10. describes narrative biological criteria relative to surface water mixing zones and specifies requirements necessary for the protection and propagation of a balanced indigenous aquatic community.

c.	Narrative biological criteria shall be consistent with the objective of maintaining and improving all surface waters to a level that provides for the survival and propagation of a balanced indigenous aquatic community of fauna and flora attainable in waters of the State; and in all cases shall protect against degradation of the highest existing or classified uses or biological conditions in compliance with the Antidegradation Rules contained in this regulation. Section D.1.a describes narrative biological criteria relative to activities in Outstanding National Resource Waters, Outstanding Resource Waters and Shellfish Harvesting Waters.

d.	In order to determine the biological quality of the waters of the State, it is necessary that the biological component be assessed by comparison to a reference condition(s) based upon similar hydrologic and watershed characteristics that represent the optimum natural condition for that system.  Such reference condition(s) or reaches of waterbodies shall be those observed to support the greatest variety and abundance of aquatic life in the region as is expected to be or would be with a minimal amount of disturbance from anthropogenic sources.  Impacts from urbanization and agriculture should be minimal and natural vegetation should dominate the land cover.  There should also be an appropriate diversity of substrate.  Reference condition(s) shall be determined by consistent sampling and reliable measures of selected indicative communities of flora and fauna as established by the Department and may be used in conjunction with acceptable physical, chemical, and microbial water quality measurements and records judged to be appropriate for this purpose.   Narrative biological criteria relative to activities in all waters are described in Section E.

e.	In the Class Descriptions, Designations, and Specific Standards for Surface Waters Section, all water use classifications protect for a balanced indigenous aquatic community of fauna and flora.  In addition, Trout Natural and Trout Put, Grow, and Take classifications protect for reproducing trout populations and stocked trout populations, respectively.

G.	CLASS DESCRIPTIONS, DESIGNATIONS, AND SPECIFIC STANDARDS FOR SURFACE WATERS.

1.	All surface waters of the State, except as discussed in Section C., shall be identified within one of the classes described below.  The Department may determine in accordance with Section 312 of the Clean Water Act that for some waterbodies (or portions of waterbodies), the designation of No Discharge Zone (NDZ) for Marine Sanitation Devices (MSDs) shall be enacted with application of the existing classified standards of the waterbody.  Those waters classified by name shall be listed in Regulation 61-69 along with the NDZ designation, if applicable.

2.	Where a surface water body is tributary to waters of a higher class, the quality of the water in the tributary shall be protected to maintain the standards of the higher classified receiving water.

3.	For items not listed in each class, criteria published pursuant to Sections 304(a) and 307(a) of the Federal Clean Water Act or other documents shall be used as guides to determine conditions which protect water uses.  Many of these criteria are listed in the appendix to this regulation.  For consideration of natural conditions, refer to Sections: C.9., D.4., E.10., E.12.c.(2), E.12.c.(3), F.4.d., G.4., G.6., and G.9.   For the following numeric criteria for turbidity (with the exception of Outstanding National Resource Waters, Outstanding Resource Waters, Trout waters, and Shellfish Harvesting Waters), compliance with these turbidity criteria may be considered to be met as long as the waterbody supports a balanced indigenous aquatic community when land management activities employ Best Management Practices (BMPs).  For consideration, BMPs must be in full compliance with all specifications governing the proper design, installation, operation and maintenance of such BMPs and all applicable permit conditions and requirements must be met.

4.	Outstanding National Resource Waters (ONRW) are freshwaters or saltwaters which constitute an outstanding national recreational or ecological resource.

QUALITY STANDARDS FOR OUTSTANDING

NATIONAL RESOURCE WATERS

ITEMS	STANDARDS

a. Color, dissolved oxygen, fecal	Water quality conditions shall be

coliform, pH, temperature, 	maintained and protected to the

turbidity, or other parameters.	extent of the Department's statutory

authority.  Numeric and narrative criteria 		for Class ONRW shall be those

applicable to the classification

of the waterbody immediately

prior to reclassification to

Class ONRW, including consideration

of natural conditions.

5.	In order to maintain the existing quality of Class ONRW waters the following additional standards apply:

ITEMS	STANDARDS

a.  Discharge from domestic,	None allowed.

industrial, or agricultural waste

treatment facilities;

aquaculture; open water

dredged spoil disposal.

b.  Stormwater and other nonpoint	None allowed.

source runoff including that from

agricultural uses or permitted

discharge from aquatic farms,

concentrated aquatic animal

production facilities, and

uncontaminated groundwater from

mining.

c.  Dumping or disposal of	None allowed.

garbage, cinders, ashes, oils,

sludge, or other refuse

d.  Activities or discharges from	Allowed if there will be no

waste treatment facilities in 	measurable impact on the

waters upstream or tributary	downstream ONRW consistent

to ONRW waters.	with Antidegradation Rules.

.6.	Outstanding Resource Waters (ORW) are freshwaters or saltwaters which constitute an outstanding recreational or ecological resource or those freshwaters suitable as a source for drinking water supply purposes with treatment levels specified by the Department.

QUALITY STANDARDS FOR OUTSTANDING

RESOURCE WATERS

ITEMS	STANDARDS

a. Color, dissolved oxygen, fecal	Water quality conditions shall be

coliform, pH, temperature, 	maintained and protected to the

turbidity, or other parameters.	extent of the Department's statutory

authority.  Numeric and narrative criteria 		for Class ORW shall be those

applicable to the classification

of the waterbody immediately

prior to reclassification to

Class ORW, including consideration

of natural conditions.

7.	In order to maintain the existing quality of Class ORW waters the following additional standards apply:

ITEMS	STANDARDS

a.  Discharge from domestic,	None allowed.

industrial, or agricultural waste

treatment facilities;

aquaculture; open water

dredged spoil disposal.

b.  Stormwater and other nonpoint	Allowed if water quality necessary

source runoff including that from	for existing and classified uses

agricultural uses or permitted	shall be maintained and protected

discharge from aquatic farms,	consistent with Antidegradation Rules. 	concentrated aquatic animal

production facilities, and

uncontaminated groundwater from

mining.

c.  Dumping or disposal of	None allowed.

garbage, cinders, ashes, oils,

sludge, or other refuse

d.  Activities or discharges from	Allowed if water quality necessary

waste treatment facilities in 	for existing and classified uses

waters upstream or tributary	shall be maintained and protected	to ORW waters.	consistent with Antidegradation Rules.

8.	Trout Waters.  The State recognizes three types of trout waters:  Natural; Put, Grow, and Take; and Put and Take.

a.	Natural (TN) are freshwaters suitable for supporting reproducing trout populations and a cold water balanced indigenous aquatic community of fauna and flora.  Also suitable for primary and secondary contact recreation and as a source for drinking water supply after conventional treatment in accordance with the requirements of the Department.  Suitable for fishing and the survival and propagation of a balanced indigenous aquatic community of fauna and flora.  Suitable also for industrial and agricultural uses.

b.	Put, Grow, and Take (TPGT) are freshwaters suitable for supporting growth of stocked trout populations and a balanced indigenous aquatic community of fauna and flora.  Also suitable for primary and secondary contact recreation and as a source for drinking water supply after conventional treatment in accordance with the requirements of the Department.  Suitable for fishing and the survival and propagation of a balanced indigenous aquatic community of fauna and flora.  Suitable also for industrial and agricultural uses.

c. Put and Take (TPT) are freshwaters suitable for primary and secondary contact recreation and as a source for drinking water supply after conventional treatment in accordance with the requirements of the Department.  Suitable for fishing and the survival and propagation of a balanced indigenous aquatic community of fauna and flora.  Suitable also for industrial and agricultural uses.   The standards of Freshwaters classification protect these uses.

9.	The standards below protect the uses of Natural and Put, Grow, and Take trout waters.

QUALITY STANDARDS FOR TROUT WATERS

ITEMS	STANDARDS

a. Garbage, cinders, ashes, oils,	None allowed.

sludge, or other refuse.

b. Treated wastes, toxic wastes,	None alone or in combination

deleterious substances, colored or 	with other substances or wastes in

other wastes, except those given in	sufficient amounts to be injurious

a. above.	to reproducing trout populations

in natural waters or stocked

populations in put, grow, and take

waters or in any manner adversely

affect the taste, color, odor, or

sanitary condition thereof or impair

the waters for any other best usage

as determined for the specific waters 			which are assigned to this class.

c.  Toxic pollutants listed in	As prescribed in Section E.

the appendix.	of this regulation.

d.  Stormwater and other nonpoint	Allowed if water quality necessary

source runoff including that from	for existing and classified uses

agricultural uses or permitted	shall be maintained and protected

discharge from aquatic farms,	consistent with Antidegradation Rules. 	concentrated aquatic animal

production facilities, and

uncontaminated groundwater from

mining.

e.  Dissolved oxygen.	Not less than 6 mg/1.

f.  Fecal coliform.	Not to exceed a geometric mean

of 200/100 ml, based on five 				consecutive samples during any

30 day period; nor shall more

than 10% of the total samples during 			any 30 day period exceed 400/100ml.

g.  pH.	Between 6.0 and 8.0.

h. Temperature.	Not to vary from levels existing under

natural conditions, unless determined

that some other temperature shall protect	the classified uses.

i. Turbidity.	Not to exceed 10 Nephelometric 			Turbidity Units (NTUs) or 10% above 			natural conditions, provided existing 			uses are maintained.

10.	 Freshwaters (FW) are freshwaters suitable for primary and secondary contact recreation and as a source for drinking water supply after conventional treatment in accordance with the requirements of the Department.  Suitable for fishing and the survival and propagation of a balanced indigenous aquatic community of fauna and flora.  Suitable also for industrial and agricultural uses.

QUALITY STANDARDS FOR FRESHWATERS

ITEMS	STANDARDS

a.  Garbage, cinders, ashes, oils,	None allowed.

sludge, or other refuse.

b.  Treated wastes, toxic wastes,	None alone or in combination

deleterious substances, colored 	with other substances or wastes in

or other wastes except those given	sufficient amounts to make the

in (a) above.	waters unsafe or unsuitable for

primary contact recreation or to

impair the waters for any other best 			usage as determined for the specific

waters which are assigned to this 			class.

c.  Toxic pollutants listed in	As prescribed in Section E

the appendix.	of this regulation.

d.  Dissolved Oxygen.	Daily average not less than 5.0 mg/l 			with a low of 4.0 mg/l.

e.  Fecal coliform.	Not to exceed a geometric mean of 			200/100 ml, based on five consecutive 			amples during any 30 day period; nor 			shall more than 10% of the total 			samples during any 30 day period 			exceed 400/100 ml.

f.  pH.	Between 6.0 and 8.5.

g. Temperature.	As prescribed in E.10. of this

regulation.

h. Turbidity *	Not to exceed 50 NTUs 				provided existing uses are 				maintained.

* Lakes only	Not to exceed 25 NTUs provided

existing uses are maintained.

11.	Shellfish Harvesting Waters* (SFH) are tidal saltwaters protected for shellfish harvesting and    uses listed in Class SA and Class SB.    Suitable for primary and secondary contact recreation, crabbing, and fishing.  Also suitable for the survival and propagation of a balanced indigenous aquatic community of marine fauna and flora.

QUALITY STANDARDS FOR SHELLFISH HARVESTING WATERS

ITEMS	STANDARDS

a.  Garbage, cinders, ashes, oils,	None allowed.

sludge, or other refuse.

b.  Treated wastes, toxic wastes,	None alone or in combination

deleterious substances, colored or 	with other substances or wastes in

other wastes except those given 	sufficient amounts to adversely

in (a) above.	affect the taste, color, odor, or

sanitary condition of clams,

mussels, or oysters for human 				consumption; or to impair the waters 			for any other best usage as determined 			for the specific waters which are 			assigned to this class.

c.  Toxic pollutants listed in	As prescribed in Section E

the appendix.	of this regulation.

d.  Dissolved oxygen.	Daily average not less than 5.0 mg/l 			with a low of 4 mg/l.

e.  Fecal coliform.	Not to exceed an MPN fecal

coliform geometric mean of 14/100

ml; nor shall more than 10% of the 			samples exceed an MPN of 43/100 ml.

f.  pH.	Shall not vary more than 3/10 of a pH 			unit above or below that of effluent-			free waters in the same geological area 			having a similar total salinity, 				alkalinity and temperature, but not 			lower than 6.5 or above 8.5.

g. Temperature.	As prescribed in E.10. of this 				regulation.

h. Turbidity	Not to exceed 25 NTUs provided			existing uses are maintained.

i. *The Department may designate prohibited areas where shellfish harvesting for market purposes or human consumption shall not be allowed, consistent with the Antidegradation Rule, Section D.1.a of this regulation.

12.	 Class SA are tidal saltwaters suitable for primary and secondary contact recreation, crabbing, and fishing, except harvesting of clams, mussels, or oysters for market purposes or human consumption and uses listed in Class SB.  Also suitable for the survival and propagation of a balanced indigenous aquatic community of marine fauna and flora.

QUALITY STANDARDS FOR CLASS SA WATERS

ITEMS	STANDARDS

a.  Garbage, cinders, ashes, oils,	None allowed.

sludge, or other refuse.

b.  Treated wastes, toxic wastes,	None alone or in combination

deleterious substances, colored, or 	with other substances or wastes in

other wastes except those given in 	sufficient amounts to make the

a. above.	waters unsafe or unsuitable for

primary contact recreation or to impair 			the waters for any other best usage as 			determined for the specific waters 			which are assigned to this class.

c.  Toxic pollutants listed in	As prescribed in Section E

the appendix.	of this regulation.

d.  Dissolved Oxygen.	Daily average not less than 5.0 mg/l 			with a low of 4.0 mg/l

e.  Fecal coliform.	Not to exceed a geometric mean of 			200/100 ml, based on five consecutive 			samples during any 30 day period; nor 			shall more than 10% of the total 			samples during any 30 day period 			exceed 400/100 ml.

f.  pH.	Shall not vary more than one-half of a 				pH unit above or below that of 					effluent-free waters in the same 					geological area having a similar total 				salinity, alkalinity and temperature, 				but not lower than 6.5 or above 8.5.

g. Temperature.	As prescribed in E.10. of this 					regulation.

h. Turbidity	Not to exceed 25 NTUs provided 				existing uses are maintained.

13.	Class SB are tidal saltwaters suitable for primary and secondary contact recreation, crabbing, and fishing, except harvesting of clams, mussels, or oysters for market purposes or human consumption.  Also suitable for the survival and propagation of a balanced indigenous aquatic community of marine fauna and flora.

QUALITY STANDARDS FOR CLASS SB WATERS

ITEMS	STANDARDS

a.  Garbage, cinders, ashes, oils,	None allowed.

sludge, or other refuse.

b.  Treated wastes, toxic wastes,	None alone or in combination

deleterious substances, colored, or 	with other substances or wastes in

other wastes except those given 	sufficient amounts to be harmful

in a. above.	to the survival of marine fauna

and flora or the culture or propagation 				thereof; to adversely affect the taste, 				color, odor, or sanitary condition of 				fish for human consumption; to make 				the waters unsafe or unsuitable for 				primary contact recreation; or 					to impair the waters for any other best 				usage as determined for the specific 				waters which are assigned to this class.

c.  Toxic pollutants listed in	As prescribed in Section E

the appendix.	of this regulation.

d.  Dissolved oxygen.	Not less than 4.0 mg/l.

e.  Fecal coliform.	Not to exceed a geometric mean of

200/100 ml based on five consecutive 				samples during any 30 day period; nor

shall more than 10% of the total 				samples examined during any 30 day 				period exceed 400/100 ml.

f.  pH.	Shall not vary more than one-half of a 				pH unit above or below that of 					effluent-free waters in the same 					geological area having a similar total 				salinity, alkalinity and temperature, 				but not lower than 6.5 or above 8.5.

g. Temperature.	As prescribed in E.10. of this 					regulation.

h. Turbidity	Not to exceed 25 NTUs provided 				existing uses are maintained.

H.  CLASS DESCRIPTIONS AND SPECIFIC STANDARDS FOR GROUND WATERS.

1.	All ground waters of the State, except within mixing zones, shall be identified within one of the classes described below.

2.	It is the policy of the Department to maintain the quality of ground water consistent with the highest potential uses.  Most South Carolina ground water is presently suitable for drinking water without treatment and the State relies heavily upon ground water for drinking water.  For this reason, all South Carolina ground water is classified Class GB effective on June 28,1985.

3.	The Department recognizes that Class GB may not be suitable for some ground water. Class GA is established for exceptionally valuable ground water and Class GC is established for ground water with little potential as an underground source of drinking water.

4.	In keeping with this policy the Department declares that effective June 28, 1985, all ground waters of the State shall be protected to a quality consistent with the use associated with the classes described herein.   Further, the Department may require the owner or operator of a contaminated site to restore the ground water quality to a level that maintains and supports the existing and classified uses (except classified uses within mixing zones, as described in this regulation).  For purposes of this section, the term operator means any person in control of, or having responsibility for, the operation of on-site activities or property and owner means a person or a previous person who has assumed legal ownership of a property through the provisions of a contract of sale or other legally binding transfer of ownership.  The term owner also means any person who owned, operated, or otherwise controlled activities at such site before the title or control of which was conveyed to a unit of State or local government due to bankruptcy, foreclosure, tax delinquency, abandonment, or similar means.  However, nothing in this section shall be construed to supersede specific statutory or regulatory provision that relieves owners or operators of certain contaminated sites from liability for restoration of groundwater, including, without limitation, S.C. Code 44-2-80 (b) and (c).  The term does not include a unit of State or local government which acquired ownership or control involuntarily through bankruptcy, tax delinquency, abandonment, or other circumstances in which the government involuntarily acquires title by virtue of its function as sovereign.  The exclusion provided under this paragraph shall not apply to any State or local government which has caused or contributed to the release or threatened release of a contaminant from the site, and such a State or local government shall be subject to these provisions in the same manner and to the same extent, both procedurally and substantively, as any nongovernmental entity.

5.	A ground water monitoring program approved by the Department may be required for any existing or proposed disposal system or other activities to determine the ground water quality affected by such systems or activities.  Such monitoring program may be required through the Department's permitting and certification programs.

6.	Those ground waters which are classified Class GA or Class GC after petition and proper administrative procedures other than Class GB shall be described by location and listed in Regulation 61-69.

7.	Class GA are those ground waters that are highly vulnerable to contamination because of the hydrological characteristics of the areas under which they occur and that are also characterized by either of the following two factors:

a.	Irreplaceable, in that no reasonable alternative source of drinking water is available to substantial populations; or

b.	Ecologically vital, in that the ground water provides the base flow for a particularly sensitive ecological system that, if polluted, would destroy a unique habitat.

8.	The standards below protect these ground waters:

QUALITY STANDARDS FOR CLASS GA GROUND WATERS

ITEMS	STANDARDS

a.  Treated wastes, toxic wastes,	None allowed.

deleterious substances, or

constituents thereof.

9.	Class GB.  All ground waters of the State, unless classified otherwise, which meet the definition of underground sources of drinking water (USDW) as defined in Section B.

QUALITY STANDARDS FOR CLASS GB GROUND WATERS

ITEMS	STANDARDS

a.  Inorganic chemicals.	Maximum contaminant levels as set 				forth in R.61-58, State Primary

Drinking Water Regulations.

b.  Organic chemicals.	Maximum contaminant levels as set 				forth in R.61-58, State Primary

Drinking Water Regulations.

c.  Man-made radionuclides, priority	Not to exceed concentrations or

pollutant volatile organic compounds, 	amounts such as to interfere with

pesticides, herbicides, polychlorinated 	use, actual or intended, as

biphenyls, any other synthetic organic 	determined by the Department.

compounds not specified above, treated

wastes, thermal wastes, deleterious

substances, colored wastes or other

wastes or constituents thereof.

10. 	Class GC are those ground waters not considered potential sources of drinking water and of limited beneficial use, i.e., ground waters that exceed a concentration of 10,000 mg/l total dissolved solids or are otherwise contaminated beyond levels that allow cleanup using methods reasonably employed in public water system treatment.  These ground waters also must not migrate to Class GA or Class GB ground waters or have a discharge to surface water that could cause degradation.

QUALITY STANDARDS FOR CLASS GC GROUND WATERS

ITEMS	STANDARDS

a.  Treated wastes, toxic wastes,	None which interfere with any

deleterious substances, or other 	existing use of an underground

constituents thereof.	source of drinking water.

I.	SEVERABILITY.  Should any section, paragraph, or other part of this regulation be declared invalid for any reason, the remainder shall not be affected.

APPENDIX: WATER QUALITY NUMERIC CRITERIA FOR THE PROTECTION OF

AQUATIC LIFE AND HUMAN HEALTH

This appendix contains three charts (priority pollutants, nonpriority pollutants, and organoleptic effects) of numeric criteria for the protection of human health and aquatic life.  The appendix also contains three attachments which address hardness conversions and application of ammonia criteria.  Footnotes specific to each chart follow the chart.  General footnotes pertaining to all are at the end of the charts prior to the attachments.  Please refer to the text of the regulation for other general information and specifications in applying these numeric criteria.

PRIORITY TOXIC POLLUTANTS

| Priority Pollutant |  | CAS Number | Freshwater Aquatic Life |  | Saltwater Aquatic Life |  | Human Health |  |  | FR Cite/ Source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | CMC (g/L) | CCC (g/L) | CMC (g/L) | CCC (g/L) | For Consumption of: |  |  |  |
|  |  |  |  |  |  |  | Water & Organism (g/L) | Organism Only (g/L) | MCL (g/L) |  |
| 1 | Antimony | 7440360 |  |  |  |  | 14 B, ee | 4300 B, ee | 6 ee | 57FR60848 SDWA |
| 2 | Arsenic | 7440382 | 340 A, D, K | 150 A, D, K | 69 A, D, Y | 36 A, D, Y | 0.018 C, R | 0.14 C, R | 50 C | 65FR31681 57FR60848 SDWA |
| 3 | Beryllium | 7440417 |  |  |  |  | J, ee | J, ee | 4 ee | 65FR31681 SDWA |
| 4 | Cadmium | 7440439 | 0.95 D, E, K | 0.83 D, E, K | 43 D, Y | 9.3 D, Y | J, ee | J, ee | 5 ee | 65FR31681 SDWA |
| 5a | Chromium III | 16065831 | 580 D, E, K | 28 D, E, K |  |  | J, ee | J, ee | 100 Total ee | EPA820/B-96-001 65FR31681 SDWA |
| 5b | Chromium VI | 18540299 | 16 D, K | 11 D, K | 1,100 D, Y | 50 D, Y | J, ee | J, ee | 100 Total ee | 65FR31681 SDWA |
| 6 | Copper | 7440508 | 3.8 D, E, K, Z | 2.9 D, E, K, Z | 5.8 D, Z, cc | 3.7 D, Z, cc | 1,300 T, ee |  |  | 65FR31681 |
| 7 | Lead | 7439921 | 14 D, E, Y | 0.54 D, E, Y | 220 D, Y | 8.5 D, Y |  |  |  | 65FR31681 |
| 8 | Mercury | 7439976 | 1.6 D, K, dd | 0.91 D, K, dd | 2.1 D, bb, dd | 1.1 D, bb, dd | 0.050 B, ee | 0.051 B, ee | 2ee | 65FR31681 SDWA |
| 9 | Nickel | 7440020 | 150 D, E, K | 16 D, E, K | 75 D, Y | 8.3 D, Y | 610 B, ee | 4, 600 B, ee |  | 65FR31681 |
| 10 | Selenium | 7782492 | L, Q, S | 5.0 S | 300 D, Y, aa | 71 D, Y, aa | 170 Z, ee | 11, 000 ee | 50 ee | 65FR31681 IRIS 09/01/91 SDWA |
| 11 | Silver | 7440224 | 0.37 D, E, G |  | 2.3 D, G |  |  |  |  | 65FR31681 |
| 12 | Thallium | 7440280 |  |  |  |  | 1.7 B, ee | 6.3 B, ee | 2 ee | 57FR60848 SDWA |
| 13 | Zinc | 7440666 | 37 D, E, K | 37 D, E, K | 95 D, Y | 86 D, Y | 9,100 T, ee | 69,000 T, ee |  | 65FR31681 IRIS 10/01/92 |
| 14 | Cyanide | 57125 | 22 K, P | 5.2 K, P | 1 P, Y | 1 P, Y | 700 B, ee | 220,000 B, H, ee | 200 ee | EPA820/B-96-001 57FR60848 SDWA |
| 15 | Asbestos | 1332214 |  |  |  |  |  |  | 7 million fibers/L I, ee | 57FR60848 |
| 16 | 2, 3, 7, 8-TCDD (Dioxin) | 1746016 |  |  |  |  |  | 0.12 ppq C | 3.0E  8 C | State Standard SDWA |
| 17 | Acrolein | 107028 |  |  |  |  | 320 ee | 780 ee |  | 57FR60848 |
| 18 | Acrylonitrile | 107131 |  |  |  |  | 0.059 B, C | 0.66 B, C |  | 57FR60848 |
| 19 | Benzene | 71432 |  |  |  |  | 1.2 B, C | 71 B, C | 5 C | 65FR31681 SDWA |
| 20 | Bromoform | 75252 |  |  |  |  | 4.3 B, C | 360 B, C | 100 Total THMs C | 65FR31681 SDWA |
| 21 | Carbon Tetrachloride | 56235 |  |  |  |  | 0.25 B, C | 4.4 B, C | 5 C | 57FR60848 SDWA |
| 22 | Chlorobenzene | 108907 |  |  |  |  | 680 B | 21,000 B, H | 100 | 57FR60848 SDWA |
| 23 | Chlorodibromomethane | 124481 |  |  |  |  | 0.41 B, C | 34 B, C | 100 Total THMs C | 65FR31681 SDWA |
| 24 | Chloroform | 67663 |  |  |  |  | 5.7 B, C | 470 B, C | 100 Total THMs C | 65FR31681 SDWA |
| 25 | Dichlorobromomethane | 75274 |  |  |  |  | 0.56 B, C | 46 B, C | 100 Total THMs C | 65FR31681 SDWA |
| 26 | 1, 2-Dichloroethane | 107062 |  |  |  |  | 0.38 B, C | 99 B, C | 5 C | 57FR60848 SDWA |
| 27 | 1, 1-Dichloroethylene | 75354 |  |  |  |  | 0.057 B, C | 3.2 B, C | 7 C | 57FR60848 SDWA |
| 28 | 1, 2-Dichloropropane | 78875 |  |  |  |  | 0.52 B, C | 39 B, C | 5 C | 65FR31681 SDWA |
| 29 | 1, 3-Dichloropropene | 542756 |  |  |  |  | 10 B, ee | 1,700 B, ee |  | 57FR60848 |
| 30 | Ethylbenzene | 100414 |  |  |  |  | 3,100 B, ee | 29,000 B, ee | 700 ee | 65FR31681 SDWA |
| 31 | Methyl Bromide | 74839 |  |  |  |  | 48 B, ee | 4000 B, ee |  | 65FR31681 |
| 32 | Methylene Chloride | 75092 |  |  |  |  | 4.7 B, C | 1600 B, C | 5 C | 65FR31681 SDWA |
| 33 | 1, 1, 2, 2-Tetrachloroethane | 79345 |  |  |  |  | 0.17 B, C | 11 B, C |  | 57FR60848 |
| 34 | Tetrachloroethylene | 127184 |  |  |  |  | 0.8 C | 8.85 C | 5 C | 57FR60848 SDWA |
| 35 | Toluene | 108883 |  |  |  |  | 6,800 B, ee | 200,000 B, ee | 1000 ee | 65FR31681 SDWA |
| 36 | 1, 2-Trans-Dichloroethylene | 156605 |  |  |  |  | 700 B, ee | 140,000 B, ee | 100 ee | 65FR31681 SDWA |
| 37 | 1, 1, 1-Trichloroethane | 71556 |  |  |  |  | J, ee | J, ee | 200 ee | 65FR31681 SDWA |
| 38 | 1, 1, 2-Trichloroethane | 79005 |  |  |  |  | 0.60 B, C | 42 B, C | 5 C | 57FR60848 SDWA |
| 39 | Trichloroethylene | 79016 |  |  |  |  | 2.7 C | 81 C | 5 C | 57FR60848 SDWA |
| 40 | Vinyl Chloride | 75014 |  |  |  |  | 2.0 C | 525 C | 2 C | 57FR60848 SDWA |
| 41 | 2-Chlorophenol | 95578 |  |  |  |  | 120 B, T, ee | 400 B, T, ee |  | 65FR31681 |
| 42 | 2, 4-Dichlorophenol | 120832 |  |  |  |  | 93 B, T, ee | 790 B, T, ee |  | 57FR60848 |
| 43 | 2, 4-Dimethylphenol | 105679 |  |  |  |  | 540 B, T, ee | 2,300 B, T, ee |  | 65FR31681 |
| 44 | 2-Methyl- 4, 6-Dinitrophenol | 534521 |  |  |  |  | 13.4 ee | 765 ee |  | 57FR60848 |
| 45 | 2, 4-Dinitrophenol | 51285 |  |  |  |  | 70 B, ee | 14,000 B, ee |  | 57FR60848 |
| 46 | 3-Methyl- 4-Chlorophenol | 59507 |  |  |  |  | T, ee | T, ee |  | Gold Book |
| 47 | Pentachlorophenol | 87865 | 19 F, K | 15 F, K | 13 Y | 7.9 Y | 0.28 B, C | 8.2 B, C, H | 1 C | 65FR31681 SDWA |
| 48 | Phenol | 108952 |  |  |  |  | 21,000 B, T, ee | 4,600,000 B, H, U, ee |  | 65FR31681 57FR60848 |
| 49 | 2, 4, 6-Trichlorophenol | 88062 |  |  |  |  | 2.1 B, C, T | 6.5 B, C |  | 65FR31681 |
| 50 | Acenaphthene | 83329 |  |  |  |  | 1,200 B, T, ee | 2,700 B, U, ee |  | 65FR31681 |
| 51 | Anthracene | 120127 |  |  |  |  | 9,600 B, ee | 110,000 B, ee |  | 65FR31681 |
| 52 | Benzidine | 92875 |  |  |  |  | 0.00012 B, C | 0.00054 B, C |  | 57FR60848 |
| 53 | Benzo (a) Anthracene | 56553 |  |  |  |  | 0.0044 B, C | 0.049 B, C |  | 65FR31681 |
| 54 | Benzo (a) Pyrene | 50328 |  |  |  |  | 0.0044 B, C | 0.049 B, C | 0.2 C | 65FR31681 SDWA |
| 55 | Benzo (b) Fluoranthene | 205992 |  |  |  |  | 0.0044 B, C | 0.049 B, C |  | 65FR31681 |
| 56 | Benzo (k) Fluoranthene | 207089 |  |  |  |  | 0.0044 B, C | 0.049 B, C |  | 65FR31681 |
| 57 | Bis 2-Chloroethyl Ether | 111444 |  |  |  |  | 0.031 B, C | 1.4 B, C |  | 57FR60848 |
| 58 | Bis 2-Chloroisopropyl Ether | 39638329 |  |  |  |  | 1,400 B, ee | 170,000 B, ee |  | 65FR31681 57FR60848 |
| 59 | Bis 2-EthylhexylPhthalate (DEHP) | 117817 | V | V | V | V | 1.8 B, C | 5.9 B, C | 6 C | 57FR60848 SDWA |
| 60 | 2-Chloronaphthalene | 91587 |  |  |  |  | 1,700 B | 4,300 B |  | 65FR31681 |
| 61 | Chrysene | 218019 |  |  |  |  | 0.0044 B, C | 0.049 B, C |  | 65FR31681 |
| 62 | Dibenzo (a, h) Anthracene | 53703 |  |  |  |  | 0.0044 B, C | 0.049 B, C |  | 65FR31681 |
| 63 | 1, 2-Dichlorobenzene | 95501 |  |  |  |  | 2,700 B, ee | 17,000 B, ee | 600 ee | 65FR31681 SDWA |
| 64 | 1, 3-Dichlorobenzene | 541731 |  |  |  |  | 400 ee | 2,600 ee |  | 65FR31681 |
| 65 | 1, 4-Dichlorobenzene | 106467 |  |  |  |  | 400 ee | 2600 ee | 75 ee | 65FR31681 SDWA |
| 66 | 3, 3'-Dichlorobenzidine | 91941 |  |  |  |  | 0.04 B, C | 0.077 B, C |  | 57FR60848 |
| 67 | 2, 4-Dinitrotoluene | 121142 |  |  |  |  | 0.11 C | 9.1 C |  | 57FR60848 |
| 68 | 1, 2-Diphenylhydrazine | 122667 |  |  |  |  | 0.040 B, C | 0.54 B, C |  | 57FR60848 |
| 69 | Fluoranthene | 206440 |  |  |  |  | 300 B, ee | 370 B, ee |  | 65FR31681 |
| 70 | Fluorene | 86737 |  |  |  |  | 1,300 B, ee | 14,000 B, ee |  | 65FR31681 |
| 71 | Hexachlorobenzene | 118741 |  |  |  |  | 0.00075 B, C | 0.00077 B, C | 1 C | 65FR31681 SDWA |
| 72 | Hexachlorobutadiene | 87683 |  |  |  |  | 0.44 B, C | 50 B, C |  | 57FR60848 |
| 73 | Hexachlorocyclo-pentadiene | 77474 |  |  |  |  | 240 B, T, ee | 17,000 B, H, U, ee | 50 ee | 57FR60848 SDWA |
| 74 | Hexachloroethane | 67721 |  |  |  |  | 1.9 B, C | 8.9 B, C |  | 57FR60848 |
| 75 | Ideno 1, 2, 3 - (cd) Pyrene | 193395 |  |  |  |  | 0.0044 B, C | 0.049 B, C |  | 65FR31681 |
| 76 | Isophorone | 78591 |  |  |  |  | 36 B, C | 2,600 B, C |  | IRIS 11/01/97 |
| 77 | Nitrobenzene | 98953 |  |  |  |  | 17 B, ee | 1,900 B, H, U, ee |  | 57FR60848 |
| 78 | N-Nitrosodimethylamine | 62759 |  |  |  |  | 0.00069 B, C | 8.1 B, C |  | 57FR60848 |
| 79 | N-Nitrosodi-n-Propylamine | 621647 |  |  |  |  | 0.005 B, C | B, C |  | 65FR31681 |
| 80 | N-Nitrosodiphenylamine | 86306 |  |  |  |  | 5.0 B, C | B, C |  | 57FR60848 |
| 81 | Pyrene | 129000 |  |  |  |  | 960 B, ee | 11,000 B, ee |  | 65FR31681 |
| 82 | 1, 2, 4-Trichlorobenzene | 120821 |  |  |  |  | 260 ee | 940 ee | 70 ee | IRIS 11/01/96 SDWA |
| 83 | Aldrin | 309002 | 3.0 G |  | 1.3 G |  | 0.00013 B, C | 0.00014 B, C |  | 65FR31681 |
| 84 | alpha-BHC | 319846 |  |  |  |  | 0.0039 B, C | 0.013 B, C |  | 65FR31681 |
| 85 | beta-BHC | 319857 |  |  |  |  | 0.014 B, C | 0.046 B, C |  | 65FR31681 |
| 86 | gamma-BHC (Lindane) | 58899 | 0.95 K |  | 0.16 G |  | 0.019 C | 0.063 C | 0.2 C | 65FR31681 SDWA |
| 87 | Chlordane | 57749 | 2.4 G | 0.0043 G, X | 0.09 G | 0.004 G, X | 0.0021 B, C | 0.0022 B, C | 2 C | 65FR31681 IRIS02/07/98 SDWA |
| 88 | 4, 4'-DDT | 50293 | 1.1 G | 0.001 G, X | 0.13 G | 0.001 G, X | 0.00059 B, C | B, C |  | 65FR31681 |
| 89 | 4, 4'-DDE | 72559 |  |  |  |  | 0.00059 B, C | 0.00059 B, C |  | 65FR31681 |
| 90 | 4, 4'-DDD | 72548 |  |  |  |  | 0.00083 B, C | 0.00084 B, C |  | 65FR31681 |
| 91 | Dieldrin | 60571 | 0.24 K | 0.056 K, N | 0.71 G | 0.0019 G, X | 0.00014 B, C | 0.00014 B, C |  | 65FR31681 |
| 92 | alpha-Endosulfan | 959988 | 0.22 W | 0.056 G, W | 0.034 G, W | 0.0087 G, W | 110 B, ee | 240 B, ee |  | 65FR31681 |
| 93 | beta-Endosulfan | 33213659 | 0.22 G, W | 0.056 G, W | 0.034 G, W | 0.0087 G, W | 110 B, ee | 240 B, ee |  | 65FR31681 |
| 94 | Endosulfan Sulfate | 1031078 |  |  |  |  | 110 B, ee | 240 B, ee |  | 65FR31681 |
| 95 | Endrin | 72208 | 0.086 K | 0.036 K, N | 0.037 G | 0.0023 G, X | 0.76 B, ee | 0.81 B, H, ee | 2 ee | 65FR31681 SDWA |
| 96 | Endrin Aldehyde | 7421934 |  |  |  |  | 0.76 B, ee | 0.81 B, H, ee |  | 65FR31681 |
| 97 | Heptachlor | 76448 | 0.52 G | 0.0038 | 0.053 G | 0.0036 G, X | 0.00021 B, C | 0.00021 B, C | 0.4 C | 65FR31681 SDWA |
| 98 | Heptachlor Epoxide | 1024573 | 0.52 G, V | 0.0038 G, U, X | 0.053 G, V | 0.0036 G, U, X | 0.00010 B, C | 0.00011 B, C | 0.2 C | 65FR31681 SDWA |
| 99 | Polychlorinated Biphenyls PCBs: | -- |  | 0.014 M, X |  | 0.03 M, X | 0.00017 B, C, O | 0.00017 B, C, O | 0.5 C | 65FR31681 63FR16182 SDWA |
| 100 | Toxaphene | 8001352 | 0.73 | 0.0002 X | 0.21 | 0.0002 X | 0.00073 B, C | 0.00075 B, C | 3 C | 65FR31681 SDWA |

Footnotes:

A	This water quality criterion was derived from data for arsenic (III), but is applied here to total arsenic, which might imply that arsenic (III) and arsenic  (V) are equally toxic to aquatic life and that their toxicities are additive.  In the arsenic criteria document (EPA 440/5-84-033, January 1985), Species Mean Acute Values are given for both arsenic (III) and arsenic (V) for five species and the ratios of the SMAVs for each species range from 0.6 to 1.7.  Chronic values are available for both arsenic (III) and arsenic (V) for one species; for the fathead minnow, the chronic value for arsenic (V) is 0.29 times the chronic value for arsenic (III).  No data are known to be available concerning whether the toxicities of the forms of arsenic to aquatic organisms are additive.

B	This criterion has been revised to reflect The Environmental Protection Agencys q1* or RfD, as contained in the Integrated Risk Information System (IRIS) as of April 8, 1998.  The fish tissue bioconcentration factor (BCF) from the 1980 Ambient Water Quality Criteria document was retained in +each case.

C	This criterion is based on carcinogenicity of 10-6 risk.   As prescribed in Section E of this regulation, application of this criterion for permit effluent limitations requires the use annual average flow or comparable tidal condition as determined by the Department

D	Freshwater and saltwater criteria for metals are expressed in terms of total recoverable metals.  As allowed in Section E of this regulation, these criteria may be expressed as dissolved metal for the purposes of deriving permit effluent limitations.  The dissolved metal water quality criteria value may be calculated by using these 304(a) aquatic life criteria expressed in terms of total recoverable metal, and multiplying it by a conversion factor (CF).  The term "Conversion Factor" (CF) represents the conversion factor for converting a metal criterion expressed as the total recoverable fraction  in the water column to a criterion expressed as the dissolved fraction in the water column. (Conversion Factors for saltwater CCCs are not currently available.   Conversion factors derived for saltwater CMCs have been used for both saltwater CMCs and CCCs).  See "Office of Water Policy and Technical Guidance on Interpretation and Implementation of Aquatic Life Metals Criteria,   October 1, 1993, by Martha G. Prothro, Acting Assistant Administrator for Water, available from the Water Resource center, USEPA, 401 M St., SW,  mail code RC4100, Washington, DC 20460; and 40CFR131.36(b)(1).  Conversion Factors can be found in Attachment 1 - Conversion Factors for Dissolved Metals.

E	The freshwater criterion for this metal is expressed as a function of hardness (mg/L) in the water column.  The value given here corresponds to a hardness of 25 mg/L as expressed as CaCO3.  Criteria values for other hardness may be calculated from the following:  CMC (dissolved) = exp{mA [ln( hardness)]+ bA} (CF), or CCC (dissolved) = exp{mC [ln (hardness)]+ bC} (CF) and the parameters specified in Attachment 2 - Parameters for Calculating Freshwater Dissolved Metals Criteria That Are Hardness-Dependent.

F	Freshwater aquatic life values for pentachlorophenol are expressed as a function of pH, and are calculated as follows:  CMC = exp(1.005(pH)-4.869); CCC = exp(1.005(pH)-5.134).  Values displayed in table correspond to a pH of 7.8.

G	This criterion is based on 304(a) aquatic life criterion issued in 1980, and was issued in one of the following documents: Aldrin/Dieldrin (EPA 440/5-80-019),  Chlordane (EPA 440/5-80-027), DDT (EPA 440/5-80-038), Endosulfan (EPA 440/5-80-046), Endrin (EPA 440/5-80-047), Heptachlor (440/5-80-052),  Hexachlorocyclohexane (EPA 440/5-80-054), Silver (EPA 440/5-80-071).  The Minimum Data Requirements and derivation procedures were different in the 1980 Guidelines than in the 1985 Guidelines.  For example, a CMC derived using the 1980 Guidelines was derived to be used as an instantaneous maximum.  If assessment is to be done using an averaging period, the values given should be divided by 2 to obtain a value that is more comparable to a CMC derived using the 1985 Guidelines.

H 	No criterion for protection of human health from consumption of aquatic organisms excluding water was presented in the 1980 criteria document or in the 1986 Quality Criteria for Water.  Nevertheless, sufficient information was presented in the 1980 document to allow the calculation of a criterion, even though the results of such a calculation were not shown in the document.

I 	This criterion for asbestos is the Maximum Contaminant Level (MCL) developed under the Safe Drinking Water Act (SDWA) and the National Primary Drinking Water Regulation (NPDWR).

J	EPA has not calculated a 304(a) human health criterion for this contaminant.  The criterion is the Maximum Contaminant Level developed under the Safe Drinking Water Act (SDWA) and the National Primary Drinking Water Regulation (NPDWR).

K	This criterion is based on a 304(a) aquatic life criterion that was issued in the 1995 Updates: Water Quality Criteria Documents for the Protection of Aquatic Life in Ambient Water, (EPA-820-B-96-001, September 1996).  This value was derived using the GLI Guidelines (60FR15393-15399, March 23, 1995; 40CFR132 Appendix A); the difference between the 1985 Guidelines and the GLI Guidelines are explained on page iv of the 1995 Updates.  None of the decisions concerning the derivation of this criterion were affected by any considerations that are specific to the Great Lakes.

L	This appendix contains aquatic life criteria for selenium that are the same as those published in the CTR, with the exception of removal of the acute criterion based on EPAs June 2, 2000 revocation of the value.

M	PCBs are a class of chemicals which include aroclors, 1242, 1254, 1221, 1232, 1248, 1260, and 1016, CAS numbers 53469219, 11097691, 11104282, 11141165, 12672296, 11096825 and 12674112 respectively.  The aquatic life criteria apply to this set of PCBs.

N	The derivation of the CCC for this pollutant did not consider exposure through the diet, which is probably important for aquatic life occupying upper trophic levels.

O	This criterion applies to total PCBs, i.e., the sum of all congener or all isomer analyses.

P	This water quality criterion is expressed as g free cyanide (as CN)/L.

Q	This value was announced (61FR58444-58449, November 14, 1996) as a proposed GLI 303 (c) aquatic life criterion.

R	This water quality criterion refers to the inorganic form only.

S	This water quality criterion is expressed in terms of total recoverable metal in the water column.  It is scientifically acceptable to use the conversion factor of 0.922 that was used in the GLI to convert this to a value that is expressed in terms of dissolved metal.

T	The organoleptic effect criterion is more stringent than the value for priority toxic pollutants.

U	This value was derived from data for heptachlor and the criteria document provides insufficient data to estimate the relative toxicities of heptachlor and heptachlor epoxide.

V	There is a full set of aquatic life toxicity data that show that DEHP is not toxic to aquatic organisms at or below its solubility limit.

W	This value was derived from data for endosulfan and is most appropriately applied to the sum of alpha-endosulfan and beta-endosulfan.

X 	This CCC is based on the Final Residue value procedure in the 1985 Guidelines.  Since the publication of the Great Lakes Aquatic Life Criteria Guidelines in 1995 (60FR15393-15399, March 23, 1995), the EPA no longer uses the Final Residue value procedure for deriving CCCs for new or revised 304(a) aquatic life criteria.

Y	This water quality criterion is based on a 304(a) aquatic life criterion that was derived using the 1985 Guidelines (Guidelines for Deriving Numerical National Water Quality Criteria for the Protection of Aquatic Organisms and Their Uses, PB85-227049, January 1985) and was issued in one of the following criteria documents: Arsenic (EPA 440/5-84-033), Cadmium (EPA 440/5-84-032), Chromium (EPA 440/5-84-029), Copper (EPA 440/5-84-031), Cyanide (EPA 440/5-84-028), Lead (EPA 440/5-84-027), Nickel (EPA 440/5-86-004), Pentachlorophenol (EPA 440/5-86-009), Toxaphene, (EPA 440/5-86-006), Zinc (EPA 440/5-87- 003).

Z	When the concentration of dissolved organic carbon is elevated, copper is substantially less toxic and use of Water-Effect Ratios might be appropriate.

aa	The selenium criteria document (EPA 440/5-87-006, September 1987) provides that if selenium is as toxic to saltwater fishes in the field as it is to freshwater fishes in the field, the status of the fish community should be monitored whenever the concentration of selenium exceeds 5.0 g/L in salt water because the saltwater CCC does not take into account uptake via the food chain.

bb	This water quality criterion was derived on page 43 of the mercury criteria document (EPA 440/5-84-026, January 1985).  The saltwater CCC of 0.025 ug/L given on page 23 of the criteria document is based on the Final Residue value procedure in the 1985 Guidelines.  Since the publication of the Great Lakes Aquatic Life criteria Guidelines in 1995 (60FR15393-15399, March 23, 1995), the EPA no longer uses the Final Residue value procedure for deriving CCCs for new or revised 304(a) aquatic life criteria.

cc	This water quality criterion was derived in Ambient Water Quality Criteria Saltwater Copper Addendum (Draft, April 14, 1995) and was promulgated in the Interim Final National Toxics Rule (60FR22228-222237, May 4, 1995).

dd	This water quality criterion was derived from data for inorganic mercury (II), but is applied here to total mercury.  If a substantial portion of the mercury in the water column is methylmercury, this criterion will probably be under protective.  In addition, even though inorganic mercury is converted  to methylmercury and methylmercury bioaccumulates to a great extent, this criterion does not account for uptake via the food chain because sufficient data were not available when the criterion was derived.

ee	This criterion is a noncarcinogen.  As prescribed in Section E of this regulation, application of this criterion for determining permit effluent limitations requires the use of 7Q10 or 30Q5 (if provided by the applicant), or comparable tidal condition as determined by the Department.

NON PRIORITY POLLUTANTS

| Non Priority Pollutant |  | CAS Number | Freshwater Aquatic Life |  | Saltwater Aquatic Life |  | Human Health |  |  | FR Cite/Source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | CMC (g/L) | CCC (g/L) | CMC (g/L) | CCC (g/L) | For Consumption of: |  | MCL (g/L) |  |
|  |  |  |  |  |  |  | Water & Organism (g/L) | Organism Only (g/L) |  |  |
| 1 | Alachlor | -- |  |  |  |  |  |  | 2 M | SDWA |
| 2 | Ammonia | 7664417 | CRITERIA ARE pH AND TEMPERATURE DEPENDENT - SEE DOCUMENT FOR DETAILS C |  |  |  |  |  |  | EPA822-R99-014 EPA440/5-88-004 |
| 3 | Aesthetic Qualities | -- | NARRATIVE STATEMENT AND NUMERIC CRITERIA - SEE TEXT |  |  |  |  |  |  | Gold Book |
| 4 | Atrazine | -- |  |  |  |  |  |  | 3 M | SDWA |
| 5 | Bacteria | -- | FOR PRIMARY CONTACT RECREATION AND SHELLFISH USES -- SEE TEXT |  |  |  |  |  |  | Gold Book |
| 6 | Barium | 7440393 |  |  |  |  | 1000 A, L |  | 2000 L | Gold Book |
| 7 | Carbofuran | 1563662 |  |  |  |  |  |  | 40 L | SDWA |
| 8 | Chlorine | 7782505 | 19 L | 11 L | 13 L | 7.5 L |  |  |  | Gold Book |
| 9 | Chlorophenoxy Herbicide 2, 4, 5, -TP | 93721 |  |  |  |  | 10 A, L |  | 50 L | Gold Book SDWA |
| 10 | Chlorophenoxy Herbicide 2, 4-D | 94757 |  |  |  |  | 100 A, L |  | 70 L | Gold Book SDWA |
| 11 | Chlorophyll a | -- | NARRATIVE STATEMENT AND NUMERIC CRITERIA - SEE TEXT |  |  |  |  |  |  | State Standard |
| 12 | Chloropyrifos | 2921882 | 0.083 F, L | 0.041 F, L | 0.011 F, L | 0.0056 F, L |  |  |  | Gold Book |
| 13 | Color | -- | NARRATIVE STATEMENT - SEE TEXT |  |  |  |  |  |  | State Standard |
| 14 | Dalapon | 75990 |  |  |  |  |  |  | 200 L | SDWA |
| 15 | Demeton | 8065483 |  | 0.1 E, L |  | 0.1 E, L |  |  |  | Gold Book |
| 16 | 1, 2-Dibromo-3-chloropropane (DBCP) | 96128 |  |  |  |  |  |  | 0.2 M | SDWA |
| 17 | Di(2-ethylhexyl) adipate | 103231 |  |  |  |  |  |  | 400 L | SDWA |
| 18 | Dinoseb | 88857 |  |  |  |  |  |  | 7 L | SDWA |
| 19 | Dinitrophenols | 25550587 |  |  |  |  | 70 L | 14, 000 L |  | Gold Book |
| 20 | Diquat | 85007 |  |  |  |  |  |  | 20 L | SDWA |
| 21 | Endothall | 145733 |  |  |  |  |  |  | 100 L | SDWA |
| 22 | Ether, Bis Chloromethyl | 542881 |  |  |  |  | 0.00013 D, L | 0.00078 D, L |  | IRIS 01/01/91 |
| 23 | Ether, Cis-1, 2-dichloroethylene | 156592 |  |  |  |  |  |  | 70 L | SDWA |
| 24 | Ethylene dibromide | -- |  |  |  |  |  |  | 0.05 M | SDWA |
| 25 | Fluoride | 7681494 |  |  |  |  |  |  | 4000 L | SDWA |
| 26 | Glyphosate | 1071836 |  |  |  |  |  |  | 700 L | SDWA |
| 27 | Guthion | 86500 |  | 0.01 E, L |  | 0.01 E, L |  |  |  | Gold Book |
| 28 | Hexachlorocyclo-hexane-Technical | 319868 |  |  |  |  | 0.0123 L | 0.0414 L |  | Gold Book |
| 29 | Iron | 7439896 |  | 1000 E, L |  |  | 300 A, L |  |  | Gold Book |
| 30 | Malathion | 121755 |  | 0.1 E, G, L |  | 0.1 E, G, L |  |  |  | Gold Book |
| 31 | Manganese | 7439965 |  |  |  |  | 50 A, L | 100 A, L |  | Gold Book |
| 32 | Methoxychlor | 72435 |  | 0.03 E, G, L |  | 0.03 E, G, L | 100 A, L |  | 40 L | Gold Book SDWA |
| 33 | Mirex | 2385855 |  | 0.001 E, G, L |  | 0.001 E, G, L |  |  |  | Gold Book |
| 34 | Nitrates | 14797558 |  |  |  |  |  |  | 10, 000 L | SDWA |
| 35 | Nitrites | 14797650 |  |  |  |  |  |  | 1000 L | SDWA |
| 36 | Nitrogen, Total | -- | NARRATIVE STATEMENT AND NUMERIC CRITERIA - SEE TEXT |  |  |  |  |  |  | State Standard |
| 37 | Nitrosamines | -- |  |  |  |  | 0.0008 L | 1.24 L |  | Gold Book |
| 38 | Nitrosodibutylamine, N | 924163 |  |  |  |  | 0.0064 A, L |  |  | Gold Book |
| 39 | Nitrosodiethylamine, N | 55185 |  |  |  |  | 0.0008 A, L |  |  | Gold Book |
| 40 | Nitrosopyrrolidine, N | 930552 |  |  |  |  | 0.016 L |  |  | Gold Book |
| 41 | Oil and Grease | -- | NARRATIVE STATEMENT - SEE TEXT |  |  |  |  |  |  | Gold Book |
| 42 | Oxamyl | 23135220 |  |  |  |  |  |  | 200 L | SDWA |
| 43 | Oxygen, Dissolved | 7782447 | WARMWATER, COLDWATER, AND EXCEPTIONS FOR NATURAL CONDITIONS - SEE TEXT K |  |  |  |  |  |  | Gold Book State Standard |
| 44 | Parathion | 56382 | 0.065 H, L | 0.013 H, L |  |  |  |  |  | Gold Book |
| 45 | Pentachlorobenzene | 608935 |  |  |  |  | 3.5 E | 4.1 E |  | IRIS 03/01/88 |
| 46 | pH | -- | SEE TEXT I |  |  |  |  |  |  | Gold Book State Standard |
| 47 | Phosphorus, Total |  | NARRATIVE STATEMENT AND NUMERIC CRITERIA - SEE TEXT |  |  |  |  |  |  | State Standard |
| 48 | Picloram | 1918021 |  |  |  |  |  |  | 500 L | SDWA |
| 49 | Salinity | -- | NARRATIVE STATEMENT - SEE TEXT |  |  |  |  |  |  | Gold Book |
| 50 | Simazine | 122349 |  |  |  |  |  |  | 4 L | SDWA |
| 51 | Solids Suspended and Turbidity | -- | NARRATIVE STATEMENT AND NUMERIC CRITERIA - SEE TEXT |  |  |  |  |  |  | Gold Book State Standard |
| 52 | Styrene | 100425 |  |  |  |  |  |  | 100 L | SDWA |
| 53 | Sulfide-Hydrogen Sulfide | 7783064 |  | 2.0 E, G, L |  | 2.0 E, G, L |  |  |  | Gold Book |
| 54 | Tainting Substances | -- | NARRATIVE STATEMENT - SEE TEXT |  |  |  |  |  |  | Gold Book |
| 55 | Temperature | -- | SPECIES DEPENDENT CRITERIA - SEE TEXT J |  |  |  |  |  |  | Red Book |
| 56 | 1, 2, 4, 5-Tetrachlorobenzene | 95943 |  |  |  |  | 2.3 E | 2.9 E |  | IRIS03/01/91 |
| 57 | Tributyltin (TBT) | 688733 | 0.46 L | 0.063 L | 0.38 L | 0.001 L |  |  |  | 62FR42554 |
| 58 | 1, 2, 4-Trichlorobenzene | 120821 |  |  |  |  |  |  | 70 L | SDWA |
| 59 | 2, 4, 5-Trichlorophenol | 95954 |  |  |  |  | 2, 600 B, D |  |  | IRIS 03/01/88 |
| 60 | Xylenes, Total | -- |  |  |  |  |  |  | 10, 000 L | SDWA |
| 61 | Beta particles and photon emitters | -- |  |  |  |  |  |  | 4 Millirems/yr | SDWA |
| 62 | Gross alpha particle activity | -- |  |  |  |  |  |  | 15 picocuries per liter (pCi/l) | SDWA |
| 63 | Radium 226 and Radium 228 (combined) | -- |  |  |  |  |  |  | 5 pCi/l | SDWA |

Footnotes:

A	This human health criterion is the same as originally published in the Red Book which predates the 1980 methodology and did not utilize the fish ingestion BCF approach.   This same criterion value is now published in the Gold Book.

B 	The organoleptic effect criterion is more stringent than the value presented in the non priority pollutants table.

C	According to the procedures described in the Guidelines for Deriving Numerical National Water Quality Criteria for the Protection of Aquatic Organisms and Their Uses, except possibly where a very sensitive species is important at a site, freshwater aquatic life should be protected if both conditions specified in Attachment 3 - Calculation of Freshwater Ammonia Criterion are satisfied.

D	This criterion has been revised to reflect The Environmental Protection Agencys q1* or RfD, as contained in the Integrated Risk Information System (IRIS) as of April 8, 1998.  The fish tissue bioconcentration factor (BCF) used to derive the original criterion was retained in each case.

E	The derivation of this value is presented in the Red Book (EPA 440/9-76-023, July, 1976).

F	This value is based on a 304(a) aquatic life criterion that was derived using the 1985 Guidelines (Guidelines for Deriving Numerical National Water Quality Criteria for the Protection of Aquatic Organisms and Their Uses, PB85-227049, January 1985) and was issued in the following criteria document: Chloropyrifos (EPA 440/5-86-005).

G	This CCC is based on the Final Residue Value procedure in the 1985 Guidelines.  Since the publication of the Great Lakes Aquatic Life Criteria Guidelines in 1995 (60 FR 15393-15399, March 23, 1995), the EPA no longer uses the Final Residue Value procedure for deriving CCCs for new or revised 304(a) aquatic life criteria.

H	This value is based on a 304(a) aquatic life criterion that was issued in the 1995 Updates: Water Quality Criteria Documents for the Protection of Aquatic Life in  Ambient Water (EPA-820-B-96-001).  This value was derived using the GLI Guidelines (60FR15393-15399, March 23, 1995; 40CFR132 Appendix A); the differences between the 1985 Guidelines and the GLI Guidelines are explained on page iv of the 1995 Updates.  No decision concerning this criterion was affected by any considerations that are specific to the Great Lakes.

I	South Carolina has established some site-specific standards for pH.  These site-specific standards are listed in S.C. Regulation 61-69, Classified Waters.

J	U.S.  EPA,  1976,  Quality Criteria for Water 1976.

K	South Carolina has established numeric criteria in Section G for waters of the State based on the protection of warmwater and coldwater species.  For the exception to be used for waters of the State that do not meet the numeric criteria established for the waterbody due to natural conditions, South Carolina has specified the allowable deficit in Section D.4. and used the following document as a source.  U.S. EPA,  1986,  Ambient Water Quality Criteria for Dissolved Oxygen,  EPA 440/5-86-003,  National Technical Information Service, Springfield, VA.  South Carolina has established some site-specific standards for DO.  These site-specific standards are listed in S.C. Regulation 61-69, Classified Waters.

L	This criterion is a noncarcinogen.  As prescribed in Section E of this regulation, application of this criterion for determining permit effluent limitations requires the use of 7Q10 or 30Q5 (if provided by the applicant), or comparable tidal condition as determined by the Department

M	This criterion is based on an added carcinogenicity risk.     As prescribed in Section E of this regulation, application of this criterion for permit effluent limitations requires the use annual average flow or comparable tidal condition as determined by the Department.

ORGANOLEPTIC EFFECTS

| Pollutant |  | CAS Number | Organoleptic Effect Criteria (g/L) | FR Cite/Source |
| --- | --- | --- | --- | --- |
| 1 | Acenaphthene | 83329 | 20 | Gold Book |
| 2 | Monochlorobenzene | 108907 | 20 | Gold Book |
| 3 | 3-Chlorophenol | -- | 0.1 | Gold Book |
| 4 | 4-Chlorophenol | 106489 | 0.1 | Gold Book |
| 5 | 2, 3-Dichlorophenol | -- | 0.04 | Gold Book |
| 6 | 2, 5-Dichlorophenol | -- | 0.5 | Gold Book |
| 7 | 2, 6-Dichlorophenol | -- | 0.2 | Gold Book |
| 8 | 3, 4-Dichlorophenol | -- | 0.3 | Gold Book |
| 9 | 2, 4, 5-Trichlorophenol | 95954 | 1 | Gold Book |
| 10 | 2, 4, 6-Trichloropehnol | 88062 | 2 | Gold Book |
| 11 | 2, 3, 4, 6-Tetrachlorophenol | -- | 1 | Gold Book |
| 12 | 2-Methyl-4-Chlorophenol | -- | 1800 | Gold Book |
| 13 | 3-Methyl-4-Chlorophenol | 59507 | 3000 | Gold Book |
| 14 | 3-Methyl-6-Chlorophenol | -- | 20 | Gold Book |
| 15 | 2-Chlorophenol | 95578 | 0.1 | Gold Book |
| 16 | Copper | 7440508 | 1000 | Gold Book |
| 17 | 2, 4-Dichlorophenol | 120832 | 0.3 | Gold Book |
| 18 | 2, 4-Dimethylphenol | 105679 | 400 | Gold Book |
| 19 | Hexachlorocyclopentadiene | 77474 | 1 | Gold Book |
| 20 | Nitrobenzene | 98953 | 30 | Gold Book |
| 21 | Pentachlorophenol | 87865 | 30 | Gold Book |
| 22 | Phenol | 108952 | 300 | Gold Book |
| 23 | Zinc | 7440666 | 5000 | 45 FR79341 |

Footnote:

1.	These criteria are based on organoleptic (taste and odor) effects.  Because of variations in chemical nomenclature systems,  this listing of pollutants does not duplicate the listing in Appendix A of 40 CFR Part 423.  Also listed are the chemical Abstracts Service (CAS) registry numbers, which provide a unique identification for each chemical.

# WATER QUALITY CRITERIA ADDITIONAL NOTES

1.	Criteria Maximum Concentration and Criterion Continuous Concentration

The Criteria Maximum Concentration (CMC) is an estimate of the highest concentration of a material in surface water to which an aquatic community can be exposed briefly without resulting in an unacceptable effect.  The Criterion Continuous Concentration (CCC) is an estimate of the highest concentration of a material in surface water to which an aquatic community can be exposed indefinitely without resulting in an unacceptable effect.  The CMC and CCC are just two of the six parts of a aquatic life criterion; the other four parts are the acute averaging period, chronic averaging period, acute frequency of allowed exceedence, and chronic frequency of allowed exceedence.

2.	Criteria for Priority Pollutants, Non Priority Pollutants and Organoleptic Effects

This appendix lists all priority toxic pollutants and some non priority toxic pollutants, and both human health effect and organoleptic effect criteria issued pursuant to CWA 304(a), the SDWA, and the NPDWR.  Blank spaces indicate that EPA has no CWA 304(a) criteria recommendations.  Because of variations in chemical nomenclature systems, this listing of toxic pollutants does not duplicate the listing in Appendix A of 40 CFR Part 423.

3.	Human Health Risk

The human health criteria for the priority and non priority pollutants are based on carcinogenicity of 10-6 risk.

4.	Water Quality Criteria published pursuant to Section 304(a) or Section 303(c) of the CWA

Many of the values in the appendix were published in the California Toxics Rule.  Although such values were published pursuant to Section 303(c) of the CWA, they represent the EPAs most recent calculation of water quality criteria.

5.	Calculation of Dissolved Metals Criteria

The 304(a) criteria for metals are shown as total recoverable metals.  As allowed in Section E of this regulation, these criteria may be expressed as dissolved metals.  Dissolved metals criteria may be calculated in one of two ways (please refer to Attachments).  For freshwater metals criteria that are hardness-dependent, the dissolved metal criteria may be calculated using a hardness of 25 mg/l as expressedas CaCO3.  Saltwater and freshwater metals criteria that are not hardness-dependent are calculated by multiplying the total recoverable criteria before rounding by the appropriate conversion factors.  The final  metals criteria in the table are rounded to two significant figures.  Information regarding the calculation of hardness dependent conversion factors are included in the footnotes.

6.	Chemical Abstract Services Number

The Chemical Abstract Services number (CAS) for each pollutant is provided (where available).

7.	Gold Book Reference

The Gold Book reference listed in the appendix refers to the May 1, 1986 EPA publication EPA 440/5-86-001.

8.	Federal Register Reference

The FR listed in the appendix refers to the appropriate Federal Register listing. and source refers to the origin of the value.  Many of the numeric values contained in this appendix  have been modified, revised, or altered and therefore, the source as listed may not be the same as it appears in this table.   Also, South Carolina may have selected to use a different value or may have promulgated a different value in its previous iterations of this regulation, so differences from these sources should be expected.

9.	Maximum Contaminant Levels

The appendix includes Maximum Contaminant Levels (MCLs) developed under the Safe Drinking Water Act (SDWA) and the National Primary Drinking Water Regulation (NPDWR).

10.	Organoleptic Effects

The appendix contains 304(a) criteria for pollutants with toxicity-based criteria as well as non-toxicity based criteria.  The basis for the non-toxicity based criteria are organoleptic effects (e.g., taste and odor) which would make water and edible aquatic life unpalatable but not toxic to humans.  The table includes criteria for organoleptic effects for 23 pollutants.  Pollutants with organoleptic effect criteria more stringent than the criteria based on toxicity (e.g., included in both the priority and non-priority pollutant tables) are footnoted as such.

11.	Category Criteria

In the 1980 criteria documents, certain water quality criteria were published for categories of pollutants rather than for individual pollutants within that category.  Subsequently, in a series of separate actions, the EPA derived criteria for specific pollutants within a category.  Therefore, in this appendix South Carolina is replacing criteria representing categories with individual pollutant criteria (e.g., 1, 3-dichlorobenzene, 1, 4-dichlorobenzene and 1, 2-dichlorobenzene).

12.	Specific Chemical Calculations

A.	Selenium

(1) Human Health

In the 1980 Selenium document, a criterion for the protection of human health from consumption of water and organisms was calculated based on a BCF of 6.0 l/kg and a maximum water-related contribution of 35 g Se/day.  Subsequently, the EPA Office of Health and Environmental Assessment issued an errata notice (February 23, 1982), revising the BCF for selenium to 4.8 L/kg.  In 1988, EPA issued an addendum (ECAO-CIN-668) revising the human health criteria for selenium.  Later in the final National Toxic Rule (NTR, 57 FR 60848), EPA withdrew previously published selenium human health criteria, pending EPA review of new epidemiological data.

This appendix includes human health criteria for selenium, calculated using a BCF of 4.8 L/kg along with the current IRIS RfD of 0.005 mg/kg/day.  South Carolina included these water quality criteria in the appendix because the data necessary for calculating a criteria in accordance with EPAs 1980 human health methodology are available.

(2) Aquatic Life

This appendix contains aquatic life criteria for selenium that are the same as those published in the CTR, with the exception of removal of the acute criterion based on EPAs June 2, 2000 revocation of the value.

B.	1, 2, 4-Trichlorobenzene and Zinc

Human health criteria for 1, 2, 4-trichlorobenzene and zinc have not been previously published.  Sufficient information is now available for calculating water quality criteria for the protection of human health from the consumption of aquatic organisms and the consumption of aquatic organisms and water for both these compounds.

C.	Chromium (III)

The aquatic life water quality criteria for chromium (III) included in the appendix are based on the values presented in the document titled: 1995 Updates: Water Quality Criteria Documents for the Protection of Aquatic Life in Ambient Water.

D.	Ether, Bis (Chloromethyl), Pentachlorobenzene, Tetrachlorobenzene 1, 2, 4, 5-,  Trichlorphenol

Human health criteria for these pollutants were last published in EPAs Quality Criteria for Water 1986 or Gold Book.   Some of these criteria were calculated using Acceptable Daily Intake (ADIs) rather than RfDs.  Updated q1*s and RfDs are now available in IRIS for ether, bis (chloromethyl), pentachlorobenzene, tetrachlorobenzene 1, 2, 4, 5-, and trichlorophenol, and were used to revise the water quality criteria for these compounds.  The water quality criteria for ether, bis (chloromethyl) were revised using an updated q1*, while criteria for pentachlorobenzene, and tetrachlorobenzene 1, 2, 4, 5-, and trichlorophenol were derived using an updated RfD value.

E.	PCBs

In this appendix, South Carolina is publishing aquatic life and human health criteria based on total PCBs rather than individual arochlors.  These criteria replace the previous criteria for the seven individual arochlors.

Attachment 1 - Conversion Factors for Dissolved Metals

| Metal | Conversion Factor freshwater CMC | Conversion Factor freshwater CCC | Conversion Factor saltwater CMC | Conversion Factor saltwater CCC |
| --- | --- | --- | --- | --- |
| Arsenic | 1.000 | 1.000 | 1.000 | 1.000 |
| Cadmium | 1.136672-[(ln hardness)(0.041838)] | 1.101672-[(ln hardness)(0.041838)] | 0.994 | 0.994 |
| Chromium III | 0.316 | 0.860 | -- | -- |
| Chromium VI | 0.982 | 0.962 | 0.993 | 0.993 |
| Copper | 0.960 | 0.960 | 0.83 | 0.83 |
| Lead | 1.46203-[(ln hardness)(0.145712)] | 1.46203-[(ln hardness)(0.145712)] | 0.951 | 0.951 |
| Mercury | 0.85 | 0.85 | 0.85 | 0.85 |
| Nickel | 0.998 | 0.997 | 0.990 | 0.990 |
| Selenium | -- | -- | 0.998 | 0.998 |
| Silver | 0.85 | -- | 0.85 | -- |
| Zinc | 0.978 | 0.986 | 0.946 | 0.946 |

Attachment 2 - Parameters for Calculating Freshwater Dissolved Metals Criteria That Are Hardness-Dependent

| Chemical | mA | bA | mC | bC | Freshwater Conversion Factors (CF) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Acute | Chronic |
| Cadmium | 1.128 | -3.6867 | 0.7852 | -2.715 | 1.136672-[ln (hardness)(0.041838)] | 1.101672-[ln (hardness)(0.041838)] |
| Chromium III | 0.8190 | 3.7256 | 0.8190 | 0.6848 | 0.316 | 0.860 |
| Copper | 0.9422 | -1.700 | 0.8545 | -1.702 | 0.960 | 0.960 |
| Lead | 1.273 | -1.460 | 1.273 | -4.705 | 1.46203-[ln (hardness)(0.145712)] | 1.46203-[ln (hardness)(0.145712)] |
| Nickel | 0.8460 | 2.255 | 0.8460 | 0.0584 | 0.998 | 0.997 |
| Silver | 1.72 | -6.52 | -- | -- | 0.85 | -- |
| Zinc | 0.8473 | 0.884 | 0.8473 | 0.884 | 0.978 | 0.986 |

# Attachment 3 - Calculation of Freshwater Ammonia Criterion

1.  The one-hour average concentration of total ammonia nitrogen (in mg N/L) does not exceed, more than once every three years on the average, the CMC calculated using the following equation:

0.275                             39.0

CMC = -------------------- +  ------------------------

1 + 107.204 - pH              1 + 10 pH  - 7.204

In situations where salmonids are absent, the CMC may be calculated using the following equation:

0.411                             58.4

CMC = -------------------- +  ------------------------

1 + 107.204 - pH              1 + 10 pH  - 7.204

2.   The thirty-day average concentration of total ammonia nitrogen (in mg N/L) does not exceed, more than once every three years on the average, the CCC calculated using the following equations:

When fish early life stages (ELS) are present:

0.0577	                  2.487

CCC =    --------------------  +  ----------------------    x   min (2.85, 1.45 x 100.028 x (25 - T))

1 + 107.688 - pH           1 + 10 pH - 7.688

When fish early life stages are absent:

0.0577	                 2.487

CCC =    --------------------  +  ----------------------    x   1.45 x 100.028 x (25 - max (T, 7))

1 + 107.688 - pH            1 + 10 pH - 7.688

and the highest four-day average within the 30-day period does not exceed 2.5 times the CCC.

In the absence of information substantiating that ELS are absent, the ELS present equation will be used.

Preliminary Fiscal Impact Statement:

No costs to the State or significant cost to its political subdivisions as a whole should be incurred by these amendments.  See Statement of Need and Reasonableness.

Statement of Need and Reasonableness:

The statement of need and reasonableness was determined by staff analysis pursuant to S.C. Code Section 1-23-115(C)(1)-(3) and (9)-(11):

DESCRIPTION OF REGULATION: Amendment of Regulation 61-68, Water Classifications and Standards.

Purpose: Proposed amendment of R.61-68 will clarify, strengthen, and improve the overall quality of the existing regulation and make appropriate revisions of the state's water quality standards in accordance with Section 303(c)(2)(B) of the Federal Clean Water Act.

Legal Authority: S.C. Code Sections 48-1-40, 48-1-60, and 48-1-80, implementing the Clean Water Act.

Plan for Implementation: The proposed amendment would be incorporated within R.61-68 upon approval of the General Assembly and publication in the State Register.  The proposed amendment will be implemented in the same manner in which the present regulation is implemented.

DETERMINATION OF NEED AND REASONABLENESS OF THE PROPOSED REGULATION BASED ON ALL FACTORS HEREIN AND EXPECTED BENEFIT: This amendment is required to comply with Federal requirements of Section 303(c)(2)(B) of the Clean Water Act.

-	The adoption of federal toxics criteria to reflect the most current final published criteria according to Sections 304(a) and 307(a) of the Clean Water Act.

The proposed changes to R.61-68 relating to human health and aquatic life criteria are reasonable because the stated criteria in the amendment are based on sound scientific principles and are required in order to comply with the goals of Section 101(a)(2) and 303(c) of the Clean Water Act for protection and maintenance of the uses of the waters of the State.  These changes include using a more protective cancer risk level of one in one million (10-6) in place of the existing risk level of one in one hundred thousand (10 -5) in order to ensure protection for all populations within South Carolina.

-	The adoption of numeric criteria for phosphorus, nitrogen, and chlorophyll a.

In an effort to resolve issues with the assessment of the narrative standard for nutrients, the Department believes that numeric criteria for specific parameters associated with over-enrichment for nutrients should be adopted.  The Department used EPA’s document “Nutrient Criteria Technical Guidance Manual: Lakes and Reservoirs (EPA 822-D-99-001 April, 1999)” in conjunction with the Department’s extensive database of nutrient data to develop the proposed numeric criteria for phosphorus, nitrogen, and chlorophyll a for lakes.  The Department affirms that these proposed numeric values are representative of South Carolina waters and are appropriate and necessary.  While understanding that the Department’s intent through this regulation is the important goal of protecting and maintaining a balanced aquatic community for lakes in the State from the impacts caused by nutrient over-enrichment and since the Department has incurred significant amounts of staff time and valuable resources in defending its use of the narrative nutrient standard for monitoring and permitting through litigious suits; the Department affirms that the proposed numeric criteria are reasonable in order to resolve these issues.

-	Selection of a more appropriate hardness value.

In the mid-1980's, the EPA published numeric criteria for several metals under Section 304(a) and 307(a) of the CWA.  Published criteria documents on some of these metals indicated variability for aquatic life toxicity with an associated hardness (as CaCO4) level.  The criteria data indicated that the relationship was shown to be inversely proportional in that as the hardness levels decrease, the toxicity associated with the metal increases.  In 1992, the EPA published the National Toxics Rule (NTR) in the Federal Register in its promulgation of numeric criteria for some States and Territories.  Contained in this ruling was a review by the EPA of its numeric criteria for these metals.  Specifically noticed was the range of hardness values for data that the EPA had considered when deriving the numeric criteria.  This range was from 25 mg/l to 400 mg/l for hardness (in previous publications, the criteria hardness values began at 50 mg/l).

R.61-68 presently states that the Department will use a hardness value of 50 mg/l as its lowest value for use with the aquatic life criteria and allows for a higher value to be determined based on the characteristics of the waterbody.  Due to the fact that the majority of ambient values for hardness in waters of the State are less than 25 mg/l, the Department proposes to lower the hardness value included in the regulation to 25 mg/l and include an upper limitation of 400 mg/l for consistency with the federal aquatic life criteria range of data.  This will provide better protection for resident aquatic communities due to the ambient hardness levels found in South Carolina.

-	Adoption of a statewide turbidity standard.

The CWA’s objectives include the protection and restoration of the physical integrity of our nation’s waters.  Physical criteria is a concept that takes into account the physical attributes of the aquatic environment, such as the quality of habitat and hydrologic balance.  Impairments of the waters have been determined to be a result of turbidity, silt, and sediment deposition into waters from the eroded material from the stream bed and stream banks that are scoured out during elevated wet weather peak discharges and extended hydroperiods.  This can lead to eutrophication, increased turbidity, decreased light penetration, submerged aquatic vegetation loss, spawning bed smothering, and shellfish habitat damage. A recent EPA document noted that scientific experts agree that overall physical habitat loss is the single biggest factor in the loss of aquatic species and that physical habitat parameters are important and often overlooked parameters that influence and at some sites control whether or not an aquatic life use is or will be attained.

While the EPA clearly recognizes the importance of physical criteria, they have not recommended a specific criterion for turbidity.  Presently the water quality standards include a turbidity standard for the protection of trout waters of the State.  It was determined that habitat loss was especially crucial to these species for propagation and well-being.  While the turbidity standard included in the regulation for trout waters is appropriate, the Department was concerned that the criterion may be overly conservative for all waters of the State.  Trout waters are often clear.  Many of the waters of the State are often more turbid due to natural, not anthropogenic, effects.  Therefore a single value for turbidity seems inappropriate and hence why the Department is proposing different values for different types of waters of the State.  In order to protect our valuable water resources from habitat loss and impairment, the Department affirms that the proposed numeric values for turbidity are necessary and reasonable.

-	Inclusion of language to clarify the components of water quality standards.

To convey the concepts of the water quality standards, the Department believes it is important to state their components in straightforward language that the public will understand.  All of these components of the water quality standards are contained in the many sections of the regulation, but no where does it simply state that these items are themselves water quality standards.  This deficiency in the regulation has lead to some miscommunication between Department staff, the regulated community, and other interested parties.  It is our intent to remedy this situation by necessarily revising the water quality standards regulation to include additional language that states the various components of water quality standards and their purpose.

-	Inclusion of language to strengthen the existing narrative standards.

Narrative criteria allow the Department to control and abate water pollution for items for which no numeric criteria exist and fulfills the intent of the CWA Section 101(a)(3) prohibition of the discharge of toxic pollutants in toxic amounts.  These narrative standards encompass the thousands of pollutants for which no numeric criteria exist.  Narrative standards also provide protection from the synergistic and additive effects of these pollutants.  Biological criteria, or biocriteria, must be reflected through the narrative standards contained in R.61-68.  It is reasonable for the Department to strengthen those narrative standards of the water quality standards regulation so that they may fulfill the goals of the Act.

EPA regulations require that narrative criteria be implemented through the NPDES permit limits.  More specifically, when the permitting authority determines that a discharge causes, has the reasonable potential to cause, or contributes to an excursion above a narrative criterion, the permit must, under most circumstances, contain effluent limits for whole effluent toxicity and under some circumstances must contain effluent limits for specific pollutants that are based on an interpretation of the State’s narrative criterion.

-	Inclusion of a description for Outstanding National Resource Waters (ONRWs).

Federal antidegradation policy contained in 40 CFR 131.12 requires that States include in their water quality standards a level of protection for Outstanding National Resource Waters (ONRWs).  These waters are determined to be of such outstanding ecological or recreational value to the Nation that they require the “highest” level of protection that the water quality standards can provide.  In order to protect these special waters, the prohibition of any permanent permitted discharge of any kind is required.

South Carolina has incorporated these requirements into our water quality standards.  Section D of the regulation states, “The water quality of outstanding resource surface waters designated as Class ORW shall be maintained and protected through the application of the standards for Class ORW as described in Section G.4 and 5.  The Department may determine, through the classification process, that some Class ORW waters are nationally significant.  Upon such determination, all activities described in Section G.4 and 5 shall be prohibited.”

As described in the existing antidegradation rules, any waterbody classified as an ONRW would no longer allow certain activities.  Since the current regulation already contains the class ONRW, the Department believes including the class description in the regulation would clarify that this classification exists.  Therefore, the  Department is proposing to include Class ONRW and its specific standards in Section G of the regulation.

-	Clarification of surface water mixing zone allowances by including specific delineations and requirements.

Some concerns have been expressed by both the regulated community and environmental groups over the lack of specificity in the State’s mixing zone policy.  These groups have stated that they believe this lack of specificity may result in subjective and inconsistent implementation of water quality standards from site-to-site.  Still a certain amount of flexibility is essential when dealing with complex water quality problems on a waterbody, watershed, or basin scale.  Explicit requirements for mixing zones need to be addressed in the water quality standards regulation while still providing flexibility so that the Department may be able to use the mixing zone allowance to address all of the multiple, differing, and complex situations that arise in permit development and issuance for the waters of the State.

The present mixing zone language in R.61-68 goes a long way to providing a basis for mixing zone allowances and at the same time, establishing requirements for the protection of the waters of the State.  All of the statements expressed in the existing language are essential to this goal.  Therefore, the Department does not intend to remove any of the requirements presently stated in the regulation.  Rather, we intend to clarify the statements and to add specific requirements dealing with the dynamics of individual mixing zones.  This is not an easy task due to the fact that it would be impossible to enumerate every instance and situation where mixing occurs in the waters of the State.  Therefore, the Department proposes that it is appropriate to deal with the issue through statements about the broader categories of both water quality and pollutants and specifically noting when a prohibition or limitation occurs.  We believe that leaving the language as general statements will allow enough flexibility to include case-by-case considerations for specific size delineations and mixing zone limitations.  The Department is also proposing additional restrictions for the use of mixing zones in an effort to protect aquatic life and human health.  The Department affirms that the additional language is both necessary and reasonable in order to resolve concerns related to the allowance and application of mixing zones.

-	Clarification of language dealing with contaminated ground water.

Ground waters of the State are intended to be protected and do have water quality criteria to protect and maintain them as drinking water sources.  Due to some past situations where the responsibility of remediation of contaminated ground water has become an issue, the Department is proposing additional language that requires that responsible parties be held accountable for remediation of the contaminated ground water.

-	Inclusion of a reporting requirement for unauthorized discharges to waters of the State.

Included in the existing regulation is the requirement that all discharges be permitted by the Department.  Section 48-1-100 of the PCA establishes this authority of the Department.  Section 48-1-90 of the PCA states that it is unlawful for any person, directly or indirectly, to discharge wastes into waters of the State except as provided by a permitted discharge authorized by the Department.  It is clear from these statements that the goal of the PCA is to ensure that discharges to waters of the State be regulated.

Several of the Department’s regulations (e.g., R.61-9, Water Pollution Control Permits) contain language that requires a permittee to report noncompliance which may endanger the public health or the environment.  The Department believes it is important that not only permittees be required to report such discharges, but that responsible party(ies) report any illegal discharges to the Department.  It is only through notification and thus knowledge of possible violations to the water quality standards that the Department can ensure the protection of the public health and the welfare of the environment.

Because of this concern, the Department is proposing the inclusion of a reporting requirement in the water quality standards regulation for unauthorized discharges to waters of the State.  We believe the language proposed is consistent with the reporting requirements contained in other Departmental regulations.  Unauthorized discharges may, in fact, cause violations of established water quality standards.  Without knowledge of unauthorized pollution, the Department’s ability to provide the public with assurances that those standards are being attained and protected is impeded.

-	Clarification of the applicability of flow conditions.

To ensure the protection of the existing and classified uses of the waters of the State, it is important that the Department specify the flow conditions that will be used when applying the State’s water quality numeric criteria.  Since water quality standards maintain the waters of the State for the propagation and protection of terrestrial and aquatic fauna and flora and also protect the public health, safety and welfare, the Department specifies which flow conditions are applicable for both aquatic life and human health numeric criteria.

Water quality criteria to protect aquatic life consist of three components: magnitude, duration, and frequency.  Magnitude refers to the acceptable concentration of a pollutant.  Duration is the period of time (averaging period) over which the ambient concentration is averaged for comparison with criteria concentrations.  Frequency is how often the criteria can be exceeded to allow the aquatic community sufficient time to recover from excursions of aquatic life criteria and to thrive after recovery. The numeric aquatic life criteria are expressed as short-term (acute) and long-term (chronic) concentrations in order that the criteria more accurately reflect toxicological and practical realities.  The combination of these two criteria provides protection of aquatic life and its uses.  Recommended averaging periods are kept relatively short because excursions higher than the average can kill or cause substantial damage in short periods. The frequency limitations specify that both the acute and chronic criteria may be exceeded once in a three-year period on average.  In order to approximate each of these components of the numeric criteria, water quality standards have prescribed flow requirements that enable the Department to issue permits in such a way as to address these requirements. The recommended once in a three-year period coupled with the 4-day chronic averaging period used for the chronic criteria approximately corresponds to the historically used criterion concentrations that occurs in a once-in-a-ten year seven-day-average low flow (7Q10).

Human health water quality numeric criteria are scientifically-derived values developed to protect human health. Regulation 61-68 has contained numeric organoleptic criteria since 1991.  Organoleptic criteria are developed to ensure the aesthetic qualities of the waters of the State that are important for both public and private concerns and uses.  While the regulation specifies the flow conditions appropriate for protection of the classified and existing uses for aquatic life criteria and also for other human health criteria, organoleptic criteria were not mentioned per se.  Also, the Department evaluated other flow conditions to be used in order to protect the public health and welfare and found that some specific language for the application of the numeric criteria was needed. Therefore, the Department is proposing additional language to specify the flow conditions to be used in applying numeric criteria for the protection of human health and organoleptic effects.

Another issue addressed by these revisions is the use of Hydrograph Controlled Releases (HCRs).  HCRs mean the onsite storage or holding of treated wastewater or the use of an alternative discharge option of this regulation, during specified critical streamflow conditions and then discharging the treated wastewater to the stream when streamflow is sufficient to assimilate the wastewater.  While R.61-68 allows other flow conditions to be used by the Department in very specific situations,  it does not specifically address this type of discharge.  As the requests for these types of discharges have increased, the Department realized that specific applicable flow conditions for HCRs should be included in the standards regulation to ensure the protection of the overall health and well-being of the waterbody.  With the understanding that the flow conditions established in the regulation are included in order to ensure that the waters of the State are protected, the Department believes there are specific instances where the standards do not allow the use of HCRs.  We also believe that there are situations where an HCR may be appropriate for use in order to meet the water quality standards when no other option is available, provided that the antidegradation rules are properly reviewed and implemented.  Therefore, recognizing the increase in instances where these types of discharges have been requested and finding that the regulation does not specifically address this important issue, the Department is proposing necessary language to reflect the appropriate flow conditions to be used when applying the numeric criteria and additional language to specify the restrictions of the use of HCRs.

-	Clarification of existing language to ensure that downstream uses of waters of the State are protected.

From the earliest versions of both state and federal laws and regulations, it is clear that all uses of the waters of the State must be protected and maintained.  Consistent with this goal is the intent of water quality standards to protect and maintain the existing and classified uses of waters downstream from point and nonpoint sources of pollution.  The federal water quality standards regulation contains this important provision and 40 CFR 130.10 (b) states: “In designating uses of a water body and the appropriate criteria for those uses, the State shall take into consideration the water quality standards of downstream waters and shall ensure that its water quality standards provide for the attainment and maintenance of the water quality standards of downstream waters.”  This federal regulation clearly provides the requirement that State water quality standards must contain provisions for the protection of downstream uses.  In a previous version of the water quality standards regulation, the language stated that “water quality must be maintained at a level which will not cause a contravention of the higher standards of the downstream body.”  While in its present version, R. 61-68 does contain several narrative statements that convey this important provision of water quality standards, it lacks a specific section that clearly states this intent.  The Department affirms that including a restatement of this requirement of water quality standards will clarify the intent of the State’s water quality standards and their goal for the protection and maintenance of downstream uses.

-	Review and revision of language for antidegradation.

Present federal regulation requires that State water quality standards contain an antidegradation policy.  This policy requires the establishment of three levels of protection that provide for the maintenance of a level of water quality that will: 1) protect and support existing and classified uses, 2) where water quality is better than that necessary to support the existing and classified uses then that water quality is also maintained and protected unless, through a public process, some lowering of water quality is deemed to be necessary to allow important economic or social development to occur, and 3) identify water bodies of nationally exceptional recreational or ecological significance (Outstanding National Resource Waters or ONRWs) and maintain and protect water quality in such water bodies. The same regulation also requires that States have an implementation procedure for their antidegradation policy.

While the Department has prepared the State’s implementation procedures for its antidegradation policy as required under federal regulation 40 CFR 131.12(a) and while the implementation procedures are based on the State’s antidegradation rules in R.61-68, many of the details of implementation are not specifically contained in the water quality standards.  The Department is proposing to include specific language under the antidegradation rules that address alternatives analysis and public participation in the process of allowing a lowering of water quality based on important economic and social development.  We believe the additional language will resolve some misunderstandings regarding the existing antidegradation standards and is both necessary and reasonable.

-	Inclusion of new definitions.

R.61-68 contains definitions for terms used in the regulation that are not defined in the PCA and may not be readily available to the public or may have specific meaning within the context of the regulation.  This section is necessary and vital in that it provides clarity to the intent of the regulation.  The Department believes it is important to provide definitions for terms that may have other meanings in another context or may be termed differently when addressing the complex issues contained in the regulation.  Therefore, several new definitions have been proposed for inclusion in the revisions to the water quality standards regulation.

-	Stylistic changes which include corrections for: clarity and readability, grammar, punctuation, typography, codification, references, language style, and overall improvement of the text of the regulation.

DETERMINATION OF COSTS AND BENEFITS: Existing staff and resources will be utilized to implement this amendment to the regulation.  No additional cost will be incurred by the State if the revisions are implemented and therefore, no additional state funding is being requested.  However, due to recent litigation defending the Department’s narrative water quality standards (specifically with regard to nutrients and toxicity issues), the Department has incurred an estimated additional cost of $100,000 in staff time and resources for these purposes.  Since litigation is not only a costly, but lengthy process and even though the Department anticipates that ultimately the court decisions will support the narrative water quality standards; we believe it is necessary and beneficial that we propose amendment to the regulation to either adopt numeric criteria where applicable or to provide more specific narrative statements that will address these concerns.   It is our hope that this will alleviate potential future litigation costs by making the appropriate revisions in this public forum and through this public process.  If the amendment is not implemented, the Department does anticipate that further litigation costs will be incurred in the settlement of these same issues and therefore, additional state funding may be necessary.

In reviewing the potential for significant economic impact of the proposed amendment, the Department specifically evaluated situations in which costs would most likely be incurred by the regulated community. These estimates addressed the specific revisions by issue after determining those of greatest potential impact.  The Department found that the overall impact to the State’s political subdivisions or the regulated community as a whole was not likely to be significant in that the existing narrative standards would have incurred similar cost or the fact that the design standards required under the amendment will be substantially consistent with the current guidelines and review guidelines utilized by the Department.  Further, much of the proposed amendment, for which an estimated cost may be incurred by the regulated community at the time of permit issuance, are essential and necessary to protect and maintain the existing uses supported by the water quality standards and are therefore, beyond the scope of cost analysis in that they provide the minimum level of protection for aquatic life and human health as required by the Federal CWA.

UNCERTAINTIES OF ESTIMATES: Minimal to moderate.

EFFECT ON ENVIRONMENT AND PUBLIC HEALTH: Implementation of this amendment will not compromise the protection of the environment or the health and safety of the citizenry of the State.  The amendment will promote and protect aquatic life and human health by the regulation of pollutants into waters of the State.

DETRIMENTAL EFFECT ON THE ENVIRONMENT AND PUBLIC HEALTH IF THE REGULATION IS NOT IMPLEMENTED: Failure by the Department to incorporate appropriately protective water quality standards in the regulation that is the basis for issuance of National Pollutant Discharge Elimination System (NPDES) permits, stormwater permits, wasteload and load allocations, groundwater remediation plans, and multiple other program areas will lead to contamination of the waters of the State with detrimental effects on the health of flora and fauna found in the State as well as the citizens of South Carolina.