AGY:	Department of Health and Environmental Control

FII:	6

FIV:	28

PRD:	20030926

EFD:	20040625

EXD:	20040525

REG:	2855

PRI:	9

PRV:	27

COM:	Agriculture, Natural Resources and Environmental Affairs Committee 	20 HANR

Medical Affairs Committee 13 SMA

RES:	1243

STA:	Final

AUT:	48-1-10

SUB:	Water Classifications and Standards

HST:	2855

BY	DATE	ACTION DESCRIPTION	COM	VOL/ISSUE	EXP DATE	R. NUM

___	________	__________________________	_______	___________	__________	_______

-	20030926	Proposed Reg Published in SR		27/9

-	20040113	Received by Lt. Gov & Speaker			20040511

S	20040113	Referred to Committee 	SMA 13

H	20040113	Referred to Committee 	HANR 20

H	20040505	Committee Request Withdrawal	HANR20

120 Day Period Tolled

-	20040505	Withdrawn and Resubmitted			20040525

S	20040513	Resolution Intro to Approv	SMA 13			S1243

-	20040525	Approved by:  Expiration Date		28/6

TXT:

Resubmitted May 5, 2004

Document No. 2855

DEPARTMENT OF HEALTH AND ENVIRONMENTAL CONTROL

CHAPTER 61

Statutory Authority: 1976 Code Section 48-1-10 et seq.

R.61-68.   Water Classifications and Standards

Synopsis:

This amendment of R.61-68 of R.61-68 will strengthen and improve the existing regulation and make appropriate revisions of the State's water quality standards in accordance with Section 303(c)(2)(B) of the Federal Clean Water Act (CWA).  Section 303(c)(2)(B) requires that South Carolina=s water quality standards be reviewed and revised, where necessary, at least every three years for the purposes of considering the Environmental Protection Agency=s (EPA) most recently published numeric and narrative criteria and to comply with recent Federal regulatory revisions and recommendations.  The Department has also included two revisions that will improve the regulation.  R.61-68 also includes revisions due to recodification of additional language from the proposed text changes so that every section, subsection, item, and subitem could be cited correctly.  In response to comments submitted , the DHEC Board has also added language in Section E. that provides for an alternative species (Daphnia ambigua) for toxicity testing.  See also the Statement of Need and Reasonableness herein.

Changes made pursuant to Legislative Review requested by House Agricultural,

Natural Resources, and Environmental Affairs Committee

SECTION					REVISION

R.61-68.E.14.c.8.	 		Retained the word “calculating” in two places.

(existing code section

R.61-68.E.12.c.8.)

R.61-68.E.14.c.9.	 		Retained the word “calculating” in two places.

(existing code section

R.61-68.E.12.c.9.)

Discussion of Revisions as Proposed by the Department:

(1):	Adoption of federal toxics criteria to reflect the most current final published criteria according to Sections 304(a) and 307(a) of the Clean Water Act.

SECTION					REVISION

Appendix					These revisions reflect EPA’s most recently published criteria.  This included several human health values being revised to adjust for a higher fish tissue consumption rate and several pollutant values that no longer use a bioconcentration factor, but now use a bioaccumulation factor.   EPA’s revised criteria also included the footnotes as well as several of the Federal Register citations.  The Department has removed the proposed methylmercury criterion for human health and replaced it with the current state water quality criterion and amended the footnotes to reflect the change.  The Department also corrected an error in the table based on commenter’s note.

(2):	Review and revision of the bacterial indicator for protection of recreational uses.

SECTION					REVISION

R.61-68.E.14.c.9.			Added language to reflect the addition of enterococci as a bacterial indicator for saltwaters of the State.  The Department has also added language that clarifies that the enterococci indicator will only be used for compliance with NPDES permit limits when an effluent analysis is conducted using an EPA-approved methodology that has been finalized through 40 CFR 136.

R.61-68.E.14.d.5.			Added language to reflect the addition of enterococci as a bacterial indicator for saltwaters of the State.

R.61-68.G.4.a.				Added language to reflect the addition of enterococci as a bacterial indicator for saltwaters of the State.

R.61-68.G.6.a.				Added language to reflect the addition of enterococci as a bacterial indicator for saltwaters of the State.

R.61-68.G.11.e.				Added language to reflect the addition of enterococci as a bacterial indicator for saltwaters of the State.

R.61-68.G.12.e.				Added language to reflect the addition of enterococci as a bacterial indicator for saltwaters of the State.

R.61-68.G.13.e.				Added language to reflect the addition of enterococci as a bacterial indicator for saltwaters of the State.

(3):	Inclusion of an allowance for a variance from water quality standards.

SECTION					REVISION

R.61-68.B.59.				Added a definition for variance.

R.61-68.E.7.a.-f.			Added language restating the factors that must be demonstrated prior to the granting of a variance.

R.61-68.E.8.a.-f.			Added a variance provision to the water quality standards setting forth the conditions and circumstances under which a variance may be granted by the Department.

R.61-68.E.9.					Added language to include the variance provision.

(4):	Stylistic changes which may include corrections for: readability, grammar, punctuation, typography, codification, references, and language style.

The regulation also includes revisions due to recodification of additional language from the proposed text changes so that every section, subsection, item, and subitem could be cited correctly.

SECTION					REVISION

R.61-68.E.12.c.				Typographical correction.

R.61-68.E.14.c.8.	 		Removed language for clarity.

(existing code section

R.61-68.E.12.c.8.)

R.61-68.E.14.c.9.	 		Removed language for clarity.

(existing code section

R.61-68.E.12.c.9.)

(5):	Language added by the DHEC Board in response to comments received.

SECTION					REVISION

R.61-68.E.14.c.10.			Added language to provide D. ambigua as an alternative species for toxicity testing.

R.61-68.E.17.d.	 			Added language to provide D. ambigua as an alternative species for toxicity testing.

Instructions:  Amend R.61-68 pursuant to each individual instruction below:

Text of Proposed Amendment:

At R.61-68.B., add in alpha-numeric order, new definition for Variance to read; renumber remaining sections.

Variance means a short-term exemption from meeting certain otherwise applicable water quality standards.

At R.61-68.E.2., revise to read.

The classes and standards described in Section G and H of this regulation implement the above State policy by protecting the waters of South Carolina.  Consistent with the above policy, the Department adopts the following general standards in items 3-17 for all waters of South Carolina.

At R.61-68.E.7. and 8., add new subsections to read.

7.	Before the Department may grant a variance for any water of the State, there must be a demonstration that one of the following factors for reclassifying uses has been satisfied:

a.	Natural conditions prevent the attainment of the use; or

b.	Natural, ephemeral, intermittent, low flow conditions, or water levels prevent the attainment of the use; or

c.	Human caused conditions or sources prevent the attainment of the use and cannot be remedied or would cause more environmental damage to correct than to leave in place; or

d.	Dams, diversions, or other types of hydrologic modifications preclude the attainment of the use, and it is not feasible to restore the waterbody to its original condition or to operate such modification in a way that would result in the attainment of the use; or

e.	Physical conditions related to the natural features of the water body, such as the lack of a proper substrate, cover, flow, depth, pools, riffles, and the like, preclude attainment of aquatic life protection uses; or

f.	Controls more stringent than those required by Sections 301(b) and 306 of the Clean Water Act would result in adverse social and economic impact, disproportionate to the benefits to the public health, safety or welfare as a result of maintaining the standard.

8.	If the demonstration necessary under Section E.7 above has been satisfied, the Department may then grant a variance provided the following apply:

a.	The variance is granted to an individual discharger for a specific pollutant(s) or parameter(s) and does not otherwise modify water quality standards; and

b.	The variance identifies and justifies the criterion that shall apply during the existence of the variance; and

c.	The variance is established as close to the underlying criterion as is possible and upon expiration of the variance, the underlying criterion shall become the effective water quality standard for the waterbody; and

d.	The variance is reviewed every three years, at a minimum, and extended only where the conditions for granting the variance still apply; and

e.	The variance does not exempt the discharger from compliance with any applicable technology or other water quality-based permit effluent limitations; and

f.	The variance does not affect permit effluent limitations for other dischargers.

At R.61-68.E.9., revise to read; renumber remaining subsections.

Prior to removing any uses or granting a variance, notice and an opportunity for a public hearing shall be provided.

At renumbered R.61-68.E.12.c. (existing code section R.61-68.E.10.c.), revise to read.

The weekly average water temperature of all Freshwaters which are lakes shall not be increased more than 5oF (2.8oC) above natural conditions and shall not exceed 90oF (32.2oC) as a result of the discharge of heated liquids unless a different site-specific temperature standard as provided for in C.12. has been established, a mixing zone as provided in C.10. has been established, or a Section 316(a) determination under the Federal Clean Water Act has been completed.

At renumbered R.61-68.E.14.c.8. (existing code section R.61-68.E.12.c.8), revise to read.

In order to protect for the consumption use of shellfish, for SFH waters and other waters with approved shellfish harvesting uses, the stated value of 14/100 ml for fecal coliform shall be used as a monthly average number for calculating permit effluent limitations and the stated value of 43/100ml for fecal coliform shall be used as a daily maximum number for calculating permit effluent limitations.

At renumbered R.61-68.E.14.c.9. (existing code section R.61-68.E.12.c.9.), revise to read.

In order to protect recreational uses for all waters of the State, the stated value of 200/100 ml for fecal coliform shall be used as a monthly average number for calculating permit effluent limitations and the stated value of 400/100ml for fecal coliform shall be used as daily maximum number for calculating permit effluent limitations.  In order to protect recreational uses in Class SB saltwaters of the State, the stated value of 35/100 ml for enterococci shall be used as a monthly average number for permit effluent limitations and the stated value of 501/100 ml for enterococci shall be used as a daily maximum number for permit effluent limitations.  In order to protect recreational uses in all other saltwaters of the State, the stated value of 35/100 ml for enterococci shall be used as a monthly average number for permit effluent limitations and the stated value of 104/100 ml for enterococci shall be used as a daily maximum number for permit effluent limitations.  Implementation of the enterococci standards in NPDES permit effluent limitations shall be subsequent to EPA publishing the applicable test methods in 40 CFR 136.

At renumbered R.61-68.E.14.c.10. (existing code section R.61-68.E.12.c.10), revise to read.

All effluent permit limitations which include WET will require that the WET tests be conducted using Ceriodaphnia dubia (C. dubia), except as stated.  If the salinity of a discharge to a saline waterbody is high enough to be toxic to C. dubia, Mysidopsis bahia (M. bahia) will be used.  Low salinity discharges to saltwater may be tested using either C. dubia or M. bahia with salinity adjustment, as determined by the Department.  If the hardness of a waterbody is low enough to be toxic to C. dubia, then Daphnia ambigua (D. ambigua) may be used.  The Department may consider an alternative species if it can be demonstrated that the proposed species meets the requirements of 40 CFR.136.4 and 5.  EPA test methods (40 CFR Part 136) for acute and chronic toxicity testing with freshwater organisms or marine and estuarine organisms must be followed.  Any modifications to species selection or the methodology used shall be approved by the EPA.

At renumbered R.61-68.E.14.d.5. (new section added after existing code section R.61-68.E.12.d.4.), add new subsection to read.

The assessment of enterococci for purposes of issuing swimming advisories for ocean beaches for recreational use will be based on the single sample maximum of 104/100 ml.

At renumbered R.61-68.E.17.d. (existing code section R.61-68.E.15.d.), revise to read.

Ambient toxicity tests used for screening purposes shall be conducted using Ceriodaphnia dubia (C. dubia), except as stated.  If salinity of a waterbody is high enough to be toxic to C. dubia, Mysidopsis bahia (M. bahia) will be used.  If the hardness of a waterbody is low enough to be toxic to C. dubia, then Daphnia ambigua (D. ambigua) may be used.  The Department may consider an alternative species if it can be demonstrated that the proposed species meets the requirements of 40 CFR.136.4 and 5.  EPA test methods (40 CFR Part 136) for acute and chronic toxicity testing with freshwater organisms or marine and estuarine organisms must be followed.  Any modifications to species selection or the methodology used shall be approved by the EPA.

At R.61-68.G.3., revise to read.

For items not listed in each class, criteria published pursuant to Sections 304(a) and 307(a) of the Federal Clean Water Act or other documents shall be used as guides to determine conditions which protect water uses.  Many of these criteria are listed in the appendix to this regulation.  For consideration of natural conditions, refer to Sections: C.9., D.4., E.12., E.14.c.(2), E.14.c.(3), F.4.d., G.4., G.6., and G.9.  For the following numeric criteria for turbidity (with the exception of Outstanding National Resource Waters, Outstanding Resource Waters, Trout waters, and Shellfish Harvesting Waters), compliance with these turbidity criteria may be considered to be met as long as the waterbody supports a balanced indigenous aquatic community when land management activities employ Best Management Practices (BMPs).  For consideration, BMPs must be in full compliance with all specifications governing the proper design, installation, operation and maintenance of such BMPs and all applicable permit conditions and requirements must be met.

At R.61-68.G.4.a., revise to read.

Quality Standards for

Outstanding National Resource Waters

ITEMS	STANDARDS

a. Color, dissolved oxygen, fecal	Water quality conditions shall be

coliform, enterococci, pH, temperature, 	maintained and protected to the

turbidity, or other parameters.	extent of the Department’s statutory

authority.  Numeric and narrative 		criteria for Class ONRW shall be those

applicable to the classification

of the waterbody immediately

prior to reclassification to

Class ONRW, including consideration

of natural conditions.

At R.61-68.G.6.a., revise to read.

Quality Standards for

Outstanding Resource Waters

ITEMS	STANDARDS

a. Color, dissolved oxygen, fecal	Water quality conditions shall be

coliform, enterococci, pH, temperature, 	maintained and protected to the

turbidity, or other parameters.	extent of the Department’s statutory

authority.  Numeric and narrative 		criteria for Class ORW shall be those

applicable to the classification

of the waterbody immediately

prior to reclassification to

Class ORW, including consideration

of natural conditions.

At R.61-68.G.10.g., revise to read.

Quality Standards for

Freshwaters

ITEMS	STANDARDS

g. Temperature.	As prescribed in E.12.of this

regulation.

At R.61-68.G.11.f., add subsection to read; renumber remaining subsections.

Quality Standards for

Shellfish Harvesting Waters

ITEMS	STANDARDS

f.  Enterococci.	Not to exceed a geometric mean of 35/100 ml based on at least four samples collected from a given sampling site over a 30 day period; nor shall samples exceed a single sample maximum of 104/100 ml.

At R.61-68.G.11.h., revise to read.

h. Temperature.	As prescribed in E.12.of this 		regulation.

At R.61-68.G.12.f., add subsection to read; renumber remaining subsections.

Quality Standards for

Class SA Waters

ITEMS	STANDARDS

f.  Enterococci.	Not to exceed a geometric mean of 35/100 ml based on at least four samples collected from a given sampling site over a 30 day period; nor shall samples exceed a single sample maximum of 104/100 ml.

At R.61-68.G.12.h., revise to read.

h. Temperature.	As prescribed in E.12.of this 		regulation.

At R.61-68.G.13.f., add subsection to read; renumber remaining subsections.

Quality Standards for

Class SB Waters

ITEMS	STANDARDS

f.  Enterococci.	Not to exceed a geometric mean of 35/100 ml based on at least four samples collected from a given sampling site over a 30 day period; nor shall samples exceed a single sample maximum of 501/100 ml.

At R.61-68.G.13.h., revise to read.

h. Temperature.	As prescribed in E.12.of this 		regulation.

At R.61-68.Appendix, replace in its entirety as follows to read.

# APPENDIX: WATER QUALITY NUMERIC CRITERIA FOR THE PROTECTION OF

AQUATIC LIFE AND HUMAN HEALTH

This appendix contains three charts (priority pollutants, nonpriority pollutants, and organoleptic effects) of numeric criteria for the protection of human health and aquatic life.  The appendix also contains three attachments which address hardness conversions and application of ammonia criteria.  Footnotes specific to each chart follow the chart.  General footnotes pertaining to all are at the end of the charts prior to the attachments.  Please refer to the text of the regulation for other general information and specifications in applying these numeric criteria.

PRIORITY TOXIC POLLUTANTS

| Priority Pollutant |  | CAS Number | Freshwater Aquatic Life |  | Saltwater Aquatic Life |  | Human Health |  |  | FR Cite/ Source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | CMC (g/L) | CCC (g/L) | CMC (g/L) | CCC (g/L) | For Consumption of: |  |  |  |
|  |  |  |  |  |  |  | Water & Organism (g/L) | Organism Only (g/L) | MCL (g/L) |  |
| 1 | Antimony | 7440360 |  |  |  |  | 5.6 B, ee | 640 B, ee | 6 ee | 65FR66443 SDWA |
| 2 | Arsenic | 7440382 | 340 A, D, K | 150 A, D, K | 69 A, D, Y | 36 A, D, Y | 0.018 C, R, ff | 0.14 C, R, ff | 10 C | 65FR31682 57FR60848 SDWA |
| 3 | Beryllium | 7440417 |  |  |  |  | J, ee | J, ee | 4 ee | 65FR31682 SDWA |
| 4 | Cadmium | 7440439 | 0.53 D, E, K | 0.10 D, E, K | 43 D, Y | 9.3 D, Y | J, ee | J, ee | 5 ee | 65FR31682 SDWA |
| 5a | Chromium III | 16065831 | 580 D, E, K | 28 D, E, K |  |  | J, ee | J, ee | 100 Total ee | EPA820/B-96-001 65FR31682 SDWA |
| 5b | Chromium VI | 18540299 | 16 D, K | 11 D, K | 1,100 D, Y | 50 D, Y | J, ee | J, ee | 100 Total ee | 65FR31682 SDWA |
| 6 | Copper | 7440508 | 3.8 D, E, K, Z | 2.9 D, E, K, Z | 5.8 D, Z, Y, cc | 3.7 D, Z, Y, cc | 1,300 T, ee |  |  | 65FR31682 |
| 7 | Lead | 7439921 | 14 D, E, Y | 0.54 D, E, Y | 220 D, Y | 8.5 D, Y |  |  |  | 65FR31682 |
| 8a | Mercury | 7439976 | 1.6 D, K, dd | 0.91 D, K, dd | 2.1 D, bb, dd | 1.1 D, bb, dd | 0.050 B, ee | 0.051 B, ee | 2 ee | 65FR31682 |
| 9 | Nickel | 7440020 | 150 D, E, K | 16 D, E, K | 75 D, Y | 8.3 D, Y | 610 B, ee | 4, 600 B, ee |  | 65FR31682 |
| 10 | Selenium | 7782492 | L, Q, S | 5.0 S | 290 D, aa | 71 D, aa | 170 Z, ee | 4,200 ee | 50 ee | 65FR31682 65FR66443 SDWA |
| 11 | Silver | 7440224 | 0.37 D, E, G |  | 2.3 D, G |  |  |  |  | 65FR31682 |
| 12 | Thallium | 7440280 |  |  |  |  | 1.7 B, ee | 6.3 B, ee | 2 ee | 65FR31682 SDWA |
| 13 | Zinc | 7440666 | 37 D, E, K | 37 D, E, K | 95 D, Y | 86 D, Y | 7,400 T, ee | 26,000 T, ee |  | 65FR31682 65FR66443 |
| 14 | Cyanide | 57125 | 22 K, P | 5.2 K, P | 1 P, Y | 1 P, Y | 700 B, ee | 220,000 B, H, ee | 200 ee | EPA820/B-96-001 57FR60848 SDWA |
| 15 | Asbestos | 1332214 |  |  |  |  |  |  | 7 million fibers/L I, ee | 57FR60848 |
| 16 | 2, 3, 7, 8-TCDD (Dioxin) | 1746016 |  |  |  |  |  | 0.046 ppq C, O | 30 ppq C, O | State Standard SDWA |
| 17 | Acrolein | 107028 |  |  |  |  | 190 ee | 290 ee |  | 65FR66443 |
| 18 | Acrylonitrile | 107131 |  |  |  |  | 0.051 B, C | 0.25 B, C |  | 65FR66443 |
| 19 | Benzene | 71432 |  |  |  |  | 2.2 B, C | 51 B, C | 5 C | IRIS 01/19/00 65FR66443 SDWA |
| 20 | Bromoform | 75252 |  |  |  |  | 4.3 B, C | 140 B, C | 100 Total THMs C | 65FR66443 SDWA |
| 21 | Carbon Tetrachloride | 56235 |  |  |  |  | 0.23 B, C | 1.6 B, C | 5 C | 65FR66443 SDWA |
| 22 | Chlorobenzene | 108907 |  |  |  |  | 680 B, T, ee | 21,000 B, H, T, ee | 100 T, ee | 65FR31682 SDWA |
| 23 | Chlorodibromomethane | 124481 |  |  |  |  | 0.40 B, C | 13 B, C | 100 Total THMs C | 65FR66443 SDWA |
| 24 | Chloroform | 67663 |  |  |  |  | 5.7 B, C, hh | 470 B, C, hh | 100 Total THMs C | 62FR42160 SDWA |
| 25 | Dichlorobromomethane | 75274 |  |  |  |  | 0.55 B, C | 17 B, C | 100 Total THMs C | 65FR66443 SDWA |
| 26 | 1, 2-Dichloroethane | 107062 |  |  |  |  | 0.38 B, C | 37 B, C | 5 C | 65FR66443 SDWA |
| 27 | 1, 1-Dichloroethylene | 75354 |  |  |  |  | 0.057 B, C | 3.2 B, C | 7 C | 65FR66443 SDWA |
| 28 | 1, 2-Dichloropropane | 78875 |  |  |  |  | 0.50 B, C | 15 B, C | 5 C | 65FR66443 SDWA |
| 29 | 1, 3-Dichloropropene | 542756 |  |  |  |  | 10 B, ee | 1,700 B, ee |  | 57FR60848 |
| 30 | Ethylbenzene | 100414 |  |  |  |  | 3,100 B, ee | 29,000 B, ee | 700 ee | 65FR31682 SDWA |
| 31 | Methyl Bromide | 74839 |  |  |  |  | 47 B, ee | 1,500 B, ee |  | 65FR66443 |
| 32 | Methylene Chloride | 75092 |  |  |  |  | 4.6 B, C | 590 B, C | 5 C | 65FR66443 SDWA |
| 33 | 1, 1, 2, 2-Tetrachloroethane | 79345 |  |  |  |  | 0.17 B, C | 4.0 B, C |  | 65FR66443 |
| 34 | Tetrachloroethylene | 127184 |  |  |  |  | 0.69 C | 3.3 C | 5 C | 65FR66443 SDWA |
| 35 | Toluene | 108883 |  |  |  |  | 6,800 B, ee | 200,000 B, ee | 1000 ee | 65FR31682 SDWA |
| 36 | 1, 2-Trans-Dichloroethylene | 156605 |  |  |  |  | 700 B, ee | 140,000 B, ee | 100 ee | 65FR31682 SDWA |
| 37 | 1, 1, 1-Trichloroethane | 71556 |  |  |  |  | J, ee | J, ee | 200 ee | 65FR31682 SDWA |
| 38 | 1, 1, 2-Trichloroethane | 79005 |  |  |  |  | 0.59 B, C | 16 B, C | 5 C | 65FR66443 SDWA |
| 39 | Trichloroethylene | 79016 |  |  |  |  | 2.5 C | 30 C | 5 C | 65FR66443 SDWA |
| 40 | Vinyl Chloride | 75014 |  |  |  |  | 2.0 C | 530 C | 2 C | 65FR66443 SDWA |
| 41 | 2-Chlorophenol | 95578 |  |  |  |  | 81 B, T, ee | 150 B, T, ee |  | 65FR66443 |
| 42 | 2, 4-Dichlorophenol | 120832 |  |  |  |  | 77 B, T, ee | 290 B, T, ee |  | 65FR66443 |
| 43 | 2, 4-Dimethylphenol | 105679 |  |  |  |  | 380 B, T, ee | 850 B, T, ee |  | 65FR66443 |
| 44 | 2-Methyl- 4, 6-Dinitrophenol | 534521 |  |  |  |  | 13 ee | 280 ee |  | 65FR66443 |
| 45 | 2, 4-Dinitrophenol | 51285 |  |  |  |  | 69 B, ee | 5,300 B, ee |  | 65FR66443 |
| 46 | Pentachlorophenol | 87865 | 19 F, K | 15 F, K | 13 Y | 7.9 Y | 0.27 B, C | 3.0 B, C, H | 1 C | 65FR31682 65FR66443 SDWA |
| 47 | Phenol | 108952 |  |  |  |  | 21,000 B, T, ee | 1,700,000 B, H, T, ee |  | 65FR66443 |
| 48 | 2, 4, 6-Trichlorophenol | 88062 |  |  |  |  | 1.4 B, C, T | 2.4 B, C |  | 65FR66443 |
| 49 | Acenaphthene | 83329 |  |  |  |  | 670 B, T, ee | 990 B, T, ee |  | 65FR66443 |
| 50 | Anthracene | 120127 |  |  |  |  | 8,300 B, ee | 40,000 B, ee |  | 65FR66443 |
| 51 | Benzidine | 92875 |  |  |  |  | 0.000086 B, C | 0.00020 B, C |  | 65FR66443 |
| 52 | Benzo (a) Anthracene | 56553 |  |  |  |  | 0.0038 B, C | 0.018 B, C |  | 65FR66443 |
| 53 | Benzo (a) Pyrene | 50328 |  |  |  |  | 0.0038 B, C | 0.018 B, C | 0.2 C | 65FR66443 SDWA |
| 54 | Benzo (b) Fluoranthene | 205992 |  |  |  |  | 0.0038 B, C | 0.018 B, C |  | 65FR66443 |
| 55 | Benzo (k) Fluoranthene | 207089 |  |  |  |  | 0.0038 B, C | 0.018 B, C |  | 65FR66443 |
| 56 | Bis 2-Chloroethyl Ether | 111444 |  |  |  |  | 0.030 B, C | 0.53 B, C |  | 65FR66443 |
| 57 | Bis 2-Chloroisopropyl Ether | 108601 |  |  |  |  | 1,400 B, ee | 65,000 B, ee |  | 65FR66443 |
| 58 | Bis 2-Ethylhexyl Phthalate (DEHP) | 117817 | V | V | V | V | 1.2 B, C | 2.2 B, C | 6 C | 65FR66443 SDWA |
| 59 | Butylbenzyl Phthalate | 85687 | ii | ii | ii | ii | 1,500 B, ee | 1,900 B, ee |  | 65FR66443 |
| 60 | 2-Chloronaphthalene | 91587 |  |  |  |  | 1,000 B, ee | 1,600 B, ee |  | 65FR66443 |
| 61 | Chrysene | 218019 |  |  |  |  | 0.0038 B, C | 0.018 B, C |  | 65FR66443 |
| 62 | Dibenzo (a, h) Anthracene | 53703 |  |  |  |  | 0.0038 B, C | 0.018 B, C |  | 65FR66443 |
| 63 | 1, 2-Dichlorobenzene | 95501 |  |  |  |  | 2,700 B, ee | 17,000 B, ee | 600 ee | 65FR31682 SDWA |
| 64 | 1, 3-Dichlorobenzene | 541731 |  |  |  |  | 320 ee | 960 ee |  | 65FR66443 |
| 65 | 1, 4-Dichlorobenzene | 106467 |  |  |  |  | 400 ee | 2,600 ee | 75 ee | 65FR31682 SDWA |
| 66 | 3, 3’-Dichlorobenzidine | 91941 |  |  |  |  | 0.021 B, C | 0.028 B, C |  | 65FR66443 |
| 67 | Diethyl Phthalate | 84662 | ii | ii | ii | ii | 17,000 B, ee | 44,000 B, ee |  | 65FR66443 |
| 68 | Dimethyl Phthalate | 131113 | ii | ii | ii | ii | 270,000 B, ee | 1,100,000 B, ee |  | 65FR66443 |
| 69 | Di-n-butyl Phthalate | 84742 | ii | ii | ii | ii | 2,000 B, ee | 4,500 B, ee |  | 65FR66443 |
| 70 | 2, 4-Dinitrotoluene | 121142 |  |  |  |  | 0.11 C | 3.4 C |  | 65FR66443 |
| 71 | 1, 2-Diphenylhydrazine | 122667 |  |  |  |  | 0.036 B, C | 0.20 B, C |  | 65FR66443 |
| 72 | Fluoranthene | 206440 |  |  |  |  | 130 B, ee | 140 B, ee |  | 65FR66443 |
| 73 | Fluorene | 86737 |  |  |  |  | 1,100 B, ee | 5,300 B, ee |  | 65FR66443 |
| 74 | Hexachlorobenzene | 118741 |  |  |  |  | 0.00028 B, C | 0.00029 B, C | 1 C | 65FR66443 SDWA |
| 75 | Hexachlorobutadiene | 87683 |  |  |  |  | 0.44 B, C | 18 B, C |  | 65FR66443 |
| 76 | Hexachlorocyclo-pentadiene | 77474 |  |  |  |  | 240 B, T, ee | 17,000 B, H, T, ee | 50 ee | 57FR60848 SDWA |
| 77 | Hexachloroethane | 67721 |  |  |  |  | 1.4 B, C | 3.3 B, C |  | 65FR66443 |
| 78 | Indeno 1, 2, 3 – (cd) Pyrene | 193395 |  |  |  |  | 0.0038 B, C | 0.018 B, C |  | 65FR66443 |
| 79 | Isophorone | 78591 |  |  |  |  | 35 B, C | 960 B, C |  | 65FR66443 |
| 80 | Nitrobenzene | 98953 |  |  |  |  | 17 B, ee | 690 B, H, T, ee |  | 65FR66443 |
| 81 | N-Nitrosodimethylamine | 62759 |  |  |  |  | 0.00069 B, C | 3.0 B, C |  | 65FR66443 |
| 82 | N-Nitrosodi-n-Propylamine | 621647 |  |  |  |  | 0.0050 B, C | 0.51 B, C |  | 65FR66443 |
| 83 | N-Nitrosodiphenylamine | 86306 |  |  |  |  | 3.3 B, C | 6.0 B, C |  | 65FR66443 |
| 84 | Pyrene | 129000 |  |  |  |  | 830 B, ee | 4,000 B, ee |  | 65FR66443 |
| 85 | 1, 2, 4-Trichlorobenzene | 120821 |  |  |  |  | 260 ee | 940 ee | 70 ee | IRIS 11/01/96 SDWA |
| 86 | Aldrin | 309002 | 3.0 G, X |  | 1.3 G, X |  | 0.000049 B, C | 0.000050 B, C |  | 65FR31682 65FR66443 |
| 87 | alpha-BHC | 319846 |  |  |  |  | 0.0026 B, C | 0.0049 B, C |  | 65FR66443 |
| 88 | beta-BHC | 319857 |  |  |  |  | 0.0091 B, C | 0.017 B, C |  | 65FR66443 |
| 89 | gamma-BHC (Lindane) | 58899 | 0.95 K |  | 0.16 G |  | 0.019 C | 0.063 C | 0.2 C | 65FR31682 65FR66443 SDWA |
| 90 | Chlordane | 57749 | 2.4 G | 0.0043 G, X | 0.09 G | 0.004 G, X | 0.00080 B, C | 0.00081 B, C | 2 C | 65FR31682 65FR66443 SDWA |
| 91 | 4, 4’-DDT | 50293 | 1.1 G, gg | 0.001 G, X, gg | 0.13 G, gg | 0.001 G, X, gg | 0.00022 B, C | 0.00022 B, C |  | 65FR31682 65FR66443 |
| 92 | 4, 4’-DDE | 72559 |  |  |  |  | 0.00022 B, C | 0.00022 B, C |  | 65FR66443 |
| 93 | 4, 4’-DDD | 72548 |  |  |  |  | 0.00031 B, C | 0.00031 B, C |  | 65FR66443 |
| 94 | Dieldrin | 60571 | 0.24 K | 0.056 K, N | 0.71 G | 0.0019 G, X | 0.000052 B, C | 0.000054 B, C |  | 65FR31682 65FR66443 |
| 95 | alpha-Endosulfan | 959988 | 0.22 G, W | 0.056 G, W | 0.034 G, W | 0.0087 G, W | 62 B, ee | 89 B, ee |  | 65FR31682 65FR66443 |
| 96 | beta-Endosulfan | 33213659 | 0.22 G, W | 0.056 G, W | 0.034 G, W | 0.0087 G, W | 62 B, ee | 89 B, ee |  | 65FR31682 65FR66443 |
| 97 | Endosulfan Sulfate | 1031078 |  |  |  |  | 62 B, ee | 89 B, ee |  | 65FR31682 65FR66443 |
| 98 | Endrin | 72208 | 0.086 K | 0.036 K, N | 0.037 G | 0.0023 G, X | 0.76 B, ee | 0.81 B, H, ee | 2 ee | 65FR31682 SDWA |
| 99 | Endrin Aldehyde | 7421934 |  |  |  |  | 0.29 B, ee | 0.30 B, H, ee |  | 65FR66443 |
| 100 | Heptachlor | 76448 | 0.52 G | 0.0038 G, X | 0.053 G | 0.0036 G, X | 0.000079 B, C | 0.000079 B, C | 0.4 C | 65FR31682 65FR66443 SDWA |
| 101 | Heptachlor Epoxide | 1024573 | 0.52 G, U | 0.0038 G, U, X | 0.053 G, U | 0.0036 G, U, X | 0.000039 B, C | 0.000039 B, C | 0.2 C | 65FR31682 65FR66443 SDWA |
| 102 | Polychlorinated Biphenyls PCBs | -- |  | 0.014 M, X |  | 0.03 M, X | 0.000064 B, C, M | 0.000064 B, C, M | 0.5 C | 65FR31682 65FR66443 SDWA |
| 103 | Toxaphene | 8001352 | 0.73 | 0.0002 X | 0.21 | 0.0002 X | 0.00028 B, C | 0.00028 B, C | 3 C | 65FR31682 65FR66443 SDWA |

Footnotes:

A	This water quality criterion was derived from data for arsenic (III), but is applied here to total arsenic, which might imply that arsenic (III) and arsenic (V) are equally toxic to aquatic life and that their toxicities are additive.  In the arsenic criteria document (EPA 440/5-84-033, January 1985), Species Mean Acute Values are given for both arsenic (III) and arsenic (V) for five species and the ratios of the SMAVs for each species range from 0.6 to 1.7.  Chronic values are available for both arsenic (III) and arsenic (V) for one species; for the fathead minnow, the chronic value for arsenic (V) is 0.29 times the chronic value for arsenic (III).  No data are known to be available concerning whether the toxicities of the forms of arsenic to aquatic organisms are additive.

B	This criterion has been revised to reflect The Environmental Protection Agency’s q1* or RfD, as contained in the Integrated Risk Information System (IRIS) as of May 17, 2002.  The fish tissue bioconcentration factor (BCF) from the 1980 Ambient Water Quality Criteria document was retained in each case.

C	This criterion is based on carcinogenicity of 10-6 risk.  As prescribed in Section E of this regulation, application of this criterion for permit effluent limitations requires the use annual average flow or comparable tidal condition as determined by the Department.

D	Freshwater and saltwater criteria for metals are expressed in terms of total recoverable metals.  As allowed in Section E of this regulation, these criteria may be expressed as dissolved metal for the purposes of deriving permit effluent limitations.  The dissolved metal water quality criteria value may be calculated by using these 304(a) aquatic life criteria expressed in terms of total recoverable metal, and multiplying it by a conversion factor (CF).  The term “Conversion Factor” (CF) represents the conversion factor for converting a metal criterion expressed as the total recoverable fraction in the water column to a criterion expressed as the dissolved fraction in the water column.  (Conversion Factors for saltwater CCCs are not currently available.  Conversion factors derived for saltwater CMCs have been used for both saltwater CMCs and CCCs).  See “Office of Water Policy and Technical Guidance on Interpretation and Implementation of Aquatic Life Metals Criteria”, October 1, 1993, by Martha G. Prothro, Acting Assistant Administrator for Water, available from the Water Resource center, USEPA, 401 M St., SW, mail code RC4100, Washington, DC 20460; and 40CFR§131.36(b)(1).  Conversion Factors can be found in Attachment 1 – Conversion Factors for Dissolved Metals.

E	The freshwater criterion for this metal is expressed as a function of hardness (mg/L) in the water column.  The value given here corresponds to a hardness of 25 mg/L as expressed as CaCO3.  Criteria values for other hardness may be calculated from the following:  CMC (dissolved) = exp{mA [ln( hardness)]+ bA} (CF), or CCC (dissolved) = exp{mC [ln (hardness)]+ bC} (CF) and the parameters specified in Attachment 2 – Parameters for Calculating Freshwater Dissolved Metals Criteria That Are Hardness-Dependent.  As noted in footnote D above, the values in this appendix are expressed as total recoverable, the criterion may be calculated from the following:  CMC (total) = exp{mA [ln( hardness)]+ bA}, or CCC (total) = exp{mC [ln (hardness)]+ bC}.

F	Freshwater aquatic life values for pentachlorophenol are expressed as a function of pH, and are calculated as follows:  CMC = exp(1.005(pH)-4.869); CCC = exp(1.005(pH)-5.134).  Values displayed in table correspond to a pH of 7.8.

G	This criterion is based on 304(a) aquatic life criterion issued in 1980, and was issued in one of the following documents: Aldrin/Dieldrin (EPA 440/5-80-019), Chlordane (EPA 440/5-80-027), DDT (EPA 440/5-80-038), Endosulfan (EPA 440/5-80-046), Endrin (EPA 440/5-80-047), Heptachlor (440/5-80-052), Hexachlorocyclohexane (EPA 440/5-80-054), Silver (EPA 440/5-80-071).  The Minimum Data Requirements and derivation procedures were different in the 1980 Guidelines than in the 1985 Guidelines.  For example, a “CMC” derived using the 1980 Guidelines was derived to be used as an instantaneous maximum.  If assessment is to be done using an averaging period, the values given should be divided by 2 to obtain a value that is more comparable to a CMC derived using the 1985 Guidelines.

H 	No criterion for protection of human health from consumption of aquatic organisms excluding water was presented in the 1980 criteria document or in the 1986 Quality Criteria for Water.  Nevertheless, sufficient information was presented in the 1980 document to allow the calculation of a criterion, even though the results of such a calculation were not shown in the document.

I 	This criterion for asbestos is the Maximum Contaminant Level (MCL) developed under the Safe Drinking Water Act (SDWA) and the National Primary Drinking Water Regulation (NPDWR).

J	EPA has not calculated a 304(a) human health criterion for this contaminant.  The criterion is the Maximum Contaminant Level developed under the Safe Drinking Water Act (SDWA) and the National Primary Drinking Water Regulation (NPDWR).

K	This criterion is based on a 304(a) aquatic life criterion that was issued in the 1995 Updates: Water Quality Criteria Documents for the Protection of Aquatic Life in Ambient Water, (EPA-820-B-96-001, September 1996).  This value was derived using the GLI Guidelines (60FR15393-15399, March 23, 1995; 40CFR132 Appendix A); the difference between the 1985 Guidelines and the GLI Guidelines are explained on page  iv of the 1995 Updates.  None of the decisions concerning the derivation of this criterion were affected by any considerations that are specific to the Great Lakes.

L	The CMC = 1/[(f1/CMC1) + (f2/CMC2)] where f1 and f2 are the fractions of total selenium that are treated as selenite and selenate, respectively, and CMC1 and CMC2 are 185.9 g /l and 12.82 g /l, respectively.

M	This criterion applies to total PCBs, (e.g., the sum of all congener or all isomer or homolog or Aroclor analyses.)

N	The derivation of the CCC for this pollutant did not consider exposure through the diet, which is probably important for aquatic life occupying upper trophic levels.

O	This state criterion is also based on a total fish consumption rate of 0.0175 kg/day.

P	This water quality criterion is expressed as g free cyanide (as CN)/L.

Q	This value was announced (61FR58444-58449, November 14, 1996) as a proposed GLI 303 I aquatic life criterion

R	This water quality criterion for arsenic refers to the inorganic form only.

S	This water quality criterion for selenium is expressed in terms of total recoverable metal in the water column.  It is scientifically acceptable to use the conversion factor (0.996 – CMC or 0.922 – CCC) that was used in the GLI to convert this to a value that is expressed in terms of dissolved metal.

T	The organoleptic effect criterion is more stringent than the value for priority toxic pollutants.

U	This value was derived from data for heptachlor and the criteria document provides insufficient data to estimate the relative toxicities of heptachlor and heptachlor epoxide.

V	There is a full set of aquatic life toxicity data that show that DEHP is not toxic to aquatic organisms at or below its solubility limit.

W	This value was derived from data for endosulfan and is most appropriately applied to the sum of alpha-endosulfan and beta-endosulfan.

X 	This criterion is based on a 304(a) aquatic life criterion issued in 1980 or 1986, and was issued in one of the following documents: Aldrin/Dieldrin (EPA440/5-80-019), Chlordane (EPA 440/5-80-027), DDT (EPA 440/5-80-038), Endrin (EPA 440/5-80-047), Heptachlor (EPA 440/5-80-052), Polychlorinated Biphenyls (EPA 440/5- 80-068), Toxaphene (EPA 440/5-86-006).  This CCC is based on the Final Residue value procedure in the 1985 Guidelines.  Since the publication of the Great Lakes Aquatic Life Criteria Guidelines in 1995 (60FR15393-15399, March 23, 1995), the EPA no longer uses the Final Residue value procedure for deriving CCCs for new or revised 304(a) aquatic life criteria.

Y	This water quality criterion is based on a 304(a) aquatic life criterion that was derived using the 1985 Guidelines (Guidelines for Deriving Numerical National Water Quality Criteria for the Protection of Aquatic Organisms and Their Uses, PB85-227049, January 1985) and was issued in one of the following criteria documents: Arsenic (EPA 440/5-84-033), Cadmium (EPA 440/5-84-032), Chromium (EPA 440/5-84-029), Copper (EPA 440/5-84-031), Cyanide (EPA 440/5-84-028), Lead (EPA 440/5-84-027), Nickel (EPA 440/5-86-004), Pentachlorophenol (EPA 440/5-86-009), Toxaphene, (EPA 440/5-86-006), Zinc (EPA 440/5-87- 003).

Z	When the concentration of dissolved organic carbon is elevated, copper is substantially less toxic and use of Water-Effect Ratios might be appropriate.

aa	The selenium criteria document (EPA 440/5-87-006, September 1987) provides that if selenium is as toxic to saltwater fishes in the field as it is to freshwater fishes in the field, the status of the fish community should be monitored whenever the concentration of selenium exceeds 5.0 g/L in salt water because the saltwater CCC does not take into account uptake via the food chain.

bb	This water quality criterion was derived on page 43 of the mercury criteria document (EPA 440/5-84-026, January 1985).  The saltwater CCC of 0.025 ug/L given on page 23 of the criteria document is based on the Final Residue value procedure in the 1985 Guidelines.  Since the publication of the Great Lakes Aquatic Life criteria Guidelines in 1995 (60FR15393-15399, March 23, 1995), the EPA no longer uses the Final Residue value procedure for deriving CCCs for new or revised 304(a) aquatic life criteria.

cc	This water quality criterion was derived in Ambient Water Quality Criteria Saltwater Copper Addendum (Draft, April 14, 1995) and was promulgated in the Interim Final National Toxics Rule (60FR22228-222237, May 4, 1995).

dd	This water quality criterion was derived from data for inorganic mercury (II), but is applied here to total mercury.  If a substantial portion of the mercury in the water column is methylmercury, this criterion will probably be under protective.  In addition, even though inorganic mercury is converted to methylmercury and methylmercury bioaccumulates to a great extent, this criterion does not account for uptake via the food chain because sufficient data were not available when the criterion was derived.

ee	This criterion is a noncarcinogen.  As prescribed in Section E of this regulation, application of this criterion for determining permit effluent limitations requires the use of 7Q10 or comparable tidal condition as determined by the Department.

ff	EPA is currently reassessing the criteria for arsenic.

gg	This criterion applies to DDT and its metabolites (i.e., the total concentration of DDT and its metabolites should not exceed this value).

hh  Although a new RfD is available in IRIS, the surface water criteria will not be revised until the National Primary Drinking Water Regulations: Stage 2 Disinfectants and Disinfection Byproducts Rule (Stage 2 DBPR) is completed, since public comment on the relative source contribution (RSC) for chloroform is anticipated.

ii	Although EPA has not published a completed criteria document for phthalate, it is EPA’s understanding that sufficient data exist to allow calculation of aquatic life criteria.

NONPRIORITY POLLUTANTS

| Non Priority Pollutant |  | CAS Number | Freshwater Aquatic Life |  | Saltwater Aquatic Life |  | Human Health |  |  |  | FR Cite/Source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | CMC (g/L) | CCC (g/L) | CMC (g/L) | CCC (g/L) | For Consumption of: |  |  | MCL (g/L) |  |
|  |  |  |  |  |  |  | Water & Organism (g/L) | Organism Only (g/L) |  |  |  |
| 1 | Alachlor |  |  |  |  |  |  |  |  | 2 M | SDWA |
| 2 | Ammonia | 7664417 | CRITERIA ARE pH AND TEMPERATURE DEPENDENT - SEE DOCUMENT FOR DETAILS C |  |  |  |  |  |  |  | EPA822-R99-014 EPA440/5-88-004 |
| 3 | Aesthetic Qualities |  | NARRATIVE STATEMENT AND NUMERIC CRITERIA – SEE TEXT |  |  |  |  |  |  |  | Gold Book |
| 4 | Atrazine |  |  |  |  |  |  |  |  | 3 M | SDWA |
| 5 | Bacteria |  | FOR PRIMARY CONTACT RECREATION AND SHELLFISH USES – SEE TEXT |  |  |  |  |  |  |  | Gold Book |
| 6 | Barium | 7440393 |  |  |  |  | 1,000 A, L |  |  | 2,000 L | Gold Book |
| 7 | Carbofuran | 1563662 |  |  |  |  |  |  |  | 40 L | SDWA |
| 8 | Chlorine | 7782505 | 19 | 11 | 13 | 7.5 |  |  |  | G | Gold Book SDWA |
| 9 | Chlorophenoxy Herbicide 2, 4, 5, -TP | 93721 |  |  |  |  | 10 A, L |  |  | 50 L | Gold Book SDWA |
| 10 | Chlorophenoxy Herbicide 2, 4-D | 94757 |  |  |  |  | 100 A, L |  |  | 70 L | Gold Book SDWA |
| 11 | Chlorophyll a |  | NARRATIVE STATEMENT AND NUMERIC CRITERIA – SEE TEXT |  |  |  |  |  |  |  | State Standard |
| 12 | Chloropyrifos | 2921882 | 0.083 F | 0.041 F | 0.011 F | 0.0056 F |  |  |  |  | Gold Book |
| 13 | Color |  | NARRATIVE STATEMENT – SEE TEXT |  |  |  |  |  |  |  | State Standard |
| 14 | Dalapon | 75990 |  |  |  |  |  |  |  | 200 L | SDWA |
| 15 | Demeton | 8065483 |  | 0.1 E |  | 0.1 E |  |  |  |  | Gold Book |
| 16 | 1, 2-Dibromo-3-chloropropane (DBCP) | 96128 |  |  |  |  |  |  |  | 0.2 M | SDWA |
| 17 | Di(2-ethylhexyl) adipate | 103231 |  |  |  |  |  |  |  | 400 L | SDWA |
| 18 | Dinoseb | 88857 |  |  |  |  |  |  |  | 7 L | SDWA |
| 19 | Dinitrophenols | 25550587 |  |  |  |  | 69 L | 5,300 L |  |  | 65FR66443 |
| 20 | Diquat | 85007 |  |  |  |  |  |  |  | 20 L | SDWA |
| 21 | Endothall | 145733 |  |  |  |  |  |  |  | 100 L | SDWA |
| 22 | Ether, Bis Chloromethyl | 542881 |  |  |  |  | 0.00010 D, M | 0.00029 D, M |  |  | 65FR66443 |
| 23 | Cis-1, 2-dichloroethylene | 156592 |  |  |  |  |  |  |  | 70 L | SDWA |
| 24 | Ethylene dibromide |  |  |  |  |  |  |  |  | 0.05 M | SDWA |
| 25 | Fluoride | 7681494 |  |  |  |  |  |  |  | 4000 L | SDWA |
| 26 | Glyphosate | 1071836 |  |  |  |  |  |  |  | 700 L | SDWA |
| 27 | Guthion | 86500 |  | 0.01 E |  | 0.01 E |  |  |  |  | Gold Book |
| 28 | Hexachlorocyclo-hexane-Technical | 319868 |  |  |  |  | 0.0123 L | 0.0414 L |  |  | Gold Book |
| 29 | Iron | 7439896 |  | 1,000 E |  |  | 300 A, L |  |  |  | Gold Book |
| 30 | Malathion | 121755 |  | 0.1 E |  | 0.1 E |  |  |  |  | Gold Book |
| 31 | Manganese | 7439965 |  |  |  |  | 50 A, L, N | 100 A, L |  |  | Gold Book |
| 32 | Methoxychlor | 72435 |  | 0.03 E |  | 0.03 E | 100 A, L |  |  | 40 L | Gold Book SDWA |
| 33 | Mirex | 2385855 |  | 0.001 E |  | 0.001 E |  |  |  |  | Gold Book |
| 34 | Nitrates | 14797558 |  |  |  |  | 10, 000 L |  |  | 10, 000 L | SDWA Gold Book |
| 35 | Nitrites | 14797650 |  |  |  |  |  |  |  | 1,000 L | SDWA |
| 36 | Nitrogen, Total |  | NARRATIVE STATEMENT AND NUMERIC CRITERIA - SEE TEXT |  |  |  |  |  |  |  | State Standard |
| 37 | Nitrosamines |  |  |  |  |  | 0.0008 L | 1.24 L |  |  | Gold Book |
| 38 | Nitrosodibutylamine, N | 924163 |  |  |  |  | 0.0063 A, M | 0.22 A, M |  |  | 65FR66443 |
| 39 | Nitrosodiethylamine, N | 55185 |  |  |  |  | 0.0008 A, M | 1.24 A, M |  |  | Gold Book |
| 40 | Nitrosopyrrolidine, N | 930552 |  |  |  |  | 0.016 M | 34 M |  |  | 65FR66443 |
| 41 | Oil and Grease |  | NARRATIVE STATEMENT – SEE TEXT |  |  |  |  |  |  |  | Gold Book |
| 42 | Oxamyl | 23135220 |  |  |  |  |  |  |  | 200 L | SDWA |
| 43 | Oxygen, Dissolved | 7782447 | WARMWATER, COLDWATER, AND EXCEPTIONS FOR NATURAL CONDITIONS - SEE TEXT K |  |  |  |  |  |  |  | Gold Book State Standard |
| 44 | Parathion | 56382 | 0.065 H | 0.013 H |  |  |  |  |  |  | Gold Book |
| 45 | Pentachlorobenzene | 608935 |  |  |  |  | 1.4 E | 1.5 E |  |  | 65FR66443 |
| 46 | pH |  | SEE TEXT I |  |  |  |  |  |  |  | Gold Book State Standard |
| 47 | Phosphorus, Total |  | NARRATIVE STATEMENT AND NUMERIC CRITERIA - SEE TEXT |  |  |  |  |  |  |  | State Standard |
| 48 | Picloram | 1918021 |  |  |  |  |  |  |  | 500 L | SDWA |
| 49 | Salinity |  | NARRATIVE STATEMENT - SEE TEXT |  |  |  |  |  |  |  | Gold Book |
| 50 | Simazine | 122349 |  |  |  |  |  |  |  | 4 L | SDWA |
| 51 | Solids Suspended and Turbidity |  | NARRATIVE STATEMENT AND NUMERIC CRITERIA - SEE TEXT |  |  |  |  |  |  |  | Gold Book State Standard |
| 52 | Styrene | 100425 |  |  |  |  |  |  |  | 100 L | SDWA |
| 53 | Sulfide-Hydrogen Sulfide | 7783064 |  | 2.0 E |  | 2.0 E |  |  |  |  | Gold Book |
| 54 | Tainting Substances |  | NARRATIVE STATEMENT - SEE TEXT |  |  |  |  |  |  |  | Gold Book |
| 55 | Temperature |  | SPECIES DEPENDENT CRITERIA - SEE TEXT J |  |  |  |  |  |  |  | Red Book |
| 56 | 1, 2, 4, 5-Tetrachlorobenzene | 95943 |  |  |  |  | 0.97 D | 1.1 D |  |  | 65FR66443 |
| 57 | Tributyltin (TBT) | 688733 | 0.46 | 0.063 | 0.37 | 0.010 |  |  |  |  | EPA 822-F-00-008 |
| 58 | 2, 4, 5-Trichlorophenol | 95954 |  |  |  |  | 1,800 B, D | 3,600 B, D |  |  | 65FR66443 |
| 59 | Xylenes, Total |  |  |  |  |  |  |  |  | 10, 000 L | SDWA |
| 60 | Uranium |  |  |  |  |  |  |  |  | 30 | SDWA |
| 61 | Beta particles and photon emitters |  |  |  |  |  |  |  |  | 4 Millirems/yr | SDWA |
| 62 | Gross alpha particle activity |  |  |  |  |  |  |  |  | 15 picocuries per liter (pCi/l) | SDWA |
| 63 | Radium 226 and Radium 228 (combined) |  |  |  |  |  |  |  |  | 5 pCi/l | SDWA |

Footnotes:

A	This human health criterion is the same as originally published in the Red Book which predates the 1980 methodology and did not utilize the fish ingestion BCF approach.   This same criterion value is now published in the Gold Book.

B 	The organoleptic effect criterion is more stringent than the value presented in the nonpriority pollutants table.

C	According to the procedures described in the Guidelines for Deriving Numerical National Water Quality Criteria for the Protection of Aquatic Organisms and Their Uses, except possibly where a very sensitive species is important at a site, freshwater aquatic life should be protected if both conditions specified in Attachment 3 - Calculation of Freshwater Ammonia Criterion are satisfied.

D	This criterion has been revised to reflect The Environmental Protection Agency’s q1* or RfD, as contained in the Integrated Risk Information System (IRIS) as of April 8, 1998.  The fish tissue bioconcentration factor (BCF) used to derive the original criterion was retained in each case.

E	The derivation of this value is presented in the Red Book (EPA 440/9-76-023, July, 1976).

F	This value is based on a 304(a) aquatic life criterion that was derived using the 1985 Guidelines (Guidelines for Deriving Numerical National Water Quality Criteria for the Protection of Aquatic Organisms and Their Uses, PB85-227049, January 1985) and was issued in the following criteria document: Chloropyrifos (EPA 440/5-86-005).

G	A more stringent Maximum Residual Disinfection Level (MRDL) has been issued by EPA under the Safe Drinking Water Act.  Refer to S.C. Regulation 61-58, State Primary Drinking Water Regulations.

H	This value is based on a 304(a) aquatic life criterion that was issued in the 1995 Updates: Water Quality Criteria Documents for the Protection of Aquatic Life in  Ambient Water (EPA-820-B-96-001).  This value was derived using the GLI Guidelines (60FR15393-15399, March 23, 1995; 40CFR132 Appendix A); the differences between the 1985 Guidelines and the GLI Guidelines are explained on page iv of the 1995 Updates.  No decision concerning this criterion was affected by any considerations that are specific to the Great Lakes.

I	South Carolina has established some site-specific standards for pH.  These site-specific standards are listed in S.C. Regulation 61-69, Classified Waters.

J	U.S. EPA, 1976, Quality Criteria for Water 1976.

K	South Carolina has established numeric criteria in Section G for waters of the State based on the protection of warmwater and coldwater species.  For the exception to be used for waters of the State that do not meet the numeric criteria established for the waterbody due to natural conditions, South Carolina has specified the allowable deficit in Section D.4. and used the following document as a source.  U.S. EPA,  1986,  Ambient Water Quality Criteria for Dissolved Oxygen,  EPA 440/5-86-003,  National Technical Information Service, Springfield, VA.  South Carolina has established some site-specific standards for DO.  These site-specific standards are listed in S.C. Regulation 61-69, Classified Waters.

L	This criterion is a noncarcinogen.  As prescribed in Section E of this regulation, application of this criterion for determining permit effluent limitations requires the use of 7Q10 or comparable tidal condition as determined by the Department

M	This criterion is based on an added carcinogenicity risk.     As prescribed in Section E of this regulation, application of this criterion for permit effluent limitations requires the use annual average flow or comparable tidal condition as determined by the Department.

N	This criterion for manganese is not based on toxic effects, but rather is intended to minimize objectionable qualities such as laundry stains and objectionable tastes in beverages.

ORGANOLEPTIC EFFECTS

| Pollutant |  | CAS Number | Organoleptic Effect Criteria (g/L) | FR Cite/Source |
| --- | --- | --- | --- | --- |
| 1 | Acenaphthene | 83329 | 20 | Gold Book |
| 2 | Chlorobenzene | 108907 | 20 | Gold Book |
| 3 | 3-Chlorophenol |  | 0.1 | Gold Book |
| 4 | 4-Chlorophenol | 106489 | 0.1 | Gold Book |
| 5 | 2, 3-Dichlorophenol |  | 0.04 | Gold Book |
| 6 | 2, 5-Dichlorophenol |  | 0.5 | Gold Book |
| 7 | 2, 6-Dichlorophenol |  | 0.2 | Gold Book |
| 8 | 3, 4-Dichlorophenol |  | 0.3 | Gold Book |
| 9 | 2, 4, 5-Trichlorophenol | 95954 | 1 | Gold Book |
| 10 | 2, 4, 6-Trichlorophenol | 88062 | 2 | Gold Book |
| 11 | 2, 3, 4, 6-Tetrachlorophenol |  | 1 | Gold Book |
| 12 | 2-Methyl-4-Chlorophenol |  | 1,800 | Gold Book |
| 13 | 3-Methyl-4-Chlorophenol | 59507 | 3,000 | Gold Book |
| 14 | 3-Methyl-6-Chlorophenol |  | 20 | Gold Book |
| 15 | 2-Chlorophenol | 95578 | 0.1 | Gold Book |
| 16 | Copper | 7440508 | 1,000 | Gold Book |
| 17 | 2, 4-Dichlorophenol | 120832 | 0.3 | Gold Book |
| 18 | 2, 4-Dimethylphenol | 105679 | 400 | Gold Book |
| 19 | Hexachlorocyclopentadiene | 77474 | 1 | Gold Book |
| 20 | Nitrobenzene | 98953 | 30 | Gold Book |
| 21 | Pentachlorophenol | 87865 | 30 | Gold Book |
| 22 | Phenol | 108952 | 300 | Gold Book |
| 23 | Zinc | 7440666 | 5,000 | 45FR79341 |

Footnote:

1.	These criteria are based on organoleptic (taste and odor) effects.  Because of variations in chemical nomenclature systems,  this listing of pollutants does not duplicate the listing in Appendix A of 40 CFR Part 423.  Also listed are the Chemical Abstracts Service (CAS) registry numbers, which provide a unique identification for each chemical.

WATER QUALITY CRITERIA ADDITIONAL NOTES

1.	Criteria Maximum Concentration and Criterion Continuous Concentration

The Criteria Maximum Concentration (CMC) is an estimate of the highest concentration of a material in surface water to which an aquatic community can be exposed briefly without resulting in an unacceptable effect.  The Criterion Continuous Concentration (CCC) is an estimate of the highest concentration of a material in surface water to which an aquatic community can be exposed indefinitely without resulting in an unacceptable effect.  The CMC and CCC are just two of the six parts of a aquatic life criterion; the other four parts are the acute averaging period, chronic averaging period, acute frequency of allowed exceedence, and chronic frequency of allowed exceedence.

2.	Criteria for Priority Pollutants, Non Priority Pollutants and Organoleptic Effects

This appendix lists all priority toxic pollutants and some nonpriority toxic pollutants, and both human health effect and organoleptic effect criteria issued pursuant to CWA §304(a), the SDWA, and the NPDWR.  Blank spaces indicate that EPA has no CWA §304(a) criteria recommendations.   Because of variations in chemical nomenclature systems, this listing of toxic pollutants does not duplicate the listing in Appendix A of 40CFR Part 423.

3.	Human Health Risk

The human health criteria for the priority and nonpriority pollutants are based on carcinogenicity of 10-6 risk.

4.	Water Quality Criteria published pursuant to Section 304(a) or Section 303(c) of the CWA

Many of the values in the appendix were published in the California Toxics Rule.  Although such values were published pursuant to Section 303(c) of the CWA, they represent the EPA’s most recent calculation of water quality criteria.

5.	Calculation of Dissolved Metals Criteria

The 304(a) criteria for metals are shown as total recoverable metals.  As allowed in Section E of this regulation, these criteria may be expressed as dissolved metals.  Dissolved metals criteria may be calculated in one of two ways (please refer to Attachments).  For freshwater metals criteria that are hardness-dependent, the dissolved metal criteria may be calculated using a hardness of 25 mg/l as expressed as CaCO3.  Saltwater and freshwater metals’ criteria that are not hardness-dependent are calculated by multiplying the total recoverable criteria before rounding by the appropriate conversion factors.  The final metals’ criteria in the table are rounded to two significant figures.  Information regarding the calculation of hardness dependent conversion factors are included in the footnotes.

6.	Chemical Abstract Services Number

The Chemical Abstract Services number (CAS) for each pollutant is provided (where available).

7.	Gold Book Reference

The Gold Book reference listed in the appendix refers to the May 1, 1986 EPA publication EPA 440/5-86-001.

8.	Federal Register Reference

The FR listed in the appendix refers to the appropriate Federal Register listing. and source refers to the origin of the value.  Many of the numeric values contained in this appendix  have been modified, revised, or altered and therefore, the source as listed may not be the same as it appears in this table.   Also, South Carolina may have selected to use a different value or may have promulgated a different value in its previous iterations of this regulation, so differences from these sources should be expected.

9.	Maximum Contaminant Levels

The appendix includes Maximum Contaminant Levels (MCLs) developed under the Safe Drinking Water Act (SDWA) and the National Primary Drinking Water Regulation (NPDWR).

10.	Organoleptic Effects

The appendix contains 304(a) criteria for pollutants with toxicity-based criteria as well as non-toxicity based criteria.  The basis for the non-toxicity based criteria are organoleptic effects (e.g., taste and odor) which would make water and edible aquatic life unpalatable but not toxic to humans.  The table includes criteria for organoleptic effects for 23 pollutants.  Pollutants with organoleptic effect criteria more stringent than the criteria based on toxicity (e.g., included in both the priority and non-priority pollutant tables) are footnoted as such.

11.	Category Criteria

In the 1980 criteria documents, certain water quality criteria were published for categories of pollutants rather than for individual pollutants within that category.  Subsequently, in a series of separate actions, the EPA derived criteria for specific pollutants within a category.  Therefore, in this appendix South Carolina is replacing criteria representing categories with individual pollutant criteria (e.g., 1, 3-dichlorobenzene, 1, 4-dichlorobenzene and 1, 2-dichlorobenzene).

12.	Specific Chemical Calculations

A.	Selenium

(1) Human Health

In the 1980 Selenium document, a criterion for the protection of human health from consumption of water and organisms was calculated based on a BCF of 6.0 l/kg and a maximum water-related contribution of 35 g Se/day.  Subsequently, the EPA Office of Health and Environmental Assessment issued an errata notice (February 23, 1982), revising the BCF for selenium to 4.8 L/kg.  In 1988, EPA issued an addendum (ECAO-CIN-668) revising the human health criteria for selenium.  Later in the final National Toxic Rule (NTR, 57 FR 60848), EPA withdrew previously published selenium human health criteria, pending EPA review of new epidemiological data.

This appendix includes human health criteria for selenium, calculated using a BCF of 4.8 L/kg along with the current IRIS RfD of 0.005 mg/kg/day.  South Carolina included these water quality criteria in the appendix because the data necessary for calculating a criteria in accordance with EPA’s 1980 human health methodology are available.

(2) Aquatic Life

This appendix contains aquatic life criteria for selenium that are the same as those published in the CTR.  In the CTR, EPA proposed an acute criterion for selenium based on the criterion proposed for selenium in the Water Quality Guidance for the Great Lakes System (61FR584440.  The GLI and CTR proposals take into account data showing that selenium’s two prevalent oxidation state in water, selenite and selenate, present differing potentials for aquatic toxicity, as well as new data indication that various forms of selenium are additive.  The new approach produces a different selenium acute criterion concentration, or CMC, depending upon the relative proportions of selenite, selenate, and other forms of selenium that are present.  EPA is currently undertaking a reassessment of selenium, and expects the 304(a) criterion for selenium will be revised based on the final reassessment (63FR26186).  However, until such time as revised water quality criteria for selenium are published by the EPA, the water quality criteria in this appendix are EPA’s current 304(a) criteria.

B.	Chromium (III)

The aquatic life water quality criteria for chromium (III) included in the appendix are based on the values presented in the document titled: 1995 Updates: Water Quality Criteria Documents for the Protection of Aquatic Life in Ambient Water.

C.	PCBs

In this appendix, South Carolina is publishing aquatic life and human health criteria based on total PCBs rather than individual arochlors.

Attachment 1 - Conversion Factors for Dissolved Metals

| Metal | Conversion Factor freshwater CMC | Conversion Factor freshwater CCC | Conversion Factor saltwater CMC | Conversion Factor saltwater CCC |
| --- | --- | --- | --- | --- |
| Arsenic | 1.000 | 1.000 | 1.000 | 1.000 |
| Cadmium | 1.136672-[(ln hardness)(0.041838)] | 1.101672-[(ln hardness)(0.041838)] | 0.994 | 0.994 |
| Chromium III | 0.316 | 0.860 | -- | -- |
| Chromium VI | 0.982 | 0.962 | 0.993 | 0.993 |
| Copper | 0.960 | 0.960 | 0.83 | 0.83 |
| Lead | 1.46203-[(ln hardness)(0.145712)] | 1.46203-[(ln hardness)(0.145712)] | 0.951 | 0.951 |
| Mercury | 0.85 | 0.85 | 0.85 | 0.85 |
| Nickel | 0.998 | 0.997 | 0.990 | 0.990 |
| Selenium | -- | -- | 0.998 | 0.998 |
| Silver | 0.85 | -- | 0.85 | -- |
| Zinc | 0.978 | 0.986 | 0.946 | 0.946 |

# Attachment 2 - Parameters for Calculating Freshwater Dissolved Metals Criteria That Are Hardness-Dependent

| Chemical | mA | bA | mC | bC | Freshwater Conversion Factors (CF) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Acute | Chronic |
| Cadmium | 1.0166 | -3.924 | 0.7409 | -4.719 | 1.136672-[ln (hardness)(0.041838)] | 1.101672-[ln (hardness)(0.041838)] |
| Chromium III | 0.8190 | 3.7256 | 0.8190 | 0.6848 | 0.316 | 0.860 |
| Copper | 0.9422 | -1.700 | 0.8545 | -1.702 | 0.960 | 0.960 |
| Lead | 1.273 | -1.460 | 1.273 | -4.705 | 1.46203-[ln (hardness)(0.145712)] | 1.46203-[ln (hardness)(0.145712)] |
| Nickel | 0.8460 | 2.255 | 0.8460 | 0.0584 | 0.998 | 0.997 |
| Silver | 1.72 | -6.52 | -- | -- | 0.85 | -- |
| Zinc | 0.8473 | 0.884 | 0.8473 | 0.884 | 0.978 | 0.986 |

Hardness-dependent metals criteria may be calculated from the following:

CMC (total) = exp{mA [ln( hardness)]+ bA}, or CCC (total) = exp{mC [ln (hardness)]+ bC}

CMC (dissolved) = exp{mA [ln( hardness)]+ bA} (CF), or CCC (dissolved) = exp{mC [ln (hardness)]+ bC} (CF).

# Attachment 3 - Calculation of Freshwater Ammonia Criterion

1.  The one-hour average concentration of total ammonia nitrogen (in mg N/L) does not exceed, more than once every three years on the average, the CMC calculated using the following equation:

CMC=0.2751+107.204−pH+39.01+10pH−7.204

In situations where salmonids are absent, the CMC may be calculated using the following equation:

CMC=0.4111+107.204−pH+58.41+10pH−7.204

2.  The thirty-day average concentration of total ammonia nitrogen (in mg N/L) does not exceed, more than once every three years on the average, the CCC calculated using the following equations:

When fish early life stages (ELS) are present:

CCC=0.05771+107.688−pH+2.4871+10pH−7.688×min2.85,1.45×100.028×25−T

When fish early life stages are absent:

CCC=0.05771+107.688−pH+2.4871+10pH−7.688×1.45×100.028×25−maxT,7

and the highest four-day average within the 30-day period does not exceed 2.5 times the CCC.

In the absence of information substantiating that ELS are absent, the ELS present equation will be used.

Fiscal Impact Statement:

No costs to the State or significant cost to its political subdivisions as a whole should be incurred by these amendments.  See Statement of Need and Reasonableness below.

Statement of Need and Reasonableness:

The statement of need and reasonableness was determined by staff analysis pursuant to S.C. Code Section 1-23-115(C)(1)-(3) and (9)-(11):

DESCRIPTION OF REGULATION: Amendment of Regulation 61-68, Water Classifications and Standards.

Purpose: Proposed amendment of R.61-68 will clarify, strengthen, and improve the overall quality of the existing regulation and make appropriate revisions of the State's water quality standards in accordance with Section 303(c)(2)(B) of the Federal Clean Water Act (CWA).

Legal Authority: S.C. Code Sections 48-1-40, 48-1-60, and 48-1-80, implementing the CWA.

Plan for Implementation: The proposed amendment would be incorporated within R.61-68 upon approval of the General Assembly and publication in the State Register.  The proposed amendment will be implemented in the same manner in which the present regulation is implemented.

DETERMINATION OF NEED AND REASONABLENESS OF THE PROPOSED REGULATION BASED ON ALL FACTORS HEREIN AND EXPECTED BENEFIT: This amendment is required to comply with Federal requirements of Section 303(c)(2)(B) of the CWA.

-	The adoption of federal toxics criteria to reflect the most current final published criteria according to Sections 304(a) and 307(a) of the CWA.

The proposed changes to R.61-68 relating to human health and aquatic life criteria are reasonable because the stated criteria in the amendment are based on sound scientific principles and are required in order to comply with the goals of Section 101(a)(2) and 303(c) of the CWA for protection and maintenance of the uses of the waters of the State.  These changes include using a larger fish tissue consumption rate that better reflects true consumption patterns and provides a more protective risk level for bioaccumulative pollutants.

-	Review and revision of the bacterial indicator for protection of recreational uses.

The proposed changes reflect EPA=s requirement under the CWA amendments, also known as the Beaches Environmental Assessment and Coastal Health Act (BEACH Act amendments) that South Carolina adopt either E.coli or enterococci as its bacterial indicator organism for its coastal recreational waters by April of 2004.  The Department is proposing the use of enterococci for all of its saltwaters in order to comply with EPA’s requirements.

-	Inclusion of an allowance for a variance from water quality standards.

South Carolina’s current water quality standards do not include a provision for a variance from those standards.  The Department has recently reviewed the applicability of variances where under certain situations the use attainment may not be currently achieved but may be achieved at a future date.  The Department believes that a variance provision would be a beneficial and necessary inclusion for our state’s water quality standards.  We have included language that specifies when and how a variance may be granted by the Department.

-	Stylistic changes which may include corrections for: readability, grammar, punctuation, typography, codification, references, and language style.

DETERMINATION OF COSTS AND BENEFITS: Existing staff and resources will be utilized to implement this amendment to the regulation.  No additional cost will be incurred by the State if the revisions are implemented and therefore, no additional State funding is being requested.

In reviewing the potential for significant economic impact of the proposed amendment, the Department specifically evaluated situations in which costs would most likely be incurred by the regulated community. These estimates addressed the specific revisions by issue after determining those of greatest potential impact.  The Department found that the overall impact to the State=s political subdivisions or the regulated community as a whole was not likely to be significant in that the existing narrative standards would have incurred similar cost or the fact that the design standards required under the amendment will be substantially consistent with the current guidelines and review guidelines utilized by the Department.  Further, much of the proposed amendment, for which an estimated cost may be incurred by the regulated community at the time of permit issuance, are essential and necessary to protect and maintain the existing uses supported by the water quality standards and are, therefore, beyond the scope of cost analysis in that they provide the minimum level of protection for aquatic life and human health as required by the CWA.

UNCERTAINTIES OF ESTIMATES: Minimal to moderate.

EFFECT ON ENVIRONMENT AND PUBLIC HEALTH: Implementation of this amendment will not compromise the protection of the environment or the health and safety of the citizenry of the State.  The amendment will promote and protect aquatic life and human health by the regulation of pollutants into waters of the State.

DETRIMENTAL EFFECT ON THE ENVIRONMENT AND PUBLIC HEALTH IF THE REGULATION IS NOT IMPLEMENTED: Failure by the Department to incorporate appropriately protective water quality standards in the regulation that are the basis for issuance of National Pollutant Discharge Elimination System (NPDES) permits, stormwater permits, wasteload and load allocations, groundwater remediation plans, and multiple other program areas will lead to contamination of the waters of the State with detrimental effects on the health of flora and fauna in the State as well as the citizens of South Carolina.

Statement of Rationale:

The statement of rationale was determined by staff analysis pursuant to S.C. Code Section 1-23-110(A)(3)(h).

The first two issues contained in the proposed amendment of R.61-68 are requirements of the CWA and are necessary for compliance with EPA’s recommendations for the triennial review of the water quality standards to ensure consistency with the CWA.  The two remaining issues are Department initiated and are necessary and essential to the water quality standards program in South Carolina and to the quality of the regulation itself.  The water quality standards variance provision will ensure that water quality use standards may be maintained while providing for reasonable expectations for dischargers when all applicable classified uses cannot be attained at this time to the level of achieving a numeric criteria set for that classified use.  This will ensure a reasonable expectation of eventual achievement of the use and that meaningful progress towards that achievement will be reevaluated at least every three years at the time of the triennial review of the water quality standards in compliance with Section 303(c)(2)(B) of the CWA.  The remaining issue is one of revisions based entirely on corrections or clarity of the language in the regulation in order to maintain a regulation that is efficient, readable, and accurate.