AGY:	Building Codes Council, Department of Labor, Licensing and Regulation

FII:	5

FIV:	29

PRD:	20040827

EFD:	20050527

EXD:	20050511

REG:	2917

PRI:	8

PRV:	28

COM:	Labor, Commerce and Industry Committee 	12 SLCI

Labor, Commerce and Industry Committee 26 HLCI

RES:	494

STA:	Final

AUT:	6-9-40

SUB:	International Fuel Gas Code

HST:	2917

BY	DATE	ACTION DESCRIPTION	COM	VOL/ISSUE	EXP DATE	R. NUM

___	________	__________________________	_______	___________	__________	_______

-	20040827	Proposed Reg Published in SR		28/8

-	20050111	Received by Lt. Gov & Speaker			20050511

H	20050111	Referred to Committee 	HLCI 26

S	20050113	Referred to Committee 	SLCI 12

S	20050215	Resolution Intro to Approv	SLCI 12			S494

-	20050511	Approved by:  Expiration Date		29/5

TXT:

Document No. 2917

DEPARTMENT OF LABOR, LICENSING AND REGULATION

# BUILDING CODES COUNCIL

CHAPTER 8

Statutory Authority: 1976 Code Section 6-9-40

Synopsis:

The South Carolina Building Codes Council will update the International Fuel Gas Code, 2000 Edition, to the International Fuel Gas Code, 2003 Edition, in accordance with the statutory amendments to acts governing the Building Codes Council, including proposed modifications. These provisions will be established in regulations designated under Article 3—International Fuel Gas Code.

Instructions:

Add new Article 3—International Fuel Gas Code.

Text:

Article 3—International Fuel Gas Code

Regulation 8-300.  International Fuel Gas Code

NOTE-This article is identical to the International Fuel Gas Code, 2003 Edition, in accordance with the statutory amendments to acts governing the Building Codes Council, except for the modifications referenced below.

Regulation 8-301.  Modifications

Section G505.1.1

Where commercial cooking appliances are vented by means of the Type I or Type II kitchen exhaust hood system that serves such appliances, the exhaust system shall be fan powered and the appliances shall be interlocked with the exhaust hood system to prevent appliance operation when the exhaust hood system is not operating. Where a solenoid valve is installed in the gas piping as part of an interlock system, gas piping shall not be installed to bypass such valve. Dampers shall not be installed in the exhaust system. Exception: An interlock between the cooking appliance and the exhaust hood system shall not be required for appliances that are of the manually operated type and are factory equipped with standing pilot burner ignition systems.

Fiscal Impact Statement:

There will be no cost incurred by the State or any of its political subdivisions.

Statement of Rationale:

There was no scientific or technical basis relied upon in the development of this regulation.