Agency Name: Department of Health and Environmental Control

Statutory Authority: 001-023-0010 et seq., 44-001-0010 et seq., and 48-001-0010 et seq.

Document Number: 3155

Proposed in State Register Volume and Issue: 31/9

House Committee: Agriculture, Natural Resources and Environmental Affairs Committee

Senate Committee: Medical Affairs Committee

120 Day Review Expiration Date for Automatic Approval: 05/09/2008

Final in State Register Volume and Issue: 32/5

Status: Final

Subject: Water Pollution Control Permits

History: 3155

By	Date	Action Description	Jt. Res. No.	Expiration Date

-	09/28/2007	Proposed Reg Published in SR

-	01/10/2008	Received by Lt. Gov & Speaker		05/09/2008

H	01/15/2008	Referred to Committee

S	01/15/2008	Referred to Committee

S	03/04/2008	Resolution Introduced to Approve	1176

-	05/09/2008	Approved by:  Expiration Date

-	05/23/2008	Effective Date unless otherwise

provided for in the Regulation

- Document No. 3155

DEPARTMENT OF HEALTH AND ENVIRONMENTAL CONTROL

CHAPTER 61

Statutory Authority: 1976 Code Sections 1-23-10 et. seq.,

44-1-10 et. seq., and 48-1-10 et. seq.

- 61-9.  Water Pollution Control Permits

- Synopsis:

The Department has amended Regulation 61-9 for  three purposes, as follows:

(1) These revisions incorporate changes to United States Environmental Protection Agency regulations promulgated in the Federal Register October 14, 2005, at 70 Federal Register 60,134, some of which the Department is not required to adopt.

(2)  These revisions include requirements of 2006 S.C. Act No. 387 related to appeals of permits and orders of the Department.

(3)  These revisions make miscellaneous corrections to various parts of Regulation 61-9.

- Discussion of Revisions:

(1) Changes related to U.S. Environmental Protection Agency amendments to regulations related to pretreatment:

a. At item 122.21(j)(6), revise the regulation to correct a reference based on changes elsewhere in the pretreatment amendments.

b. At item 122.44(j)(1), revise the regulation to align the terminology with terminology for municipal pretreatment programs, related to the Federal amendments.

c. At item 122.62(d)(7), revise a reference, per the Federal regulation amendments.

d. At section 403.3.Definitions, related to pretreatment, delete a reserved item, add two (2) new definitions, and renumber the existing items following those which are added, as follows:

(1) At item (b), add the new term, “Best Management Practices, BMP”, and the definition, per the Federal changes.

(2) At item (c), add the new term, “Control Authority”, and the definition, per the Federal changes.

(3) Revise the numbering of previous items (b) to (p) to read (d) through (q).

(4) Renumber new item (l) as (l)(1) and add new item (2) per Department staff, revising the definition of POTW to include “...a private facility that has been determined to be a regional provider of service identified under the 208 Water Quality Management Plan.”

(5) Revise references in the renumbered item (o)(1).

(6) Move previous item (o)(2) to (o)(3) and add new item (o)(2).  New (o)(2) defines a “Non-significant Categorical Industrial User.”  Revise a reference in item (o)(3).

e. At section 403.5 National Pretreatment Standards: Prohibited Discharges, add items as follows:

(1) Based on staff concerns, new items (c)(2)(i) and (ii) clarifying requirements for limits in pretreatment permits.

(2) New Federal item (c)(4) regarding pretreatment best management practices (BMP).

(3) Based on staff concerns, new items (d)(1) and (2) clarifying requirements for local limits in pretreatment permits.

f. At section 403.6 National Pretreatment Standards: Categorical Standards,

(1) At paragraph (b)(5), correct a reference.

(2) At paragraph (c), revise a reference, based on the Federal changes.

(3) At paragraph (d)(2), correct a word grammatically.

(4) Add new item (d)(5), allowing limits to be stated as mass rather than as guideline concentrations, with Department approval and under specific circumstances.

(5) Add new item (d)(6), allowing limits, for specific categories of industry, to be stated as concentrations rather than as guideline-based mass, with Department approval and under specific circumstances.

(6) Renumber existing item (d)(5) to (7) and revise the item related to mass versus concentration conversions, or the reverse, of limits, based on Federal requirements.

(7) Renumber existing item (d)(6) to (8) and revise the item related to mass versus concentration conversions, or the reverse, of limits, based on Federal requirements.

(8) Revise paragraphs (e) and (f), each by removing a reference.

g. At section 403.7 Removal Credits,

(1) Revise paragraph (h) introductory language and (h)(2)(i) for grammar and,

(2) Delete and reserve item (h)(2)(ii), related to combined sewer overflows, as there are no combined sewers and none are allowed by regulation in South Carolina.

(3) Delete item (h)(2)(iii), per the Federal regulation.

h. At section 403.8,

(1) Revise item (f)(1)(iii), deleting items (A) through (E) and adding new items (A), allowing use of general control mechanisms, and (B), based on Federal changes;

(2) Revise a reference in item (f)(1)(v), based on the Federal changes;

(3) Revise item (f)(1)(vi)(B) for clarity;

(4) Revise item (f)(2)(v), related to monitoring by POTW of Industrial Users;

(5) Renumber existing item (f)(2)(vi) to (vii);

(6) Add new item (f)(2)(vi) related to plans of Significant Industrial Users related to slug discharges;

(7) Renumber existing item (f)(2)(vii), related to public participation requirements, to (viii) and revise the introductory language and items (A), (B), (C), (F), and (H);

(8) In item (f)(3), add a reference;

(9) In item (f)(4), clarify that a POTW choosing not to develop local limits for a particular Industrial User is subject to concurrence by the Department;

(10) In item (f)(6), revise references.

(i) In section 403.12,

(1) Remove and reserve paragraph (a);

(2) Revise a reference in item (b)(4)(ii), based on Federal changes;

(3) Revise item (b)(5)(ii) concerning best management practices;

(4) Remove item (b)(5)(iii), defining sampling (see new 403.12(g)(3)), per the Federal revisions;

(5) Redesignate items (b)(5)(iv) through (b)(5)(viii), respectively, as items (b)(5)(iii) through (b)(5)(vii);

(6) Revise paragraph (b)(6) for grammar;

(7) Revise item (b)(7)(ii) to conform with earlier Federal language;

(8) Revise paragraph (e)(1) as to Non-significant Categorical Users and BMP, based on the Federal changes;

(9) Delete existing paragraph (e)(2), based on Federal changes;

(10) Add a new paragraph (e)(2), allowing a POTW, with Department approval, to waive sampling by an industrial user, under stated restrictions, based on Federal changes;

(11) Re-designate paragraph (e)(3) as paragraphs (e)(4);

(12) Add new paragraph (e)(3), allowing POTW, with Department approval, to reduce reporting by Industrial Users, with stated restrictions;

(13) Revise item (g)(1), related to Non-significant Categorical Users, based on the Federal changes;

(14) Revise item (g)(2), related to re-sampling after violations, based on the Federal changes;

(15) Per Federal requirements, revise references in item (g)(3) and add requirements related to sampling;

(16) Re-designate items (g)(4) and (g)(5) as items (g)(5) and (g)(6);

(17) Add new item (g)(4), relating to historical sampling, per Federal requirements;

(18) Revise newly designated item (g)(6) for a reference and for sampling requirements, based on the Federal changes;

(19) Revise paragraph (h) as to BMP, based on the Federal changes;

(20) Revise item (i)(1) as to POTW reporting their Industrial Users, based on the Federal changes;

(21) Revise paragraph (j) as to reporting by Industrial Users, based on the Federal changes;

(22) Revise item (l)(1)(ii) as to signatures on reports, based on the Federal changes;

(23) Revise item (l)(2) for grammar;

(24) Revise paragraph (m) to clarify reference to the Department;

(25) Revise item (o)(1) introductory text, based on Federal requirements;

(26) Revise paragraph (o)(2) related to BMP, based on Federal requirements and including staff revisions;

(27) Add paragraph (q) related to certification by Non-significant Categorical Industrial Users, per Federal requirements.

(j) Revise item 403.13(m)(2) related to procedures for appeals to the United States Environmental Protection Agency on Fundamentally-different Factors variances.

(k) In section 403.15,

(1) Revise paragraph (a) related to pollutants in an Industrial User’s intake water, based on Federal requirements, and

(2) Revise paragraph (b)(1), based on Federal requirements, related to pollutants in an Industrial User’s intake water, by adding new item (i) and redesignating existing language as item (ii).

(l) In Part 403, Appendix C, revise the list of Federal pretreatment categories to include new categories and revise category names.

(m) At Part 403, Appendix G, revise note 1 to Table I, Regulated Pollutants in Part 503 Eligible for a Removal Credit to refer to carbon dioxide as well as total hydrocarbons, per Federal requirements, as well as to clarify reference to South Carolina regulations;

(2)  Changes necessary to update Regulation 61-9 to comply with changes in state law pursuant to 2006 S.C. Act No 387 related to appeals of permits and orders:

(a) Revise the reference in 124.5(b) related to the section of this regulation describing appeals procedures.

(b) Delete paragraph 124.15(c), related to appeals.

(c) Remove the reserved description of section 124.19 and add requirements related to appeals, including those appropriate matters previously stated in 124.15(c) and new requirements for South Carolina Act 387 (2006).

(3)  Miscellaneous changes to address clerical corrections, renumbering, relocation, or revision of the existing regulation to reflect the changes resulting from the appropriate revised requirements, as follows:

(a) Revise the definition of “Draft Permit” in paragraph 122.2(b) related to procedures related to appeals stated in Part 124.

(b) Delete the language in paragraph 124.56(e) related to U.S. Environmental Protection Agency regulation, 40 CFR 501 State Sludge Management Program Regulations, and reserve the paragraph.

(c) Revise a reference in 403.7(a)(3)(iv)(C) based on changes in South Carolina solid waste regulations.

(d) Revise references in 503.4(a), (b), and(c) and 504.4(a), (b), and (c) based on changes in South Carolina solid waste regulations.

(e) Revise the document named in 503.12(m) and 504.12(m) to read as follows: “... fertilizer recommendations (such as “Nutrient Management for South Carolina”, Cooperative Extension Service, Clemson University, EC 476).”  This is a revised edition of the Clemson manual.

(f) In 503.50 and 504.50, state a specific date for the initial requirement to develop an odor minimization plan, based on the promulgation of the regulation.

(g) Revise 505.45(i)(9) to correct a reference.

(h) Revise the following items to entail only clerical corrections:

122.21(k)

122.26(a)(1)(ii)

122.26(b)(14), introductory language

122.26(e)(7)(i)(B)

122.32(a)(2)

122.32(f)

122.62(d)(5)

122.64(a)(4)(ii)

403.3(c)(2)

403, Appendix G, table II

503.32 Table 4

504.22(a)(2)

505.41(p)(5)

Instructions:  Amend R.61-9 pursuant to each individual instruction provided with the text below:

Note: Where a regulation item number is followed by * * *, the asterisks represent existing language which is to remain unchanged.

Text:

At 122.2(b) Definitions, revise the definition, in alphabetical order, of “Draft Permit” to read as follows:

“Draft Permit” means a document prepared by the staff of the Department, in accordance with R.61-9.124.6, prior to public notice of an application for a permit by a discharger.  This document indicates the Department’s tentative decision to issue or deny, modify, revoke and reissue, terminate, or reissue a permit to discharge.  It contains proposed effluent standards and limitations, proposed compliance schedules and other proposed conditions or restrictions deemed necessary by the Department for a discharge.  A notice of intent to terminate a permit, and a notice of intent to deny a permit, as discussed in R.61-9.124.5, are types of draft permits.  A denial of a request for modification, revocation and reissuance, or termination, as discussed in R.61-9.124.5, is not a draft permit.  A “proposed permit” is not a draft permit.

Revise item 122.21(j)(6)(ii) to read as follows:

(ii) POTW with one or more SIU shall provide the following information for each SIU, as defined at R.61-9.403.3(o), that discharges to the POTW:

Revise the introductory language of 122.21(k) as follows, items (k)(1) and following remaining unchanged:

(k)	Application requirements for new sources and new discharges.

New manufacturing, commercial, mining, and silvicultural dischargers applying for NPDES permits (except for new discharges of facilities subject to the requirements of paragraph (h) of this section or new discharges of storm water associated with industrial activity which are subject to the requirements of section 122.26(c)(1) and this section (except as provided by section 122.26(c)(1)(ii)) shall provide the following information to the Department, using application forms provided by the Department.

Revise 122.26(a)(1)(ii) to read as follows:

(ii) A discharge associated with industrial activity (see section 122.26(a)(4));

Revise the introductory language of 122.26(b)(14) to read as follows, items (14)(i) and following  remaining the same:

(14) “Storm water discharge associated with industrial activity” means the discharge from any conveyance that is used for collecting and conveying storm water and that is directly related to manufacturing, processing or raw materials storage areas at an industrial plant. The term does not include discharges from facilities or activities excluded from the NPDES program under this regulation. For the categories of industries identified in this section, the term includes, but is not limited to, storm water discharges from industrial plant yards; immediate access roads and rail lines used or traveled by carriers of raw materials, manufactured products, waste material, or by-products used or created by the facility; material handling sites; refuse sites; sites used for the application or disposal of process waste waters (as defined at 40 CFR Part 401); sites used for the storage and maintenance of material handling equipment; sites used for residual treatment, storage, or disposal; shipping and receiving areas; manufacturing buildings; storage areas (including tank farms) for raw materials and intermediate and final products; and areas where industrial activity has taken place in the past and significant materials remain and are exposed to storm water. For the purposes of this paragraph, material handling activities include storage, loading and unloading, transportation, or conveyance of any raw material, intermediate product, final product, by-product or waste product. The term excludes areas located on plant lands separate from the plant’s industrial activities, such as office buildings and accompanying parking lots as long as the drainage from the excluded areas is not mixed with storm water drained from the above described areas. Industrial facilities (including industrial facilities that are federally, State, or municipally owned or operated that meet the description of the facilities listed in paragraphs (b)(14)(i) through (xi) of this section) include those facilities designated under the provisions of paragraph (a)(1)(v) of this section. The following categories of facilities are considered to be engaging in “industrial activity” for purposes of paragraph (b)(14):

Revise 122.26(e)(7)(i)(B) to read as follows:

(B) For any municipality with a population of less than 250,000 which submits a timely Part I group application under paragraph (e)(2)(i)(B) of this section, the Department shall issue or deny permits for storm water discharges associated with an industrial activity no later than May 17, 1994, or, for any such municipality which fails to complete a Part II group permit application by May 17, 1993, one year after receipt of a complete permit application;

Revise item 122.32(a)(2) to read as follows:

(2) You are designated by the Department, including where the designation is pursuant to 40 CFR 123.35(b)(3) or (b)(4) or is based upon a petition under section 122.26(f).

Revise the introductory language of item 122.32(f) to read as follows, items (f)(1) and following remaining the same:

(f) Process for designating small MS4 to require storm water NPDES permitting.  The Department will designate small MS4s according to the following criteria as a determination that a storm water discharge results in or has the potential to result in exceedances of water quality standards, including impairment of designated uses, or other significant water quality impacts, including habitat and biological impacts.

Revise item 122.44(j)(1) to read as follows:

(1) Identify, in terms of character and volume of pollutants, any Significant Industrial Users discharging into the POTW subject to Pretreatment Standards under section 307(b) of CWA and R.61-9.403.

Revise item 122.62(d)(5) to read as follows:

(5) When the permittee has filed a request for a variance under CWA section 301(c), 301(k), or 316(a) or for “fundamentally different factors” within the time specified in section 122.21.

Revise item 122.62(d)(7) to read as follows:

(7) Reopener. When required by the “reopener” conditions in a permit, which are established in the permit under section  122.44(b) (for CWA toxic effluent limitations and standards for sewage sludge use or disposal, see also section  122.44(c)) or R.61-9. 403.18(e) (Pretreatment program).

Revise item 122.64(a)(4)(ii) to read as follows:

(ii) Cessation of substantially all manufacturing operations, which are a basis for effluent limits or which contribute to a discharge, for a period of 180 days or longer.

Revise item 124.5(b) to read as follows:

(b) See section 124.19 for appeals procedure.

Delete item 124.15(c).

Revise section 124.19, presently reserved, to read as follows:

124.19 Appeal of NPDES and State Permits.

a.  A Department decision involving issuance, denial, renewal, modification, suspension, or revocation of an NPDES, Land Application, or State permit may be appealed by an affected person with standing pursuant to applicable law, including S.C. Code Title 1, Chapter 23 and Title 44, Chapter 1.

b.  Any person to whom an order, related to an NPDES, Land Application, or State permit, is issued may appeal it pursuant to applicable law, including S.C. Code Title 1, Chapter 23 and Title 44, Chapter 1.

Delete the language of item 124.56(e) and reserve the item, to read as follows:

(e) [Reserved.]

Under section  403.3  Definitions, delete existing item (e) [Reserved], renumber existing items (f) to (p) to (g) through (q), respectively, renumber existing items (b) to (d) to (d) through (f), respectively, add new items (b) and (c), and revise items (l) and (o), renumbered as stated above, to read as follows:

(b)  The term Best Management Practices or BMP means schedules of activities, prohibitions of practices, maintenance procedures, and other management practices to implement the prohibitions listed in Sections  403.5(a)(1) and (b). BMP also include treatment requirements, operating procedures, and practices to control plant site runoff, spillage or leaks, sludge or waste disposal, or drainage from raw materials storage.

(c)  The term Control Authority refers to:

(1) The POTW, if the POTW's Pretreatment Program submission has been approved in accordance with the requirements of Section  403.11; or

(2) The Department, if the submission has not been approved.

(d)	The term “Approved POTW Pretreatment Program” or “Program” or “POTW Pretreatment Program” means a program administered by a POTW that meets the criteria established in this regulation (sections 403.8 and 403.9) and which has been approved by the Regional Administrator or the Department in accordance with section 403.11 of this regulation.

(e)	The term “Indirect Discharge” or “Discharge” means the introduction of pollutants into a POTW from any non-domestic source regulated under section 307(b), (c) or (d) of the CWA or Section 48-1-90 of the Pollution Control Act (PCA).

(f)	The term “Industrial User” or “User” means a source of Indirect Discharge.

(g)	The term “interference” means a Discharge which, alone or in conjunction with a discharge or discharges from other sources, both:

(1)	Inhibits or disrupts the POTW, its treatment processes or operations, or its sludge processes, use or disposal; and

(2)	Therefore is a cause of a violation of any requirement of the POTW’s NPDES permit (including an increase in the magnitude or duration of a violation) or of the prevention of sewage sludge use or disposal in compliance with the following statutory provisions and regulations or permits issued thereunder (or more stringent State or local regulations):  Section 405 of the Clean Water Act, the Solid Waste Disposal Act (SWDA) (including Title II, more commonly referred to as the Resource Conservation and Recovery Act (RCRA), and including State regulations contained in any State sludge management plan prepared pursuant to Subtitle D of the SWDA), the Clean Air Act, the Toxic Substances Control Act, and the Marine Protection, Research and Sanctuaries Act, and the South Carolina Pollution Control Act.

(h)	The term “National Pretreatment Standard,” “Pretreatment Standard,” or “Standard” means any regulation containing pollutant discharge limits promulgated by the EPA in accordance with section 307(b) and (c) of CWA, which applies to Industrial Users.  This term includes prohibitive discharge limits established pursuant to section 403.5.

(i)(1)	The term “New Source” means any building, structure, facility or installation from which there is or may be a discharge of pollutants, the construction of which commenced after the publication of proposed Pretreatment Standards under section 307(c) of CWA which will be applicable to such source if such Standards are thereafter promulgated in accordance with that section, provided that:

(i)	The building, structure, facility or installation is constructed at a site at which no other source is located; or

(ii)	The building, structure, facility or installation totally replaces the process or production equipment that causes the discharge of pollutants at an existing source; or

(iii)	The production or wastewater generating processes of the building, structure, facility or installation are substantially independent of an existing source at the same site.  In determining whether these are substantially independent, factors such as the extent to which the new facility is integrated with the existing plant, and the extent to which the new facility is engaged in the same general type of activity as the existing source should be considered.

(2)	Construction on a site at which an existing source is located results in a modification rather than a new source if the construction does not create a new building, structure, facility or installation meeting the criteria of subsections (1)(ii), or (1)(iii) of this section but otherwise alters, replaces, or adds to existing process or production equipment.

(3)	Construction of a new source as defined under this paragraph has commenced if the owner or operator has:

(i)	Begun, or caused to begin as part of a continuous on-site construction program:

(A)	Any placement, assembly, or installation of facilities or equipment; or

(B)	Significant site preparation work including clearing, excavation, or removal of existing buildings, structures, or facilities which is necessary for the placement, assembly, or installation of new source facilities or equipment; or

(ii)	Entered into a binding contractual obligation for the purchase of facilities or equipment which are intended to be used in its operation within a reasonable time.  Options to purchase or contracts which can be terminated or modified without substantial loss, and contracts for feasibility, engineering, and design studies do not constitute a contractual obligation under this paragraph.

(j)	The terms “NPDES Permit” or “Permit” means a permit including a Land Application permit issued to a POTW pursuant to section 402 of CWA or Section 48-1-100 of the Pollution Control Act  (See R.61-9.122 or R.61-9.505).

(k)	The term “Pass Through” means a Discharge which exits the POTW into waters of the State or of the United States in quantities or concentrations which, alone or in conjunction with a discharge or discharges from other sources, is a cause of a violation of any requirement of the POTW’s NPDES permit (including an increase in the magnitude or duration of a violation).

(l) (1) The term “POTW Treatment Plant” means that portion of the POTW which is designed to provide treatment (including recycling and reclamation) of municipal sewage and industrial waste.

(2) For purposes of Part 403, the term “POTW” shall mean publicly owned treatment works or a private facility that has been determined to be a regional provider of service identified under the 208 Water Quality Management Plan.

(m)	The term “Pretreatment” means the reduction of the amount of pollutants, the elimination of pollutants, or the alteration of the nature of pollutant properties in wastewater prior to or in lieu of discharging or otherwise introducing such pollutants into a POTW.  The reduction or alteration may be obtained by physical, chemical or biological processes, process changes or by other means, except as prohibited by section 403.6(e).  Appropriate pretreatment technology includes control equipment, such as equalization tanks or facilities, for protection against surges or slug loadings that might interfere with or otherwise be incompatible with the POTW.  However, where wastewater from a regulated process is mixed in an equalization facility with unregulated wastewater or with wastewater from another regulated process, the effluent from the equalization facility must meet an adjusted pretreatment limit calculated in accordance with section 403.6(f).

(n)	 The term “Pretreatment Requirements” means any substantive or procedural requirement related to Pretreatment, other than a National Pretreatment Standard, imposed on an Industrial User.

(o) Significant Industrial User.

(1) Except as provided in subsections (2) and (3) of this section, the term Significant Industrial User means:

(i) All Industrial Users subject to Categorical Pretreatment Standards under section 403.6 and 40 CFR chapter I, subchapter N and

(ii) Any other Industrial User that: discharges an average of 25,000 gallons per day or more of process wastewater to the POTW (excluding sanitary, noncontact cooling, and boiler blowdown wastewater); contributes a process wastestream which makes up 5 percent or more of the average dry weather hydraulic or organic capacity of the POTW Treatment plant; or is designated as such by the Control Authority on the basis that the Industrial User has a reasonable potential for adversely affecting the POTW's operation or for violating any Pretreatment Standard or requirement (in accordance with section 403.8(f)(6)).

(2) The Control Authority may determine that an Industrial User subject to categorical Pretreatment Standards under Section  403.6 and 40 CFR chapter I, subchapter N is a Non-Significant Categorical Industrial User rather than a Significant Industrial User on a finding that the Industrial User never discharges more than 100 gallons per day (gpd) of total categorical wastewater (excluding sanitary, non-contact cooling and boiler blowdown wastewater, unless specifically included in the Pretreatment Standard) and the following conditions are met:

(i) The Industrial User, prior to the Control Authority's finding, has consistently complied with all applicable categorical Pretreatment Standards and Requirements;

(ii) The Industrial User annually submits the certification statement required in Section  403.12(q) together with any additional information necessary to support the certification statement; and

(iii) The Industrial User never discharges any untreated, concentrated wastewater.

(3) Upon a finding that an Industrial User meeting the criteria in paragraph (o)(1)(ii) of this section has no reasonable potential for adversely affecting the POTW's operation or for violating any Pretreatment Standards or requirement, the Control Authority may at any time, on its own initiative or in response to a petition received from an Industrial User or POTW, and in accordance with section 403.8(f)(6), determine that such Industrial User is not a Significant Industrial User.

(p)	The term “Submission” means a request by a POTW for approval of a Pretreatment Program to the Department.

(q)	The term “Water Management Division Director” means one of the Directors of the Water Management Divisions within the Regional offices of the Environmental Protection Agency or this person’s delegated representative.

Revise item 403.5(c)(2) to read as follows:

(2)	All other POTW shall, in cases where pollutants contributed by User(s) result in Interference or Pass Through, and such violation is likely to recur, develop and enforce specific effluent limits for Industrial User(s), and all other users, as appropriate, which, together with appropriate changes in the POTW Treatment Plant’s facilities or operation, are necessary to ensure renewed and continued compliance with the POTW’s NPDES permit or sludge use or disposal practices.

(i) This evaluation must reflect the POTW’s reasonable potential analysis utilized for all pollutants completed by the Department as part of the NPDES process (R61-9.122). The POTW will utilize the Department’s analysis in the determination of appropriate pretreatment requirements.

(ii) This analysis must utilize the current Water Quality Standards (R.61-68).

Add new item 403.5(c)(4) to read as follows:

(4) POTW may develop Best Management Practices (BMP) to implement paragraphs (c)(1) and (c)(2) of this section. Such BMP shall be considered local limits and Pretreatment Standards for the purposes of this part and section 307(d) of the Act.

Add new items 403.5(d)(1) and (2), after introductory paragraph (d), to read as follows:

(1)	The POTW shall utilize the EPA Guidance Manual (EPA 833-R-04-002) for the development of Local Limits.

(2)	Appropriate removal rates shall be based on wastewater plant site-specific influent and effluent data unless otherwise approved by the Department.

Revise item 403.6(b)(5) to read as follows:

(5)	Requests for hearing and/or legal decision.  Within 30 days following the date of receipt of notice of the final determination as provided for by subsection (b)(4)(iv) of this section, the Requester may submit a petition to reconsider or contest the decision to the Regional Administrator who shall act on such petition expeditiously and state the reasons for his or her determination in writing.

Revise item 403.6(c) to read as follows:

(c) Deadline for compliance with categorical standards. Compliance by existing sources with categorical Pretreatment Standards shall be within 3 years of the date the Standard is effective unless a shorter compliance time is specified in the appropriate subpart of 40 CFR chapter I, subchapter N. Direct dischargers with NPDES Permits modified or reissued to provide a variance pursuant to section 301(i)(2) of the Act shall be required to meet compliance dates set in any applicable categorical Pretreatment Standard. Existing sources which become Industrial Users subsequent to promulgation of an applicable categorical Pretreatment Standard shall be considered existing Industrial Users except where such sources meet the definition of a New Source as defined in Section  403.3(i). New Sources shall install and have in operating condition, and shall ``start-up'' all pollution control equipment required to meet applicable Pretreatment Standards before beginning to Discharge. Within the shortest feasible time (not to exceed 90 days), New Sources must meet all applicable Pretreatment Standards.

Revise item 403.6(d)(2) to read as follows:

(2) When the limits in a categorical Pretreatment Standard are expressed only in terms of mass of pollutant per unit of production, the Control Authority may convert the limits to equivalent limitations expressed either as mass of pollutant discharged per day or effluent concentration for purposes of calculating effluent limitations applicable to individual Industrial Users.

Renumber existing items 403.6(d)(5), (6), and (7) to (7), (8),  and (9), respectively, add new items (5) and (6), and revise renumbered items (7) and (8), to read as follows:

(5) When the limits in a categorical Pretreatment Standard are expressed only in terms of pollutant concentrations, an Industrial User may request that the Control Authority convert the limits to equivalent mass limits. The determination to convert concentration limits to mass limits is within the discretion of the Control Authority and with prior approval of the Department. The Control Authority may establish equivalent mass limits after Department review and approval only if the Industrial User meets all the following conditions in paragraph (d)(5)(i)(A) through (d)(5)(i)(E) of this section.

(i) To be eligible for equivalent mass limits, the Industrial User must:

(A) Employ, or demonstrate that it will employ, water conservation methods and technologies that substantially reduce water use during the term of its control mechanism;

(B) Currently use control and treatment technologies adequate to achieve compliance with the applicable categorical Pretreatment Standard, and not have used dilution as a substitute for treatment;

(C) Provide sufficient information to establish the facility's actual average daily flow rate for all wastestreams, based on data from a continuous effluent flow monitoring device, as well as the facility's long-term average production rate. Both the actual average daily flow rate and long-term average production rate must be representative of current operating conditions;

(D) Not have daily flow rates, production levels, or pollutant levels that vary so significantly that equivalent mass limits are not appropriate to control the Discharge; and

(E) Have consistently complied with all applicable categorical Pretreatment Standards during the period, at least three years, prior to the Industrial User's request for equivalent mass limits.

(ii) An Industrial User subject to equivalent mass limits must:

(A) Maintain and effectively operate control and treatment technologies adequate to achieve compliance with the equivalent mass limits;

(B) Continue to record the facility's flow rates through the use of a continuous effluent flow monitoring device. The devices shall be installed, calibrated, and maintained to ensure that the accuracy of the measurements is consistent with the accepted capability of that type of device.  Devices selected shall be capable of measuring flows with a maximum deviation of not greater than 10 percent from the true discharge rates throughout the range of expected discharge volumes;

(C) Continue to record the facility's production rates and notify the Control Authority whenever production rates are expected to vary by more than 20 percent from its baseline production rates determined in paragraph (d)(5)(i)(C) of this section. Upon notification of a revised production rate, the Control Authority must reassess the equivalent mass limit and revise the limit as necessary to reflect changed conditions at the facility; and

(D) Continue to employ the same or comparable water conservation methods and technologies as those implemented pursuant to paragraph (d)(5)(i)(A) of this section so long as it discharges under an equivalent mass limit.

(iii) A Control Authority which chooses to establish equivalent mass limits:

(A) Must calculate the equivalent mass limit by multiplying the actual average daily flow rate of the regulated process(es) of the Industrial User by the concentration-based daily maximum and monthly average Standard for the applicable categorical Pretreatment Standard and the appropriate unit conversion factor;

(B) Upon notification of a revised production rate, must reassess, with prior Department approval, the equivalent mass limit and recalculate the limit as necessary to reflect changed conditions at the facility; and

(C) May retain the same equivalent mass limit in subsequent control mechanism terms, with prior Department approval, if the Industrial User's actual average daily flow rate was reduced solely as a result of the implementation of water conservation methods and technologies, and the actual average daily flow rates used in the original calculation of the equivalent mass limit were not based on the use of dilution as a substitute for treatment pursuant to paragraph (e) of this section. The Industrial User must also be in compliance with section  403.17 (regarding the prohibition of bypass).

(iv) The Control Authority may not express limits in terms of mass for pollutants such as pH, temperature, radiation, or other pollutants which cannot appropriately be expressed as mass.

(6) The Control Authority may, with prior Department approval, convert the mass limits of the categorical Pretreatment Standards at 40 CFR parts 414, 419, and 455 to concentration limits for purposes of calculating limitations applicable to individual Industrial Users under the following conditions. When converting such limits to concentration limits, the Control Authority must use the concentrations listed in the applicable subparts of 40 CFR parts 414, 419, and 455 and document that dilution is not being substituted for treatment as prohibited by paragraph (e) of this section.

(7) Equivalent limitations calculated in accordance with paragraphs (d)(3), (d)(4), (d)(5), and (d)(6) of this section are deemed Pretreatment Standards for the purposes of section 307(d) of the CWA and this regulation. The Control Authority must document how the equivalent limits were derived and make this information publicly available. Once incorporated into its control mechanism, the Industrial User must comply with the equivalent limitations in lieu of the promulgated categorical standards from which the equivalent limitations were derived.

(8) Many categorical pretreatment standards specify one limit for calculating maximum daily discharge limitations and a second limit for calculating maximum monthly average, or 4-day average, limitations. Where such standards are being applied, the same production or flow figure shall be used in calculating both the average and the maximum equivalent limitation.

(9) Any Industrial User operating under a control mechanism incorporating equivalent mass or concentration limits calculated from a production based standard shall notify the Control Authority within two (2) business days after the User has a reasonable basis to know that the production level will significantly change within the next calendar month. Any User not notifying the Control Authority of such anticipated change will be required to meet the mass or concentration limits in its control mechanism that were based on the original estimate of the long term average production rate.

Revise item 403.6(e) and introductory language of item (f) to read as follows, items (f)(1) and following remaining unchanged:

(e) Dilution prohibited as substitute for treatment. Except where expressly authorized to do so by an applicable Pretreatment Standard or Requirement, no Industrial User shall ever increase the use of process water, or in any other way attempt to dilute a Discharge as a partial or complete substitute for adequate treatment to achieve compliance with a Pretreatment Standard or Requirement. The Control Authority may impose mass limitations on Industrial Users which are using dilution to meet applicable Pretreatment Standards or Requirements, or in other cases where the imposition of mass limitations is appropriate.

(f) Combined wastestream formula. Where process effluent is mixed prior to treatment with wastewaters other than those generated by the regulated process, fixed alternative discharge limits may be derived by the Control Authority or by the Industrial User with the written concurrence of the Control Authority.  These alternative limits shall be applied to the mixed effluent.  When deriving alternative categorical limits, the Control Authority or Industrial User shall calculate both an alternative daily maximum value using the daily maximum value(s) specified in the appropriate categorical Pretreatment Standard(s) and an alternative consecutive sampling day average value using the monthly average value(s) specified in the appropriate categorical Pretreatment Standard(s).  The Industrial User shall comply with the alternative daily maximum and monthly average limits fixed by the Control Authority until the Control Authority modifies the limits or approves an Industrial User modification request.  Modification is authorized whenever there is a material or significant change in the values used in the calculation to fix alternative limits for the regulated pollutant.  An Industrial User must immediately report any such material or significant change to the Control Authority.  Where appropriate new alternative categorical limits shall be calculated within 30 days.

Revise item 403.7(a)(3)(iv)(C) to read as follows:

(C)	For any pollutant in sewage sludge when the POTW disposes all of its sewage sludge in a municipal solid waste landfill unit that meets the criteria in 40 CFR Part 258 and R.61-107.

Revise the introductory language of item 403.7(h) to read as follows, items (h)(1) and following to remain the same:

(h) Compensation for Overflow. ``Overflow'' means the intentional or unintentional diversion of flow from the POTW before the POTW Treatment Plant. POTW which at least once annually overflow untreated wastewater to receiving waters may claim Consistent Removal of a pollutant only by complying with either paragraphs (h)(1) or (h)(2) of this section. However, paragraph (h) of this section shall not apply where Industrial User(s) can demonstrate that Overflow does not occur between the Industrial User(s) and the POTW Treatment Plant;

Revise item 403.7(h)(2)(i) to read as follows:

(2)	(i) The Consistent Removal claimed is reduced pursuant to the following equation:

rc  =  rm  x  8760−Z8760

Where:

rm = POTW's Consistent Removal rate for that pollutant as established under paragraphs (a)(1) and (b)(2) of this section

rc = removal corrected by the Overflow factor

Z = hours per year that overflow occurred between the Industrial User(s) and the POTW Treatment Plant, the hours either to be shown in the POTW's current NPDES permit application or the hours, as demonstrated by verifiable techniques, that a particular Industrial User's Discharge overflows between the Industrial User and the POTW Treatment Plant; and

Remove the language from item 403.7(h)(2)(ii) and reserve the item, as follows:

(ii) [Reserved.]

Remove the language from item 403.7(h)(2)(iii) and reserve the item, as follows:

(iii) [Reserved.]

Revise the introductory language of item 403.8(f)(1)(iii), delete items (iii)(A) through (E), and add new items (iii)(A) and (B), to read as follows:

(iii) Control through Permit, order, or similar means, the contribution to the POTW by each Industrial User to ensure compliance with applicable Pretreatment Standards and Requirements. In the case of Industrial Users identified as significant under Section 403.3, this control shall be achieved through individual permits or equivalent individual control mechanisms issued to each such User except as follows.

(A)	(1) At the discretion of the POTW and with prior Department approval, this control may include use of general control mechanisms if the following conditions are met. All of the facilities to be covered must:

(i) Involve the same or substantially similar types of operations;

(ii) Discharge the same types of wastes;

(iii) Require the same effluent limitations;

(iv) Require the same or similar monitoring; and

(v) In the opinion of the POTW, and with prior Department approval, are more appropriately controlled under a general control mechanism than under individual control mechanisms.

(2) To be covered by the general control mechanism, the Significant Industrial User must file a written request for coverage that identifies its contact information, production processes, the types of wastes generated, the location for monitoring all wastes covered by the general control mechanism, any requests in accordance with section  403.12(e)(2) for a monitoring waiver for a pollutant neither present nor expected to be present in the Discharge, and any other information the POTW deems appropriate. A monitoring waiver for a pollutant neither present nor expected to be present in the Discharge is not effective in the general control mechanism until after the POTW has provided written notice to the Significant Industrial User that, with prior Department approval, such a waiver request has been granted in accordance with section  403.12(e)(2). The POTW must retain a copy of the general control mechanism, documentation to support the POTW's determination that a specific Significant Industrial User meets the criteria in paragraphs (f)(1)(iii)(A)(1) through (5) of this section, and a copy of the User's written request for coverage for 3 years after the expiration of the general control mechanism. A POTW may not control a Significant Industrial User through a general control mechanism where the facility is subject to production-based, categorical Pretreatment Standards or categorical Pretreatment Standards expressed as mass of pollutant discharged per day or for Industrial Users whose limits are based on the Combined Wastestream Formula or Net/Gross calculations (sections  403.6(f) and 403.15).

(B) Both individual and general control mechanisms must be enforceable and contain, at a minimum, the following conditions:

(1) Statement of duration (in no case more than five years);

(2) Statement of non-transferability without, at a minimum, prior notification to the POTW and provision of a copy of the existing control mechanism to the new owner or operator;

(3) Effluent limits, including Best Management Practices, based on applicable general Pretreatment Standards in S.C. R.61-9, part 403, categorical Pretreatment Standards, local limits, and State and local law;

(4) Self-monitoring, sampling, reporting, notification and recordkeeping requirements, including an identification of the pollutants to be monitored (including the process for seeking a waiver for a pollutant neither present nor expected to be present in the Discharge in accordance with section  403.12(e)(2), or a specific waived pollutant in the case of an individual control mechanism), sampling location, sampling frequency, and sample type, based on the applicable general Pretreatment Standards in part 403 of this chapter, categorical Pretreatment Standards, local limits, and State and local law;

(5) Statement of applicable civil and criminal penalties for violation of Pretreatment Standards and requirements, and any applicable compliance schedule. Such schedules may not extend the compliance date beyond applicable federal deadlines;

(6) Requirements to control Slug Discharges, if determined by the POTW to be necessary.

Revise item 403.8(f)(1)(v) to read as follows:

(v) Carry out all inspection, surveillance and monitoring procedures necessary to determine, independent of information supplied by Industrial Users, compliance or noncompliance with applicable Pretreatment Standards and Requirements by Industrial Users.  Representatives of the POTW shall be authorized to enter any premises of any Industrial User in which a Discharge source or treatment system is located or in which records are required to be kept under section  403.12(o) to assure compliance with Pretreatment Standards. Such authority shall be at least as extensive as the authority provided under section 308 of CWA;

Revise item 403.8(f)(1)(vi)(B) to read as follows:

(B) Pretreatment requirements which will be enforced through the remedies set forth in paragraph (f)(1)(vi)(A) of this section, will include but not be limited to, the duty to allow or carry out inspections, entry, or monitoring activities; any rules, regulations, or orders issued by the POTW; any requirements set forth in control mechanisms issued by the POTW; or any reporting requirements imposed by the POTW or these regulations in this part.  The POTW shall have authority and procedures (after informal notice to the discharger) immediately and effectively to halt or prevent any Discharge of pollutants to the POTW which reasonably appears to present an imminent endangerment to the health or welfare of persons.  The POTW shall also have authority and procedures (which shall include notice to the affected Industrial Users and an opportunity to respond) to halt or prevent any Discharge to the POTW which presents or may present an endangerment to the environment or which threatens to interfere with the operation of the POTW.  The Department shall have authority to seek judicial relief and may also use administrative penalty authority when the POTW has sought a monetary penalty which the Department believes to be insufficient.

Revise introductory language of  item 403.8(f)(2)(v), delete items (v)(A) through (D), and add new items (v)(A) through (C), to read as follows:

(v) Randomly sample and analyze the effluent from Industrial Users and conduct surveillance activities in order to identify, independent of information supplied by Industrial Users, occasional and continuing noncompliance with Pretreatment Standards. Inspect and sample the effluent from each Significant Industrial User at least once a year, except as otherwise specified below:

(A) Where the POTW has authorized the Industrial User subject to a categorical Pretreatment Standard to forego sampling of a pollutant regulated by a categorical Pretreatment Standard in accordance with section  403.12(e)(3), the POTW must sample for the waived pollutant(s) at least once during the term of the Categorical Industrial User's control mechanism. In the event that the POTW subsequently determines that a waived pollutant is present or is expected to be present in the Industrial User's wastewater based on changes that occur in the User's operations, the POTW must immediately begin at least annual effluent monitoring of the User's Discharge and inspection.

(B) Where the POTW has determined that an Industrial User meets the criteria for classification as a Non-Significant Categorical Industrial User, the POTW must evaluate, at least once per year, whether an Industrial User continues to meet the criteria in section  403.3(o)(2).

(C) In the case of Industrial Users subject to reduced reporting requirements under section  403.12(e)(3), the POTW must randomly sample and analyze the effluent from Industrial Users and conduct inspections at least once every two years. If the Industrial User no longer meets the conditions for reduced reporting in section  403.12(e)(3), the POTW must immediately begin sampling and inspecting the Industrial User at least once a year.

Renumber items 403.8(f)(2)(vi) and (vii) to (vii) and (viii), respectively, add new item (vi), and revise the introductory language of  renumbered item (viii) and the language of items (viii)(A), (B), (C), (F), and (H), to read as follows:

(vi) Evaluate whether each such Significant Industrial User needs a plan or other action to control Slug Discharges. For Industrial Users identified as significant prior to November 14, 2005, this evaluation must have been conducted at least once by October 14, 2006; additional Significant Industrial Users must be evaluated within 1 year of being designated a Significant Industrial User. For purposes of this subsection, a Slug Discharge is any discharge of a non-routine, episodic nature, including but not limited to an accidental spill or a non-customary batch discharge, which has a reasonable potential to cause interference or pass through, or in any other way violate the POTW's regulations, local limits or permit conditions. The results of such activities shall be available to the Department upon request. Significant Industrial Users are required to notify the POTW immediately of any changes at its facility affecting potential for a Slug Discharge. If the POTW decides that a slug control plan is needed, the plan shall contain, at a minimum, the following elements:

(A) Description of discharge practices, including non-routine batch discharges;

(B) Description of stored chemicals;

(C) Procedures for immediately notifying the POTW of Slug Discharges, including any discharge that would violate a prohibition under section 403.5(b) with procedures for follow-up written notification within five days;

(D) If necessary, procedures to prevent adverse impact from accidental spills, including inspection and maintenance of storage areas, handling and transfer of materials, loading and unloading operations, control of plant site run-off, worker training, building of containment structures or equipment, measures for containing toxic organic pollutants (including solvents), and/or measures and equipment for emergency response;

(vii) Investigate instances of noncompliance with Pretreatment Standards and Requirements, as indicated in the reports and notices required under section 403.12, or indicated by analysis, inspection, and surveillance activities described in paragraph (f)(2)(v) of this section.  Sample-taking and analysis and the collection of other information shall be performed with sufficient care to produce evidence admissible in enforcement proceedings or in judicial actions; and

(viii) Comply with the public participation requirements of 40 CFR part 25 in the enforcement of National Pretreatment Standards. These procedures shall include provision for at least annual public notification in a newspaper(s) of general circulation that provides meaningful public notice within the jurisdiction(s) served by the POTW of Industrial Users which, at any time during the previous 12 months, were in significant noncompliance with applicable Pretreatment requirements. For the purposes of this provision, a Significant Industrial User (or any Industrial User which violates paragraphs (f)(2)(viii)(C), (D), or (H) of this section) is in significant noncompliance if its violation meets one or more of the following criteria:

(A) Chronic violations of wastewater Discharge limits, defined here as those in which  sixty-six percent or more of all of the measurements taken for the same pollutant parameter during a six-month period exceed (by any magnitude) a numeric Pretreatment Standard or Requirement, including instantaneous limits, as defined by 403.3;

(B) Technical Review Criteria (TRC) violations, defined here as those in which thirty-three percent or more of all of the measurements taken for the same pollutant parameter during a six-month period equal or exceed the product of the numeric Pretreatment Standard or Requirement including instantaneous limits, as defined by 403.3 multiplied by the applicable TRC (TRC = 1.4 for BOD, TSS, fats, oil, and grease, and 1.2 for all other pollutants except pH);

(C) Any other violation of a Pretreatment Standard or Requirement as defined by 403.3 (daily maximum, long-term average, instantaneous limit, or narrative Standard) that the POTW determines has caused, alone or in combination with other Discharges, interference or pass-through (including endangering the health of POTW personnel or the general public);

(D)	Any discharge of a pollutant that has caused imminent endangerment to human health, welfare or to the environment or has resulted in the POTW’s exercise of its emergency authority under paragraph (f)(1)(vi)(B) of this section to halt or prevent such a discharge;

(E)	Failure to meet, within 90 days after the schedule date, a compliance schedule milestone contained in a local control mechanism or enforcement order for starting construction, completing construction, or attaining final compliance;

(F) Failure to provide, within 45 days after the due date, required reports such as baseline monitoring reports, 90-day compliance reports, periodic self-monitoring reports, and reports on compliance with compliance schedules;

(G)	Failure to accurately report noncompliance;

(H) Any other violation or group of violations, which may include a violation of Best Management Practices, which the POTW determines will adversely affect the operation or implementation of the local Pretreatment program.

Revise items 403.8(f)(3) and (4) to read as follows:

(3)  Funding.  The POTW shall have sufficient resources and qualified personnel to carry out the authorities and procedures described in paragraphs (f)(1) and (2) of this section.  In some limited circumstances, funding and personnel may be delayed where (i) the POTW has adequate legal authority and procedures to carry out the Pretreatment Program requirements described in this section, and (ii) a limited aspect of the Program does not need to be implemented immediately. (See 403.9(b).)

(4)	Local limits.  The POTW shall develop local limits as required in section 403.5(c)(1) or demonstrate to the satisfaction of the Department that they are not necessary.

Revise item 403.8(f)(6) to read as follows:

(6) The POTW shall prepare and maintain a list of its Industrial Users meeting the criteria in section  403.3(o)(1). The list shall identify the criteria in section  403.3(o)(1) applicable to each Industrial User and, where applicable, shall also indicate whether the POTW has made a determination pursuant to Section  403.3(o)(3) that such Industrial User should not be considered a Significant Industrial User. The initial list shall be submitted to the Department pursuant to section  403.9 or as a non-substantial modification pursuant to section  403.18(d). Modifications to the list shall be submitted to the Department pursuant to section  403.12(i)(1).

Delete the language of  item 403.12(a) and reserve the item, as follows:

(a) [Reserved.]

Revise item 403.12(b)(4)(ii) to read as follows:

(ii) Other streams as necessary to allow use of the combined waste stream formula of section  403.6(f). (See paragraph (b)(5)(iv) of this section.)

Revise item 403.12(b)(5)(ii) to read as follows:

(ii) In addition, the User shall submit the results of sampling and analysis identifying the nature and concentration (or mass, where required by the Standard or Control Authority) of regulated pollutants in the Discharge from each regulated process. Both daily maximum and average concentration (or mass, where required) shall be reported. The sample shall be representative of daily operations. In cases where the Standard requires compliance with a Best Management Practice or pollution prevention alternative, the User shall submit documentation as required by the Control Authority or the applicable Standards to determine compliance with the Standard;

Delete item 403.12(b)(5)(iii) and renumber items (iv) through (viii) to (iii) through (vii), respectively.

Revise item 403.12(b)(6) to read as follow:

(6) Certification. A statement, reviewed by an authorized representative of the Industrial User (as defined in paragraph (l) of this section) and certified by a qualified professional, indicating whether Pretreatment Standards are being met on a consistent basis, and, if not, whether additional operation and maintenance (O and M) and/or additional Pretreatment is required for the Industrial User to meet the Pretreatment Standards and Requirements; and

Revise item 403.12(b)(7)(ii) to read as follow:

(ii)  If the categorical Pretreatment Standard is modified by a removal allowance (section 403.7),  the combined waste stream formula (section 403.6(f)), and/or a Fundamentally Different Factors variance (section 403.13) at the time the User submits the report required by paragraph (b) of this section, the information required by paragraphs (b)(6) and (7) of this section shall be submitted by the Industrial User to the Control Authority within 60 days after the modified limit is approved.

Revise item 403.12(e)(1) to read as follows:

(1) Any Industrial User subject to a categorical Pretreatment Standard (except a Non-Significant Categorical User as defined in section 403.3(o)(2)), after the compliance date of such Pretreatment Standard, or, in the case of a New Source, after commencement of the discharge into the POTW, shall submit to the Control Authority during the months of June and December, unless required more frequently in the Pretreatment Standard or by the Control Authority or the Department, a report indicating the nature and concentration of pollutants in the effluent which are limited by such categorical Pretreatment Standards. In addition, this report shall include a record of measured or estimated average and maximum daily flows for the reporting period for the Discharge reported in paragraph (b)(4) of this section, except that the Control Authority may require more detailed reporting of flows. In cases where the Pretreatment Standard requires compliance with a Best Management Practice (or pollution prevention alternative), the User shall submit documentation required by the Control Authority or the Pretreatment Standard necessary to determine the compliance status of the User. At the discretion of the Control Authority and in consideration of such factors as local high or low flow rates, holidays, budget cycles, etc., the Control Authority may modify the months during which the above reports are to be submitted.

Delete existing item 403.12(e)(2) and add new item (e)(2) to read as follows:

(2) The Control Authority may with prior Department approval authorize the Industrial User subject to a categorical Pretreatment Standard to forego sampling of a pollutant regulated by a categorical Pretreatment Standard if the Industrial User has demonstrated through sampling and other technical factors that the pollutant is neither present nor expected to be present in the Discharge, or is present only at background levels from intake water and without any increase in the pollutant due to activities of the Industrial User. This authorization is subject to the following conditions:

(i) The Control Authority may with prior Department approval authorize a waiver where a pollutant is determined to be present solely due to sanitary wastewater discharged from the facility provided that the sanitary wastewater is not regulated by an applicable categorical Standard and otherwise includes no process wastewater and the POTW does not have an effluent NPDES limit for this pollutant.

(ii) The monitoring waiver is valid only for the duration of the effective period of the Permit or other equivalent individual control mechanism, but in no case longer than 5 years. The User must submit a new request for the waiver before the waiver can with prior Department approval be granted for each subsequent control mechanism.

(iii) In making a demonstration that a pollutant is not present, the Industrial User must provide data from at least one sampling of the facility's process wastewater prior to any treatment present at the facility that is representative of all wastewater from all processes.  The request for a monitoring waiver must be signed in accordance with paragraph (l) of this section and include the certification statement in section  403.6(b)(2)(ii). Non-detectable sample results may only be used as a demonstration that a pollutant is not present if the EPA approved method from 40 CFR part 136 with the lowest minimum detection level for that pollutant was used in the analysis, or at the lowest Practical Quantitation Limit specified by the Department, whichever is lower.

(iv) Any grant of the monitoring waiver by the Control Authority must be included as a condition in the User's control mechanism. The reasons supporting the waiver and any information submitted by the User in its request for the waiver must be maintained by the Control Authority for 3 years after expiration of the waiver.

(v) Upon approval of the monitoring waiver and revision of the User's control mechanism by the Control Authority, the Industrial User must certify on each report with the statement below, that there has been no increase in the pollutant in its wastestream due to activities of the Industrial User:

Based on my inquiry of the person or persons directly responsible for managing compliance with the Pretreatment Standard for 40 CFR ________ [specify applicable National Pretreatment Standard part(s)], I certify that, to the best of my knowledge and belief, there has been no increase in the level of  __________ [list pollutant(s)] in the waste streams due to the activities at the facility since filing of the last periodic report under S.C. R.61-9.403.12(e)(1).

(vi) In the event that a waived pollutant is found to be present or is expected to be present based on changes that occur in the User's operations, the User must immediately: Comply with the monitoring requirements of paragraph (e)(1) of this section or other more frequent monitoring requirements imposed by the Control Authority; and notify the Control Authority and the Department.

(vii) This provision does not supersede certification processes and requirements established in categorical Pretreatment Standards, except as otherwise specified in the categorical Pretreatment Standard.

Renumber existing item 403.12(e)(3) to (e)(4) and add new item (e)(3) to read as follows:

(3) The Control Authority may reduce the requirement in paragraph (e)(1) of this section to a requirement to report no less frequently than once a year, unless required more frequently in the Pretreatment Standard or by the Department, where the Industrial User meets all of the following conditions:

(i) The Industrial User's total categorical wastewater flow does not exceed any of the following:

(A) 0.01 percent of the design dry weather hydraulic capacity of the POTW, or 5,000 gallons per day, whichever is smaller, as measured by a continuous effluent flow monitoring device unless the Industrial User discharges in batches;

(B) 0.01 percent of the design dry weather organic treatment capacity of the POTW; and

(C) 0.01 percent of the maximum allowable headworks loading for any pollutant regulated by the applicable categorical Pretreatment Standard for which approved local limits were developed by a POTW in accordance with section 403.5(c) and paragraph (d) of this section;

(ii) The Industrial User has not been in significant noncompliance, as defined in section  403.8(f)(2)(vii), for any time in the past two years;

(iii) The Industrial User does not have daily flow rates, production levels, or pollutant levels that vary so significantly that decreasing the reporting requirement for this Industrial User would result in data that are not representative of conditions occurring during the reporting period pursuant to paragraph (g)(3) of this section;

(iv) The Industrial User must notify the Control Authority immediately of any changes at its facility causing it to no longer meet conditions of paragraphs (e)(3)(i) or (ii) of this section. Upon notification, the Industrial User must immediately begin complying with the minimum reporting in paragraph (e)(1) of this section; and

(v) The Control Authority must retain documentation to support the Control Authority's determination that a specific Industrial User qualifies for reduced reporting requirements under paragraph (e)(3) of this section for a period of 3 years after the expiration of the term of the control mechanism.

(4) For Industrial Users subject to equivalent mass or concentration limits established by the Control Authority in accordance with the procedures in section 403.6(d), the report required by paragraph (e)(1) shall contain a reasonable measure of the User's long term production rate. For all other Industrial Users subject to categorical Pretreatment Standards expressed only in terms of allowable pollutant discharge per unit of production (or other measure of operation), the report required by paragraph (e)(1) shall include the User's actual average production rate for the reporting period.

Revise existing items 403.12(g)(1), (2), and (3) to read as follows:

(1) Except in the case of Non-Significant Categorical Users, the reports required in paragraphs (b), (d), (e), and (h) of this section shall contain the results of sampling and analysis of the Discharge, including the flow and the nature and concentration, or production and mass where requested by the Control Authority, of pollutants contained therein which are limited by the applicable Pretreatment Standards. This sampling and analysis may be performed by the Control Authority in lieu of the Industrial User. Where the POTW performs the required sampling and analysis in lieu of the Industrial User, the User will not be required to submit the compliance certification required under section 403.12(b)(6) and section 403.12(d). In addition, where the POTW itself collects all the information required for the report, including flow data, the Industrial User will not be required to submit the report.

(2) If sampling performed by an Industrial User indicates a violation, the User shall notify the Control Authority within 24 hours of becoming aware of the violation. The User shall also repeat the sampling and analysis and submit the results of the repeat analysis to the Control Authority within 30 days after becoming aware of the violation. Where the Control Authority has performed the sampling and analysis in lieu of the Industrial User, the Control Authority must perform the repeat sampling and analysis unless it notifies the User of the violation and requires the User to perform the repeat analysis. Resampling is not required if:

(i) The Control Authority performs sampling at the Industrial User at a frequency of at least once per month; or

(ii) The Control Authority performs sampling at the User between the time when the initial sampling was conducted and the time when the User or the Control Authority receives the results of this sampling.

(3) The reports required in paragraphs (b), (d), (e), and (h) of this section must be based upon data obtained through appropriate sampling and analysis performed during the period covered by the report, which data are representative of conditions occurring during the reporting period. The Control Authority shall require that frequency of monitoring necessary to assess and assure compliance by Industrial Users with applicable Pretreatment Standards and Requirements. Grab samples must be used for pH, cyanide, total phenols, oil and grease, sulfide, and volatile organic compounds. For all other pollutants, 24-hour composite samples must be obtained through flow-proportional composite sampling techniques, unless time-proportional composite sampling or grab sampling is authorized by the Control Authority. Where time-proportional composite sampling or grab sampling is authorized by the Control Authority with approval by the Department, the samples must be representative of the discharge and the decision to allow the alternative sampling must be documented in the Industrial User file for that facility or facilities. Using protocols (including appropriate preservation) specified in 40 CFR part 136 and appropriate EPA guidance, multiple grab samples collected during a 24-hour period may be composited prior to the analysis as follows: For cyanide, total phenols, and sulfides, the samples may be composited in the laboratory or in the field; for volatile organics and oil & grease, the samples may be composited in the laboratory. Composite samples for other parameters unaffected by the compositing procedures as documented in approved EPA methodologies may be authorized by the Control Authority, as appropriate.

Renumber existing items 403.12(g)(4) and (5) to (5) and (6), add new item (4), and revise renumbered item (6) to read as follows:

(4) For sampling required in support of baseline monitoring and 90-day compliance reports required in paragraphs (b) and (d) of this section, a minimum of four (4) grab samples must be used for pH, cyanide, total phenols, oil and grease, sulfide and volatile organic compounds for facilities for which historical sampling data do not exist; for facilities for which historical sampling data are available, the Control Authority may with approval by the Department authorize a lower minimum. For the reports required by paragraphs (e) and (h) of this section, the Control Authority shall require the number of grab samples necessary to assess and assure compliance by Industrial Users with Applicable Pretreatment Standards and Requirements.

(5) All analyses shall be performed in accordance with procedures established by the Administrator pursuant to section 304(h) of CWA contained in 40 CFR Part 136 and amendments thereto or with any other test procedures approved by the Administrator. (See, section 136.4 and section 136.5) Sampling shall be performed in accordance with the techniques approved by the Department. Where 40 CFR Part 136 does not include sampling or analytical techniques for the pollutants in question, or where the Administrator determines that the Part 136 sampling and analytical techniques are inappropriate for the pollutant in question, sampling and analyses shall be performed using validated analytical methods or any other sampling and analytical procedures, including procedures suggested by the POTW or other parties, approved by the administrator.

(6) If an Industrial User subject to the reporting requirement in paragraph (e) or (h) of this section monitors any regulated pollutant at the appropriate sampling location more frequently than required by the Control Authority, using the procedures prescribed in paragraph (g)(5) of this section, the results of this monitoring shall be included in the report.

Revise item 403.12(h) to read as follows:

(h) Reporting requirements for Industrial Users not subject to categorical Pretreatment Standards. The Control Authority must require appropriate reporting from those Industrial Users with Discharges that are not subject to categorical Pretreatment Standards.  Significant Non-categorical Industrial Users must submit to the Control Authority at least once every six months (on dates specified by the Control Authority) a description of the nature, concentration, and flow of the pollutants required to be reported by the Control Authority. In cases where a local limit requires compliance with a Best Management Practice or pollution prevention alternative, the User must submit documentation required by the Control Authority to determine the compliance status of the User. These reports must be based on sampling and analysis performed in the period covered by the report, and in accordance with the techniques described in part 136 and amendments thereto.  This sampling and analysis may be performed by the Control Authority in lieu of the significant non-categorical Industrial User.

Revise item 403.12(i)(1) to read as follows:

(1) An updated list of the POTW's Industrial Users, including their names and addresses, or a list of deletions and additions keyed to a previously submitted list. The POTW shall provide a brief explanation of each deletion. This list shall identify which Industrial Users are subject to categorical Pretreatment Standards and specify which Standards are applicable to each Industrial User. The list shall indicate which Industrial Users are subject to local standards that are more stringent than the categorical Pretreatment Standards. The POTW shall also list the Industrial Users that are subject only to local Requirements. The list must also identify Industrial Users subject to categorical Pretreatment Standards that are subject to reduced reporting requirements under paragraph (e)(3), and identify which Industrial Users are Non-Significant Categorical Industrial Users;

Revise item 403.12(j) to read as follows:

(j) Notification of changed Discharge. All Industrial Users shall promptly notify the Control Authority (and the POTW if the POTW is not the Control Authority) in advance of any substantial change in the volume or character of pollutants in their discharge, including the listed or characteristic hazardous wastes for which the Industrial User has submitted initial notification under section 403.12(p).

Revise items 403.12(l)(1)(ii) and (2) to read as follows:

(ii) The manager of one or more manufacturing, production, or operating facilities, provided the manager is authorized to make management decisions which govern the operation of the regulated facility including having the explicit or implicit duty of making major capital investment recommendations, and initiate and direct other comprehensive measures to assure long-term environmental compliance with environmental laws and regulations; can ensure that the necessary systems are established or actions taken to gather complete and accurate information for control mechanism requirements; and where authority to sign documents has been assigned or delegated to the manager in accordance with corporate procedures.

(2) By a general partner or proprietor if the Industrial User submitting the reports required by paragraphs (b), (d), and (e) of this section is a partnership or sole proprietorship, respectively.

Revise item 403.12(m) to read as follows:

(m) Signatory requirements for POTW reports. Reports submitted to the Department by the POTW in accordance with paragraph (i) of this section must be signed by a principal executive officer, ranking elected official, or other duly authorized employee. The duly authorized employee must be an individual or position having responsibility for the overall operation of the facility or the Pretreatment Program. This authorization must be made in writing by the principal executive officer or ranking elected official, and submitted to the Department prior to or together with the report being submitted.

Revise the introductory language of item 403.12(o)(1) to read as follows, items (1)(i) and following remaining the same:

(1) Any Industrial User and POTW subject to the reporting requirements established in this section shall maintain records of all information resulting from any monitoring activities required by this section, including documentation associated with Best Management Practices. Such records shall include for all samples:

Revise item 403.12(o)(2) to read as follows:

(2) Any Industrial User or POTW subject to the reporting requirements established in this section (including documentation associated with Best Management Practices) shall be required to retain for a minimum of 3 years any records of monitoring activities and results (whether or not such monitoring activities are required by this section) and shall make such records available for inspection and copying by the Department and the Regional Administrator (and POTW in the case of an Industrial User). This period of retention shall be extended during the course of any unresolved litigation regarding the discharge of pollutants by the Industrial User or the operation of the POTW Pretreatment Program or when requested by the Department or the Regional Administrator.

Add new item 403.12(q) to read as follows:

(q) Annual certification by Non-Significant Categorical Industrial Users. A facility determined to be a Non-Significant Categorical Industrial User pursuant to section 403.3(o)(2) must annually submit the following certification statement, signed in accordance with the signatory requirements in paragraph (l) of this section. This certification must accompany any alternative report required by the Control Authority:

Based on my inquiry of the person or persons directly responsible for managing compliance with the categorical Pretreatment Standards under 40 CFR ____ I certify that, to the best of my knowledge and belief that during the period from __________, _____ to ____________, _____ [month, day, year]:

(a) The facility described as __________ [facility name] met the definition of a non-significant categorical Industrial User as described in section 403.3(o)(2); (b) the facility complied with all applicable Pretreatment Standards and requirements during this reporting period; and (c) the facility never discharged more than a total of 100 gallons of categorical wastewater on any given day during this reporting period. This compliance certification is based upon the following information:

_______________________________________________

_______________________________________________

Revise item 403.13(m)(2) to read as follows:

(2)	If the Regional Administrator declines to hold a hearing and the Regional Administrator affirms the findings of the Administrator’s delegate, the requester may submit a petition for a hearing to the Environmental Appeals Board (which is described in section 1.25 of 40 CFR Part 1.25) within 30 days of the Regional Administrator’s decision.

Revise the title of section 403.15 to read as follows:

403.15 Net/gross calculation.

Revise item 403.15(a) to read as follows:

(a) Application. Categorical Pretreatment Standards may be adjusted to reflect the presence of pollutants in the Industrial User’s intake water in accordance with this section. Any Industrial User wishing to obtain credit for intake pollutants must make application to the Control Authority. Upon request of the Industrial User, the applicable Standard will be calculated on a ``net’’ basis (i.e., adjusted to reflect credit for pollutants in the intake water) if the requirements of paragraph (b) of this section are met.

Revise item 403.15(b)(1) to read as follows:

(1) Either:

(i) The applicable categorical Pretreatment Standards contained in 40 CFR subchapter N specifically provide that they shall be applied on a net basis or

(ii) The Industrial User demonstrates that the control system it proposes or uses to meet applicable categorical Pretreatment Standards would, if properly installed and operated, meet the Standards in the absence of pollutants in the intake waters.

Revise item 403.15(b)(3) to read as follows:

(3) Credit shall be granted only to the extent necessary to meet the applicable categorical Pretreatment Standard(s), up to a maximum value equal to the influent value. Additional monitoring may be necessary to determine eligibility for credits and compliance with Standard(s) adjusted under this section.

Delete item 403.15(c).

In Appendix C of Part 403, revise the list of categories to read as follows:

Aluminum Forming

Asbestos Manufacturing

Battery Manufacturing

Builder’s Paper

Carbon Black

Cement Manufacturing

Centralized Waste Treatment

Coil Coating

Copper Forming

Dairy Products Processing

Electrical and Electronic Components

Electroplating

Feedlots

Ferroalloy Manufacturing

Fertilizer Manufacturing

Fruits and Vegetables Processing Manufacturing

Glass Manufacturing

Grain Mills

Ink Formulating

Inorganic Chemicals

Iron and Steel Manufacturing

Leather Tanning and Finishing

Meat Processing

Metal Finishing

Metal Molding and Casting

Nonferrous Metals Forming and Metal Powders

Nonferrous Metals Manufacturing

Oil and Gas Extraction

Organic Chemicals, Plastics, and Synthetic Fibers

Paint Formulating

Paving and Roofing (Tars and Asphalt)

Pesticides

Petroleum Refining

Pharmaceuticals

Phosphate Manufacturing

Plastics Molding and Forming

Porcelain Enameling

Pulp, Paper, and Paperboard

Rubber Manufacturing

Seafood Processing

Soaps and Detergents Manufacturing

Steam Electric

Sugar Processing

Textile Mills

Timber Products Processing

Transportation Equipment Cleaning

Waste Combustors

In Appendix G of Part 403, in “Table I. Regulated Pollutants in Part 503 Eligible for a Removal Credit”, revise the introductory language of Note 1 to read as follows, the listing of organic pollutants remaining the same:

1. The following organic pollutants are eligible for a removal credit if the requirements for total hydrocarbons (or carbon monoxide) in subpart E in R.61-9.503 and other requirements in R.61-62, Air Pollution Control Regulations and Standards, are met when sewage sludge is fired in a sewage sludge incinerator:

In Appendix G of Part 403, in Table II, revise the heading of the table to read as follows, the listing of pollutants and data remaining the same:

| Pollutants | Use or Disposal Practice |  |  |  |
| --- | --- | --- | --- | --- |
|  | LA | Surface Disposal |  | I |
|  |  | Unlined1 | Lined2 |  |

Revise items 503.4(a), (b), and (c) to read as follows:

(a)	Disposal of sewage sludge in a municipal solid waste landfill unit, as defined in 40 CFR 258.2 and R.61-107, that complies with the requirements in 40 CFR Part 258 and R.61-107 constitutes compliance with section 405(d) of the CWA.  Any person who prepares sewage sludge that is disposed in a municipal solid waste landfill unit shall ensure that the sewage sludge meets the requirements in 40 CFR Part 258 and R.61-107 concerning the quality of materials disposed in a municipal solid waste landfill unit.

(b) 	The disposal of sewage sludge involving the composting or co-composting of the sewage sludge with yard trash, land-clearing debris, or a combination of yard trash and land clearing debris shall comply with the requirements established in R.61-107. The submission and information requirements shall be determined by the Department.

(c)	The disposal of sewage sludge utilizing an innovative and experimental solid waste management technology or process shall comply with the requirements addressed in R.61-107.

Revise item 503.12(m) to read as follows:

(m) The Department may establish permit conditions to require that sludge application remain consistent with the lime and fertilizer requirements for the cover, feed, food, and fiber crops based on published lime and fertilizer recommendations (such as “Nutrient Management for South Carolina”, Cooperative Extension Service, Clemson University, EC 476).

Revise the heading of  Table 4 of section 503.32 to read as follows, the column headings and data to remain the same:

TABLE 4 OF SECTION 503.32 - If the sewage sludge is less than 7% solids

and the temperature of the sewage sludge is 50 degrees Celsius or higher;

and the time period is 30 minutes or longer.

Revise the introductory language of item 503.50(a) to read as follows, items (a)(1) and following remaining the same:

(a)  The permittee shall prepare an odor abatement plan for the sewage sludge treatment sites, any sludge storage or lagoon areas, and land application or surface disposal sites. Permittees that land-apply sludge must prepare the plan within 180 days of the effective date of this regulation (effective date of June 26, 2003). Permittees that have facilities described above that require plans have one (1) year from the June 26, 2003 effective date to prepare the plan.  Odor abatement plans must be submitted for new projects with the submission of permit applications.  The plan must include the following topics:

Revise items 504.4(a), (b), and (c) to read as follows:

(a)	Disposal of industrial sludge in a municipal solid waste landfill unit permitted under R.61-107 constitutes compliance with this regulation.  Any person who prepares industrial sludge that is disposed in a municipal solid waste landfill unit shall ensure that the industrial sludge meets the requirements in R.61-107 concerning the quality of materials disposed in a municipal solid waste landfill unit.  Disposal of industrial sludge in an industrial solid waste landfill unit complying with State Solid Waste regulations and requirements in permits constitutes compliance with this regulation.

(b) 	The disposal of industrial sludge involving the composting or co-composting of the industrial sludge with yard trash, land-clearing debris, or a combination of yard trash and land clearing debris shall comply with the requirements established by the Department in R.61-107.  The submission and information requirements shall be determined by the Department.

(c)	The disposal of industrial sludge utilizing an innovative and experimental solid waste management technology or process shall be permitted under R.61-107.

Revise item 504.12(m) to read as follows:

(m) The Department may establish permit conditions to require that the agronomic rate of sludge application remain consistent with the lime and fertilizer requirements for the cover, feed, food, and fiber crops based on published lime and fertilizer recommendations (such as “Nutrient Management for South Carolina”, Cooperative Extension Service, Clemson University, EC 476).

Revise item 504.22(a)(2) to read as follows:

(2) For any facility, except a landfill or a sludge only monofill, meeting the definition of a land disposal site on the date of this regulation, either sufficient amount of sludge must be removed from the facility in order to change the facility's classification, or a report detailing final closure must be submitted to the Bureau of Water, Department of Health and Environmental Control or an application for permitting under Solid Waste Regulations must be submitted to the Bureau of Land and Waste Management,  Department of Health and Environmental Control. Either the sludge removal must be accomplished within one year after the date of this regulation or the closeout report or permit application must be submitted to the Department within one (1) year after the date of this regulation. If closure is the selected option, the plan must provide a schedule showing how the closure will be accomplished. The land disposal site must be either closed under Regulation 61-82 or permitted by Solid Waste Management Regulations by June 28, 2001.  Facilities will be in compliance with this section if a timely and complete application for closure or permit is made and through no fault of the applicant a closure approval or permit has not been issued.

Revise the introductory language of  item 504.50(a) to read as follows, items (a)(1) and following remaining the same:

(a) The permittee shall prepare an odor abatement plan for the industrial sludge treatment sites, any sludge storage or lagoon areas, and land application or surface disposal sites. Permittees that land-apply sludge must prepare the plan within 180 days of the effective date of this regulation (effective date of June 26, 2003). Permittees that have facilities described above that require plans have one (1) year from the June 26, 2003 effective date to prepare the plan.  Odor abatement plans must be submitted for new projects with the submission of permit applications.  The plan must include the following topics:

Revise item 505.41(p)(5) to read as follows:

(5) Basins, storage ponds, or constructed impoundments (except for systems designed to operate in this manner, e.g., infiltration basins) which are in use may be required to be monitored with groundwater monitoring wells as approved by the Department. The basin, storage pond or constructed impoundment may be considered unlined if the leakage rate is greater than 500 gallons per day per acre, or information available would indicate to the Department that specific hydrological conditions would require groundwater monitoring. The Department may consider the level of treatment, or the type of wastewater (e.g., influent characteristics) in the determination of whether an unlined basin, storage pond or other constructed impoundments must have groundwater monitoring. Storage facilities for reclaimed water (as described in section 505.45) will not require groundwater monitoring unless specifically required by the Department.

Revise item 505.45(i)(9)(i) to read as follows:

(i) Land application systems.  For all POTW and for those other systems including in the influent a significant amount of, or having a significant effect from, domestic sewage, at least as stringent as 200/100 ml monthly average and 400/100 ml daily maximum, or the bacteriological standard from the nearest surface water body as defined in R.61-68 (if this surface water is classified with a more restrictive standard), except where it can be shown that neither storm water nor wastewater will run off the disposal site to a waterway and that the isolation of the disposal site will eliminate exposure of persons to pathogens. A significant amount or effect is related to the effluent having a reasonable potential to violate the above-stated bacteriological requirement.  For all other discharges, the Department may use the previously identified limits, or establish other fecal coliform limitations to reflect the specific discharge and site conditions.  Domestic sewage is defined at R.61-9.503.9.

Fiscal Impact Statement:

There is expected to be no significant cost from these amendments to industrial users, that is, pretreaters, of publicly owned treatment works (POTW).  These amendments allow reduced monitoring, both in the number of parameters monitored and in the frequency of monitoring.  This may only occur in appropriate circumstances, which are defined in the Text of these amendments above and the Statement of Need and Reasonableness below.

Minimal additional costs to POTW and the Department will accrue from the time involved in review of requests by industrial users for reduced monitoring.

Statement of Need and Reasonableness:

This statement of need and reasonableness and rationale has been developed based on staff analysis pursuant to S.C. Code Sections 1-23-110(A)(3)(h) and 115(C)(1) - (3) and (9) - (11):

- DESCRIPTION OF REGULATION: S.C Regulation 61-9, Water Pollution Control Permits.

Purpose:  These amendments of Regulation 61-9 have three purposes:  (1):  Incorporate into South Carolina regulations changes to United States Environmental Protection Agency regulations promulgated in the Federal Register October 14, 2005, at 70 Federal Register 60,134, some of which the Department is not required to adopt.  (2)  Include requirements of 2006 S.C. Act No 387 related to appeals of permits and orders of the Department.   (3)  Make miscellaneous corrections to various parts of Regulation 61-9.

Legal Authority:  This change to state law is authorized by S.C. Code  Title 1, Chapter 23; Title 44, Chapter 1; and Title 48, Chapter 1; the Clean Water Act, 33 U.S.C. 1251, et seq.; and United States Environmental Protection Agency regulations promulgated in the Federal Register (FR) of October 14, 2005.

Plan for Implementation:  These amendments will make changes to and be incorporated into R.61-9 upon approval of the Board of Health and Environmental Control and the General Assembly and publication in the State Register.   The additional effort required by the Department will consist mainly of reviews of requests by industrial users of publicly owned treatment works (POTW) for waivers of monitoring and for less frequent monitoring.  The additional work required by these amendments will total about five employee-weeks of effort during the first year.  As budget constraints appear to prohibit addition of positions, the additional work will be integrated with present operations.

DETERMINATION OF NEED AND REASONABLENESS FOR THE REGULATION AND EXPECTED BENEFIT:

(1) Incorporate into South Carolina regulations changes to United States Environmental Protection Agency regulations promulgated in the Federal Register October 14, 2005, at 70 Federal Register 60,134, some of which the Department is not required to adopt.

The October 14, 2005 Federal Register modified regulations for industries which discharge to publicly owned treatment works (POTW), that is, pretreaters, and for the POTW which receive such waste.  These amendments to the regulation allowed waivers from some monitoring and reduction of some reporting in a manner similar to that for such industries which discharge directly to waters of the State under a National Pollutant discharge Elimination System (NPDES) permit.  The South Carolina amendments which incorporate the Federal changes require S.C. DHEC approval, not required by the Federal regulation, of some the waivers and reductions in monitoring.

The Department met with representatives of pretreaters and  POTW in July 2007, prior to the submittal of the proposed amendments to the regulation to the Board for approval to publish a Notice of Proposed Regulation (NPR) and to hold a Staff Informational Forum.  No written comments pertinent to the issues of the Notices of Drafting were received.  Comments received in the meetings were considered in drafting the amendments.

Subsequent to Board approval to publish the proposed regulations in the State Register, staff contacted the following organizations concerning the notice and solicited input:  S.C. American Waterworks Association; S.C. Chamber of Commerce;  S.C. Coastal Conservation League;  S.C. Environmental Law Project;  S.C. Pretreatment Consortium;  S.C. Sierra Club;  S.C. Water Environment Association;  Southern Environmental Law Center;  and the Strom Thurmond Institute.  No written comments were received from these groups.

The Staff Informational Forum was held on October 31, 2007 with no member of the public attending or submitting comments.  However, additional meetings were held with representatives of pretreaters and POTW, again with no stated objections or written comments.

Comments were received December 12, 2007, from U.S. Environmental Protection Agency, Region 4, concerning the amendments stated in the Notice of Proposed Regulation published in the State Register September 28, 2007.  The comments recommended changes which were largely clerical, and recommendations were discussed during the public hearing conducted by the Department of Health and Environmental Control on December 13. Department staff recommended declining to make some of the changes and accepting the  others, with the concurrence of the Board.

These amendments are needed and reasonable because they reduce unnecessary monitoring and reporting requirements on permittees while still maintaining protection of the environment and public health.

(2) 	Changes necessary to update Regulation 61-9 to comply with changes in state law pursuant to 2006 S.C. Act No 387 related to appeals of permits and orders.

References in the regulation are revised to comply with the South Carolina act.  Provisions were also relocated in the regulation to parallel the location of appeals procedures in the Federal regulation.

This amendment is needed and reasonable because it carries out the requirements of South Carolina Act 387 (2006).

(3)  Miscellaneous changes to address clerical corrections, renumbering, relocation, or revision of the existing regulation to reflect the changes resulting from the appropriate revised requirements, as follows.

These amendments will make the regulation clearer.

UNCERTAINTIES OF ESTIMATES:

While none of the costs can be specifically defined, as stated above the various amendments will either have no cost or will reduce costs to permittees.

The cost to the Department for review of requests for waivers and reduction of monitoring is quite uncertain.  Staff cannot determine how frequently permittees will choose to prepare and submit requests.  However, as monitoring is a significant cost, numerous waiver requests seem likely.

EFFECT ON THE ENVIRONMENT AND PUBLIC HEALTH:

There is not expected to be any effect on the environment or public health from the changes resulting from these amendments.  Rather, the amendments are expected to contribute to improved efficiency in regulating the pretreatment of wastewater discharged by industrial users into publicly owned treatment works (POTW).

DETRIMENTAL EFFECT ON THE ENVIRONMENT AND PUBLIC HEALTH IF THE REGULATION IS NOT IMPLEMENTED:

There is not expected to be any adverse effect on the environment or public health if these amendments are not approved.   However, unnecessary costs for monitoring and reporting would continue to accrue for both industrial users and POTW if these amendments are not approved.

Statement of Rationale:

Those sections of these amendments to South Carolina Regulation 61-9, Water Pollution Control Permits, relating to operational matters are taken directly from the United States Environmental Protection Agency regulation promulgated in the Federal Register October 14, 2005 at 70 FR 60134.  These matters relate to industries which discharge industrial wastewater to publicly owned treatment works (POTW) and to the POTW which receive such wastewater and regulate the industrial discharges.

Certain parts of the Federal regulation are more stringent than the present South Carolina regulation, and all of these are adopted in this amendment.  Adoption of these is required by Federal regulation for South Carolina to maintain a discharge permit (NPDES) program in lieu of United States Environmental Protection Agency permitting.

Others of the Federal changes are less stringent than South Carolina regulation, and South Carolina is not required to adopt these.  However, these changes make complying with pretreatment regulations more efficient, and the Department has chosen to also adopt these changes.  The Department has revised these changes in some cases to allow review by the Department of requests by permittees for reduction of monitoring or reporting.

The remaining changes are administrative and consist of two categories of changes:

1. Addition of references to revised requirements for appeals of permits and orders of the Department.  These revisions were stated in South Carolina Act 387 of 2006; and

2. Clerical corrections to the existing regulation.