Agency Name: Coastal Division - Department of Health and Environmental Control

Statutory Authority: 48-39-10 et seq.

Document Number: 5200

Proposed in State Register Volume and Issue: 47/7

House Committee: Regulations and Administrative Procedures Committee

Senate Committee: Agriculture and Natural Resources Committee

120 Day Review Expiration Date for Automatic Approval 05/08/2024

Final in State Register Volume and Issue: 48/5

Status: Final

Subject: Statement of Policy; Applying for a Permit; General Guidelines for All Critical Areas; and Specific Project Standards for Beaches and the Beach/Dune System

History: 5200

By	Date	Action Description	Jt. Res. No.	Expiration Date

-	07/28/2023	Proposed Reg Published in SR

-	01/09/2024	Received President of the Senate & Speaker		05/08/2024

H	01/09/2024	Referred to Committee

S	01/09/2024	Referred to Committee

S	04/16/2024	Committee Requested Withdrawal

120 Day Period Tolled

-	04/16/2024	Withdrawn and Resubmitted		05/08/2024

-	05/08/2024	Approved by: Expiration Date

-	05/24/2024	Effective Date unless otherwise

provided for in the Regulation

Document No. 5200

DEPARTMENT OF HEALTH AND ENVIRONMENTAL CONTROL

CHAPTER 30

Statutory Authority: 1976 Code Sections 48-39-10 et seq.

30-1. Statement of Policy.

30-2. Applying for a Permit.

30-11. General Guidelines for All Critical Areas.

30-13. Specific Project Standards for Beaches and the Beach/Dune System.

30-15. Activities Allowed Seaward of the Baseline.

Synopsis:

South Carolina’s beachfront policies and jurisdictional authorities are established under the South Carolina Beachfront Management Act (S.C. Code Sections 48-39-250 et seq.). Over the past three decades, these rules have guided where and how areas along the state’s beachfront can be developed. In 2018, Act 173 amended the Beachfront Management Act to replace the state’s 40-year policy of retreat with a policy of beach preservation. However, Coastal Division regulations currently lack specific guidance to effectively implement the new policy. In 2022, the Department of Health and Environmental Control (“Department”) convened the Beach Preservation Stakeholder Workgroup (“Workgroup”), which represented a wide range of backgrounds and constituencies, to provide input and diverse perspectives on beach preservation in South Carolina. The Workgroup’s recommendations and key findings were the primary basis for developing the proposed amendments to R.30-1, R.30-2, R.30-11.D, R.30-13, and R.30-15.H to establish a regulatory definition for beaches and beach preservation, provide for a process and standards to permit pilot projects proposed within the beaches or beach/dune system critical areas, provide for consistency in the application of standards for activities across beaches and beach/dune system critical areas, and to clarify regulations related to emergency orders and the use of sandbags seaward of the baseline. The proposed amendments will provide clarity for the regulated community, guidance for regulatory staff, and allow the Department to more effectively implement the state’s beach preservation policy. The proposed amendments also include corrections for clarity and readability, grammar, punctuation, codification, and other such regulatory text improvements. The Administrative Procedures Act, S.C. Code Section 1-23-120(A), requires General Assembly review of these proposed amendments.

The Department had a Notice of Drafting published in the February 24, 2023, South Carolina State Register.

Changes made at the request of the Senate Agriculture and Natural Resources Committee by letter dated April 16, 2024:

R.30-1.D(5) Add definition for beaches to match statutory definition.

R.30-1.D(7) Add language to beach preservation definition to clarify that beach preservation supports storm protection of public and private property and includes utilization of engineered solutions that support preservation of the beaches and the beach/dune system, as permitted by statute and regulation.

R.30-11.D(1) Add language to include references to statutory and regulatory definitions of beaches and beach/dune system critical areas.

R.30-11.D(2) Add language to clarify that in many instances erosion control devices adjacent to the beach have contributed to the deterioration and loss of the dry sand beach. Add language to reference statutory requirements associated with erosion control structures. Remove language clarifying activities that do not support beach preservation.

R.30-13.H(2)(a) Remove ‘wood-like’ language associated with approved construction materials for small wooden decks.

R.30-13.O(1)(a) Remove ‘wood-like’ language associated with approved construction materials for walkways over dunes.

R.30-13.S(1) Add language to clarify that in many instances erosion control devices adjacent to the beach have contributed to the deterioration and loss of the dry sand beach.

R.30-15.H Add language to allow the use of emergency orders when erosion is beyond twenty feet of a structure, when extenuating circumstances exist.

R.30-15.H(3)(a) Add language to require applicants to provide information to the Department demonstrating sandbags are specifically manufactured to be used in a coastal environment and include factors for consideration. Add language to specify that this requirement applies one year from the effective date of these regulations.

R.30-15.H(3)(b) Add language to clarify that sandbags shall be a maximum size of approximately one cubic yard.

R.30-15.H(3)(d) Add language to specify that the toe of sandbags shall not be buried during installation and to specify that sandbags shall not be buried or covered with sand from non-natural processes, including sand scraping or renourishment.

R.30-15.H(3)(h) Add language to allow sandbags to remain in place for the duration of the emergency order process if they become buried or partially buried through natural processes, provided that the bags have been surveyed and documentation has been provided to the Department.

Section-by-Section Discussion of Amendments:

| Section | Type of Change | Purpose |
| --- | --- | --- |
| R.30-1.C(5)(a)-(c) | Technical Correction | Stylistic amendment to correct tabbed indentation. |
| R.30-1.C(6) | Revision | Amend language to reference the state’s current policy of beach preservation and to clarify reference to the Coastal Zone Management Act. |
| R.30-1.D(1) | Technical Correction | Amend to correct punctuation. |
| R.30-1.D(3) | Technical Correction | Amend to correct punctuation. |
| R.30-1.D(4)(a) | Technical Correction | Amend to correct punctuation. |
| R.30-1.D(4)(b) | Revision, Technical Correction | Amend language to reference the state’s current policy of beach preservation, amend for stylistic clarity, and to correct punctuation and spelling. |
| R.30-1.D(4)(c) | Technical Correction | Amend to correct punctuation. |
| R.30-1.D(5) | Addition | Add new definition 5 for beaches to match statutory definition. |
| R.30-1.D(5) | Technical Correction | Renumber definition 5 after addition of new beaches definition and amend for Code reference and stylistic clarity. |
| R.30-1.D(7) | Addition | Add new definition 7 to clarify beach preservation characteristics. |
| R.30-1.D(6)-(13) | Technical Correction | Renumber definitions 6 through 13 after addition of new beach preservation definition. |
| New R.30-1.D(9) | Technical Correction | Amend to correct spelling. |
| New R.30-1.D(12)(a)-(b) | Technical Correction | Amend for stylistic clarity. |
| New R.30-1.D(13) | Technical Correction | Amend to correct punctuation. |
| R.30-1.D(14) | Deletion | Delete definition of Coastal Zone Management Appellate Panel to conform with statute. |
| R.30-1.D(15)-(554 | Technical Correction | Renumber definitions 15 through 54. |
| New R.30-1.D(16) | Technical Correction | Amend to correct punctuation. |
| New R.30-1.D(18)(a)-(c) | Technical Correction, Revision | Amend for stylistic clarity and to add clarifying article. |
| New R.30-1.D(20)(c)-(e) | Technical Correction | Amend for stylistic clarity. |
| New R.30-1.D(21) | Technical Correction, Revision | Amend for Code reference clarity and to correct punctuation. |
| New R.30-1.D(22) | Technical Correction, Revision | Amend for Code reference and stylistic clarity, and to correct wording. |
| New R.30-1.D(24) | Technical Correction | Amend to correct punctuation. |
| New R.30-1.D(25) | Technical Correction | Amend to correct punctuation. |
| New R.30-1.D(27) | Technical Correction | Amend for stylistic clarity. |
| New R.30-1.D(32) | Technical Correction | Amend for stylistic clarity. |
| New R.30-1.D(34)(c)-(d) | Technical Correction | Amend for stylistic clarity. |
| New R.30-1.D(36) | Technical Correction | Amend to correct punctuation. |
| New R.30-1.D(45) | Technical Correction | Amend for stylistic clarity. |
| New R.30-1.D(46) | Technical Correction | Amend to correct capitalization and punctuation. |
| New R.30-1.D(48) | Technical Correction | Amend for stylistic clarity. |
| New R.30-1.D(50) | Technical Correction | Amend for stylistic clarity. |
| New R.30-1.D(53) | Technical Correction | Amend to correct capitalization. |
| New R.30-1.D(55) | Technical Correction | Amend for stylistic clarity. |
| R.30-2.B(8)(a) and (b) | Reorganization, Technical Correction | Divide subsection into multiple items, amend to correct punctuation, spelling, and capitalization, and amend for Code reference clarity. |
| R.30-2.B(8)(c) | Addition | Add a third item to the subsection to reference additional required information for pilot project permit applications. |
| R.30-11.D | Revision, Technical Correction | Amend language to clarify that activities in beaches critical area are subject to applicable laws and policies and to conform with the title of R.30-11.D, General Guidelines for Beaches and the Beach/Dune System. Amend for Code reference clarity. |
| R.30-11.D(1) | Revision | Amend to include the beaches critical area to conform with the title of R.30-11.D and to reference the state’s current policy of beach preservation. |
| R.30-11.D(2) | Revision | Amend to include statutory reference regarding impacts to the beach from hardened erosion control structures, to reference the state’s current policy of beach preservation, and to reference statutory requirements associated with erosion control structures. |
| R.30-11.D(5) | Revision, Technical Correction | Amend to clarify that beaches critical areas are susceptible to impacts from construction activities. Amend to correct punctuation and for Code reference clarity. |
| R.30-11.D(6) | Revision, Technical Correction | Amend to clarify that destruction of beach or dune vegetation within the beaches critical area is also prohibited. Amend to correct punctuation. |
| R.30-13.B | Revision | Amend language to clarify that the Department applies requirements for new habitable structures consistently within the beaches and beach/dune system critical areas and to conform with the title of R.30-13, Specific Project Standards for Beaches and the Beach/Dune System. |
| R.30-13.B(2) | Technical Correction | Amend for stylistic clarity. |
| R.30-13.B(4) | Technical Correction | Amend for Code reference clarity. |
| R.30-13.B(5) | Revision | Amend to clarify that the Department may authorize habitable structures seaward of the baseline under a special permit. |
| R.30-13.C | Revision | Amend language to clarify that the Department applies requirements for additions to habitable structures consistently within the beaches and beach/dune system critical areas and to conform with the title of R.30-13. |
| R.30-13.C(1) | Technical Correction | Amend for stylistic clarity. |
| R.30-13.C(4) | Revision | Amend language to clarify that the Department applies requirements for additions to habitable structures consistently within the beaches and beach/dune system critical areas and to conform with the title of R.30-13. |
| R.30-13.C(5) | Addition | Add new items to ensure that additions to habitable structures are not constructed on the primary oceanfront sand dune or on active beach and to clarify that the Department may authorize additions to habitable structures seaward of the baseline under a special permit. |
| R.30-13.D | Revision, Technical Correction | Amend language to clarify that the Department applies requirements for repair and renovation of habitable structures consistently within the beaches and beach/dune system critical areas, to ensure that habitable structures are not constructed on active beach, and to conform with the title of R.30-13. Amend to correct punctuation. |
| R.30-13.E | Revision, Technical Correction | Amend language to clarify that the Department applies requirements for replacement or rebuilding of habitable structures consistently within the beaches and beach/dune system critical areas, to conform with the title of R.30-13, and to clarify required notice and documentation to the Department. Amend to correct punctuation. |
| R.30-13.E(4) | Technical Correction | Amend to correct punctuation. |
| R.30-13.E(5) | Revision, Technical Revision | Amend to provide reference to implementing regulations associated with cited statutes and for Code reference clarity. |
| R.30-13.E(7) | Addition | Add new item to ensure that replacement habitable structures are not constructed on the primary oceanfront sand dune or on active beach. |
| R.30-13.F | Revision, Technical Correction | Amend language to clarify that the Department applies requirements for landscaping, earthmoving, and fill for landscaping consistently within the beaches and beach/dune system critical areas and to conform with the title of R.30-13. Amend to correct punctuation. |
| R.30-13.F(1) | Technical Correction | Amend to correct punctuation. |
| R.30-13.F(7) | Revision | Amend to ensure that landscaping, earthmoving, and fill for landscaping activities shall not occur on active beach and add clarifying article. |
| R.30-13.G | Revision | Amend language to clarify that the Department applies requirements for fences, lighting, trash receptables, sidewalks, and signs consistently within the beaches and beach/dune system critical areas and to conform with the title of R.30-13. |
| R.30-13.G(2) | Technical Correction | Amend for stylistic clarity. |
| R.30-13.G(5) | Revision | Amend to add clarifying article. |
| R.30-13.D(6) | Revision | Amend to clarify that lighting within the beaches critical area shall be designed to shield the beach from illumination. |
| R.30-13.H | Revision, Technical Correction | Amend language to clarify that the Department applies requirements for emergency vehicle access ways, small wooden decks, gazebos, and other structures consistently within the beaches and beach/dune system critical areas and to conform with the title of R.30-13. Amend to correct punctuation. |
| R.30-13.H(1)(c)-(e) | Technical Correction, Revision | Amend to correct punctuation, for stylistic clarity, and to add clarifying article. |
| R.30-13.H(2) | Technical Correction | Amend to correct punctuation. |
| R.30-13.H(2)(a) | Revision | Amend criteria to allow Department-approved material to be used for small wooden decks and to conform to statute, and amend language to improve sentence structure. |
| R.30-13.H(2)(b)-(f) | Technical Correction | Amend to correct punctuation. Amend for stylistic clarity, to improve sentence structure, and to add clarifying article. |
| R.30-13.I | Revision | Amend language to clarify that the Department applies requirements for construction and/or repair of drives and parking lots consistently within the beaches and beach/dune system critical areas and to conform with the title of R.30-13. |
| R.30-13.I(7) | Technical Correction, Revision | Amend to correct punctuation and to add clarifying article. |
| R.30-13.I(8) | Revision, Technical Correction | Amend to clarify that no new driveways or parking lots shall be constructed seaward of the baseline without a special permit and to ensure that driveways and parking lots are not constructed on active beach. Amend for Code reference clarity. |
| R.30-13.J | Revision | Amend language to clarify that the Department applies requirements for the installation or repair of underground and overhead water, sewer, gas, electrical, telephone lines, and cable service lines consistently within the beaches and beach/dune system critical areas and to conform with the title of R.30-13. |
| R.30-13.J(1) | Revision | Amend to ensure that service lines are not placed within active beach and provide an exception for subsea cables. |
| R.30-13.J(4) | Revision | Amend to add clarifying article. |
| R.30-13.K | Revision | Amend language to clarify that the Department applies requirements for drainage structures consistently within the beaches and beach/dune system critical areas and to conform with the title of R.30-13. |
| R.30-13.K(1) | Technical Correction | Amend to correct spelling, capitalization, and punctuation. |
| R.30-13.K(4) | Revision | Amend language to clarify that requirements of local drainage plans also apply within the beaches critical areas. |
| R.30-13.K(5)(a) | Revision | Amend to add clarifying article. |
| R.30-13.L | Revision, Technical Correction | Amend language to clarify that the Department applies requirements for sand fences, minor beach renourishment, and dune revegetation consistently within the beaches and beach/dune system critical areas and to conform with the title of R.30-13. Amend to add clarifying article and to correct punctuation. |
| R.30-13.L(1)(h) | Revision | Amend language to clarify that the Department evaluates impacts from sand fencing consistently within the beaches and beach/dune system critical areas and to conform with the title of R.30-13. |
| R.30-13.L(2) | Revision | Amend to correct spelling. |
| R.30-13.L(2)(a) | Revision | Amend language to generally refer to the Department’s recommendations for planting dune vegetation. |
| R.30-13.M | Revision | Amend language to clarify that sand that has drifted out of the beaches and/or beach/dune system critical areas may be returned to them, to clarify that the Department applies requirements for returning sand consistently within the beaches and beach/dune system critical areas, and to conform with the title of R.30-13. |
| R.30-13.M(1) | Revision | Amend to add clarifying article. |
| R.30-13.N(2)(c)-(d) | Technical Correction, Revision | Amend to add clarifying article and correct punctuation. |
| R.30-13.N(3) | Technical Correction | Amend to correct punctuation. |
| R.30-13.N(3)(a) | Revision | Amend language to clarify that the Department applies restrictions for erosion control structures consistently within the beaches and beach/dune system critical areas and to conform with the title of R.30-13. |
| R.30-13.N(3)(c)-(d) | Technical Correction | Amend to correct punctuation and for Code reference clarity. |
| R.30-13.N(3)(e)(i)-(iii) | Technical Correction | Amend for stylistic clarity. |
| R.30-13.O(1), (1)(a)-(c) | Revision, Technical Correction | Amend for code reference clarity. Amend criteria to allow Department-approved material to be used for walkways over dunes and to conform to statute. Amend for stylistic clarity. |
| R.30-13.O(1)(f)-(h) | Technical Correction, Revision | Amend for stylistic clarity, to correct punctuation, to add clarifying article, and to correct wording. |
| R.30-13.O(2)(a) | Technical Correction | Amend to correct spelling. |
| R.30-13.P | Technical Correction, Revision | Amend to correct punctuation, for Code reference clarity, and to correct wording. |
| R.30-13.Q(1) | Technical Correction | Amend to correct punctuation and for Code reference clarity. |
| R.30-13.Q(2)(c) | Revision | Amend language to clarify that the Department applies requirements for lighting associated with golf courses consistently within the beaches and beach/dune system critical areas and to conform with the title of R.30-13. |
| R.30-13.Q(2)(d)(iii) | Technical Correction | Amend to correct punctuation and to add clarifying article. |
| R.30-13.R(2)(b) | Technical Correction | Amend to correct punctuation. |
| R.30-13.S | Addition | Add section to include specific project standards for pilot projects within the beaches and beach/dune system critical areas. |
| R.30-13.S(1) | Addition | Add subsection to clarify the intended purpose of pilot projects, specify that new erosion control structures or devices will not be permitted as part of a pilot project, and reference supporting statutory language. |
| R.30-13.S(2) | Addition | Add subsection to list additional requirements for pilot project permit applications. |
| R.30-13.S(3) | Addition | Add subsection to list the standards which shall apply to permitted pilot projects. |
| R.30-13.S(4) | Addition | Add subsection to specify information the pilot project permittee must submit to the Department in a final report upon completion of the pilot project, and to clarify Departmental considerations of the study findings and the final report. |
| R.30-15.H | Revision, Technical Correction | Add language to allow the use of emergency orders when erosion is beyond twenty feet of a structure, when extenuating circumstances exist. Amend for stylistic clarity. |
| R.30-15.H(1) | Technical Correction | Amend for Code reference clarity. |
| R.30-15.H(2)(b)-(d) | Technical Correction | Amend for stylistic clarity. |
| R.30-15.H(2)(d)(i)-(iii) | Technical Correction | Amend for stylistic clarity. |
| R.30-15.H(3)(a) | Revision | Add language to require applicants to provide information to the Department demonstrating sandbags are specifically manufactured to be used in a coastal environment and include factors for consideration. Add language to specify that this requirement applies one year from the effective date of these regulations. |
| R.30-15.H(3)(b) | Revision | Add language to clarify that sandbags shall be a maximum size of approximately one cubic yard. |
| R.30-15.H(3)(d) | Revision | Add language to specify that the toe of sandbags shall not be buried during installation and to specify that sandbags shall not be buried or covered with sand from non-natural processes, including sand scraping or renourishment. |
| R.30-15.H(3)(e) | Technical Correction | Amend for stylistic clarity. |
| R.30-15.H(3)(h) | Revision | Add language to allow sandbags to remain in place for the duration of the emergency order process if they become buried or partially buried through natural processes, provided that the bags have been surveyed and documentation has been provided to the Department. |
| R.30-15.H(4) | Technical Correction | Amend for Code reference clarity. |
| R.30-15.H(4)(a), (c)-(d), and (f) | Technical Correction | Amend for stylistic clarity. |
| R.30-15.H(5) | Technical Correction | Amend for Code reference clarity. |

Text:

Chapter 30

DEPARTMENT OF HEALTH AND ENVIRONMENTAL CONTROL – COASTAL DIVISION

(Statutory Authority: S.C. Code Section 48-39-10 et seq.)

R.30-1. Statement of Policy.

Amend R.30-1.C(5) and (6) to read:

(5) There are three basic approaches to beachfront management:

(a) armor the beach with hard erosion control devices;

(b) renourish the beach with sand;

(c) retreat from the beach.

(6) The 1977 Coastal Zone Management Act, as amended, rejects construction of new erosion control devices and adopts a state policy of beach preservation, including restoration of the beaches of our state. The Department, as steward of the State’s coastal resources, has the responsibility to implement the policy of beach preservation by designating a baseline and setback line on all oceanfront properties of the State, developing a long-range comprehensive state plan for management of the beach and dune resources, and supporting the efforts of local governments in developing local long-range beach management plans. In addition, the Department shall require property owners to move new construction and reconstruction as far landward as possible, to limit the size of structures within the constraints of the Coastal Zone Management Act, and to seek innovative ways to ameliorate the effects of beach erosion.

Amend R.30-1.D to read:

D. Definitions:

(1) Abandoned Vessels/Structures - Any boat, barge, dock, pier, or other structure/vessel in the critical areas that is no longer functional for its primary, intended purpose and for which repair or salvage activity is not actively being pursued.

(2) Active Beach - the area seaward of the escarpment or the first line of stable natural vegetation, whichever first occurs, measured from the ocean landward.

(3) Administrative Law Judge - a judge appointed pursuant to S.C. Code Ann. Section 1-23-510 (1976) (as amended) who is assigned a particular matter by the Chief Administrative Law Judge, or if no administrative law judge has been assigned for a particular matter, the Chief Administrative Law Judge.

(4) Baselines:

(a) Within a standard erosion zone, the baseline is established at the location of the crest of the primary oceanfront sand dune in that zone. In a standard erosion zone in which the shoreline has been altered naturally or artificially by the construction of erosion control devices, groins, or other man-made alterations, the baselines must be established by the Department using the best scientific and historical data, as where the crest of the primary ocean front sand dune for that zone would be located if the shoreline had not been altered.

(b) Within an unstabilized inlet zone, the baseline must be determined by the Department as the most landward point of erosion at any time during the past forty (40) years, unless the best available scientific and historical data of the inlet and adjacent beaches indicate that the shoreline is unlikely to return to its former position. In collecting and utilizing the best scientific and historical data available for the implementation of the policy of beach preservation, the Department, as part of the State Comprehensive Beach Management Plan provided for in this chapter, among other factors, must consider: historical inlet migration, inlet stability, channel and ebb tidal delta changes, the effects of sediment bypassing on shorelines adjacent to the inlets, and the effects of nearby beach restoration projects on inlet sediment budgets.

(c) Within a stabilized inlet zone, the baseline location must be determined in the same manner as provided for in a standard erosion zone. However, the actual location of the crest of the primary oceanfront sand dune of that erosion zone is the baseline of that zone, not the location if the inlet had remained unstabilized.

(5) Beaches - those lands subject to periodic inundation by tidal and wave action so that no nonlittoral vegetation is established.

(6) Beach/Dune System - all land from the mean high-water mark of the Atlantic Ocean landward to the forty (40)-year setback line described in S.C. Code Section 48-39-280.

(7) Beach Preservation - maintaining the natural processes and functionality and benefits of the beaches and the beach/dune system critical areas that support storm protection of public and private property, habitat, tourism, public access, recreation opportunities, and aesthetics. Beach preservation includes the utilization of engineered solutions, as permitted by the S.C. Code Ann. 48-39-10 et seq. and these regulations, that support these processes, functionality, and benefits.

(8) Best Management Practices - measures to reduce adverse environmental impacts.

(9) Boat - A vessel or watercraft of any type or size specifically designed to be self-propelled, whether by engine, sail, paddle, or other means, which is used to travel from place to place by water.

(10) Boat Storage Structure - any structure associated with a dock that is used for the purpose of storing a boat out of the water and may include, but is not limited to, boatlifts, davits, and any other type of floating vessel platform. A boat storage structure is not a fixed pierhead, walkway, ramp, or gangway.

(11) Boat Yard - a facility where boats are repaired.

(12) Bridge:

(a) Non-vehicular - bridges designed for use by pedestrians, golf carts or other maintenance vehicles, but not cars and trucks; are not docks; and can have a maximum clear width on the deck surface of six (6) feet.

(b) Vehicular - bridges with a clear width on the deck surface of over six (6) feet and designed to support traffic by cars and trucks.

(13) Coastal Island - an area of high ground above the critical area delineation that is separated from other high ground areas by coastal tidelands or waters. An island connected to the mainland or other island only by a causeway is also considered a coastal island. The purpose of this definition is to include all islands except those that are essentially mainland, i.e., those that already have publicly accessible bridges and/or causeways. The following islands shall not be deemed a coastal island subject to this section due to their large size and developed nature: Waites Island in Horry County; Pawleys Island in Georgetown County; Isle of Palms, Sullivans Island, Folly Island, Kiawah Island, Seabrook Island, Edisto Island, Johns Island, James Island, Woodville Island, Slann Island, and Wadmalaw Island in Charleston County; Daniel Island in Berkeley County; Edisto Beach in Colleton County; Harbor Island, Hunting Island, Fripp Island, Hilton Head Island, St. Helena Island, Port Royal Island, Ladies Island, Spring Island, and Parris Island in Beaufort County.

(14) Coastal Waters - the navigable waters of the United States subject to the ebb and flood of the tide and which are saline waters, shoreward to their mean high-water mark.

(15) Coastal Zone - all coastal waters and submerged lands seaward to the State’s jurisdictional limits and all lands and waters in the counties of the State which contain any one or more of the critical areas. These counties are Beaufort, Berkeley, Charleston, Colleton, Dorchester, Horry, Jasper, and Georgetown.

(16) Critical Areas - any of the following: (1) coastal waters, (2) tidelands, (3) beach/dune systems, and (4) beaches.

(17) Department - the South Carolina Department of Health and Environmental Control (also referred to as SCDHEC).

(18) Destroyed Beyond Repair:

(a) Habitable Structures - destroyed beyond repair means more than sixty-six and two-thirds (66 2/3) percent of the replacement value of the habitable structure has been destroyed. See R.30-14(D)(3)(a).

(b) Pools - destroyed beyond repair means more than sixty-six and two-thirds (66 2/3) percent of the replacement value of the pool has been destroyed. See R.30-14(D)(3)(b).

(c) Seawalls and Bulkheads - damage to seawalls and bulkheads must be judged on the percentage of the structure remaining intact at the time of the damage assessment. Erosion control structures or devices must not be repaired or replaced if destroyed:

(i) more than eighty (80) percent above grade through June 30, 1995;

(ii) more than sixty-six and two-thirds (66 2/3) percent above grade from July 1, 1995, through June 30, 2005; or

(iii) more than fifty (50) percent above grade after June 30, 2005. See R.30-14(D)(3)(c).

(d) Revetments - must be judged on the extent of displacement of the stone, effort to return this stone to the pre-storm event configuration of the structure or device, and the ability of the revetment to retain backfill material at the time of the damage assessment. See R.30-14(D)(3)(d).

(19) Division - the Administrative Law Judge Division.

(20) Dock - All docks defined herein refer to structures that provide docking space for ten (10) boats or less.

(a) Boat Storage Dock - a floating structure that a vessel is parked on for purposes of out-of-water storage.

(b) Commercial Dock - a docking facility used for commercial purposes. A commercial dock is not necessarily a marina, a boat yard, or a dry storage facility.

(c) Community Dock - any docking facility that provides access for more than four (4) families, has effective docking space of no more than two hundred fifty (250) linear feet and is not a marina. Effective docking space means adequate length and water depth to dock a twenty (20)-foot boat.

(d) Joint use dock - any private dock intended for the use of two to four (2-4) families.

(e) Private Dock - any facility that provides access for one (1) family, and is not a marina.

(21) Emergency Orders - orders issued in response to an emergency as defined in S.C. Code Section 48-39-10(U), by the Department, or upon written notification to the Department by an appointed official of a county or municipality or of the state acting to protect the public health and safety. With regard to the beach/dune critical area, only the use of sandbags, sand scraping, renourishment, or a combination of them, in accordance with R.30-5 and R.30-15.H, is allowed pursuant to emergency orders.

(22) Emergency Repairs - repairs due to emergencies as defined in S.C. Code Section 48-39-10(U) to an existing bank, dike, fishing pier, or structure other than ocean front erosion control structures or devices which have been erected in accordance with federal and state laws or provided for by general law or acts passed by the General Assembly, if notice is given in writing to the Department within seventy-two (72) hours of the onset of the needed repair.

(23) Erosion Control Structures and Beach Renourishment:

(a) Seawall - a special type of retaining wall that is specifically designed to withstand wave forces.

(b) Bulkhead - a retaining wall designed to retain fill material, but not to withstand wave forces on an exposed shoreline.

(c) Revetment - a sloping structure built along an escarpment or in front of a bulkhead to protect the shoreline or bulkhead from erosion.

(d) Beach Renourishment - the artificial establishment and periodic renourishment of a beach with sand that is compatible with the beach in such a way as to create a dry sand beach at all stages of the tide and/or provide some level of storm protection.

(24) Feasible (feasibility) - As used within these rules and regulations (e.g., "unless no feasible alternative exists"), feasibility is determined by the Department with respect to individual project proposals. Feasibility in each case is based on the best available information, including, but not limited to, technical input from relevant agencies with expertise in the subject area, and consideration of factors of environmental, economic, social, legal, and technological suitability of the proposed activity and its alternatives. Use of this word includes, but is not limited to, the concept of reasonableness and likelihood of success in achieving the project goal or purpose. "Feasible alternatives" applies both to locations or sites and to methods of design or construction, and includes a "no action" alternative.

(25) GAPC (Geographic Areas of Particular Concern) - areas within South Carolina’s coastal zone which have been identified in the State’s Coastal Management Program as being of such importance as to merit special consideration during the Department review of permit applications. GAPCs consist of: (1) areas of unique natural resource value; (2) areas where activities, development, or facilities depend on proximity to coastal waters, in terms of use or access; and (3) areas of special historical, archeological, or cultural significance.

(26) Garage - a structure built and used for the purpose of parking and protecting vehicles. The structure may be open or enclosed. An open parking area under a habitable structure will not be counted when computing the square footage of a habitable structure.

(27) Groin - a structure designed to stabilize a beach by trapping littoral drift. Groins are usually perpendicular to the shore and extend from the shoreline into the water far enough to accomplish their purpose. Groins are narrow and vary in length from less than one hundred (100) feet to several hundred feet. Groin fields are a series of two (2) or more groins which, because of their proximity to each other, have overlapping areas of influence. Consequently, the entire groin field must be considered as one system in order to accurately analyze beach response. The following is a list of the existing groins and groin fields in South Carolina as of 1991.

LIST OF EXISTING GROINS AND GROIN FIELDS IN SOUTH CAROLINA AS OF 1991

Garden City:

1. 	Six (6) groins south of the intersection of Yucca Street and Waccamaw Drive.

2. 	Two (2) groins south of the intersection of Dolphin Street and Waccamaw Drive.

Pawleys Island:

3. 	Twenty-three (23) groins along an area south of the northern causeway.

4. 	One (1) groin at the north end of the Island.

Isle of Palms:

5. 	One (1) groin at the north end of the Island along Dewees Inlet.

6. 	Two (2) groins at 42nd and 44th Avenues.

Sullivans Island:

7. 	Six (6) groins adjacent to Breach Inlet.

Folly Beach:

8. 	Forty-seven (47) groins.

Edisto Island:

9. 	Thirty-two (32) groins from the State Park south to Mikell Street.

10. 	Two (2) groins at Louise and Bailey Streets, along the South Edisto River.

Hunting Island:

11. 	One (1) groin at the north end of the Island.

Fripp Island:

12. 	One (1) groin at the north end of the Island.

13. 	Five (5) groins along the southern end of the Island.

Hilton Head Island:

14. 	Seventeen (17) groins in an area adjacent to Port Royal Sound.

15. 	Two (2) groins at the north end of Forest Beach, north of Yucca Drive.

16. 	Three (3) groins at Braddock Point, northwest of Merganser Court.

17. 	One (1) groin at Land’s End, adjacent to Braddock Cove.

(28) Habitable Structure - a structure suitable for human habitation including, but not limited to, single or multi-family residences, hotels, condominium buildings, and buildings for commercial purposes. Each building of a condominium regime is considered a separate habitable structure, but if a building is divided into apartments, then the entire building, not the individual apartment is considered a single habitable structure. Additionally, a habitable structure includes porches, gazebos, and other attached improvements.

(29) Inlet Erosion Zone - a segment of shoreline along or adjacent to tidal inlets which is directly influenced by the inlet and its associated shoals.

(a) Unstabilized Inlets - inlets that have not been stabilized by jetties, terminal groins, or other structures.

(b) Stabilized Inlets - inlets which are stabilized by jetties, terminal groins, or other structures.

(30) Jetty - a structure that extends into the water to direct and confine river or tidal flow into a channel and to prevent or reduce shoaling of the channel by littoral material. Jetties are constructed for the purpose of stabilizing navigation channels.

(31) Joint Public Notice - a permit application public notice issued jointly between the Department and the United States Army Corps of Engineers or other agency and processed independently by the Department.

(32) Living Shoreline - A shoreline stabilization approach utilized in intertidal wetland environments that maintains, restores, and/or enhances natural estuarine processes through the strategic placement of native vegetation and/or use of green infrastructure as described in R.30-12.Q. Living shorelines promote wetland resiliency and water quality, and enhance the diverse intertidal habitat.

(33) Major Development Activity - any construction activity that is not a Minor Development Activity.

(34) Marinas - a marina is any of the following:

(a) locked harbor facility;

(b) any facility which provides fueling, pump-out, maintenance or repair services (regardless of length);

(c) any facility which has effective docking space of greater than two hundred fifty (250) linear feet or provides moorage for more than ten (10) boats;

(d) any water area with a structure which is used for docking or otherwise mooring vessels and constructed to provide temporary or permanent docking space for more than ten (10) boats, such as a mooring field; or

(e) a dry stack facility.

(35) Master Plan - a document or a map prepared by a developer or a city as a policy guide to decisions about the physical development of the project or community.

(36) Minor Development Activity - the construction, maintenance, repair, or alteration of any private pier or erosion control structure, the construction of which does not involve dredging.

(37) Nonwater-dependent - a facility which cannot demonstrate that dependence on, use of, or access to coastal waters is essential to the functioning of its primary activity.

(38) Normal Maintenance and Repair - work performed on any structure within the critical area as part of a routine and ongoing program to maintain the integrity of the structure provided that the structure is still generally intact and functional in its present condition and the work only extends to the original dimensions of the structure. See R.30-5(D).

(39) OCRM - the South Carolina Department of Health and Environmental Control’s Office of Ocean and Coastal Resource Management.

(40) Offshore Breakwater - a structure which is designed to protect an area from wave action, is generally built parallel to the shore, may or may not be submerged, and may be built singly or in series. Breakwaters may interfere with natural wave action and wave induced currents.

(41) Party - each person or agency named or admitted as a party or properly seeking and entitled to be admitted as a party, including a license or permit applicant.

(42) Planned Development - a development plan which has received local approval for a specified number of dwelling and other units. The siting and size of structures and amenities are specified or restricted within the approval. This term specifically references multi-family or commercial projects not otherwise referenced by the terms master plan or planned unit development.

(43) Planned Unit Development - a residential, commercial, or industrial development, or all three, designed as a unit and approved in writing by local government.

(44) Pool - a structure designed and used for swimming and wading.

(45) Primary Oceanfront Sand Dunes -	those dunes that constitute the front row of dunes adjacent to the Atlantic Ocean. For the purposes of establishing the jurisdictional baseline, the dune must have a minimum height of thirty-six (36) inches, as measured vertically from the seaward toe to the crest of the dune. The dune must also form a nearly continuous dune ridge for five hundred (500) shore parallel feet and may exhibit minimal breaks such as those resulting from pedestrian or emergency vehicle access points. This dune typically exhibits the presence of stable, native vegetation, and is not scarped, eroded, or overtopped by the highest predicted astronomical tides. However, this dune may be inundated by storm surge which normally accompanies major coastal storm events.

(46) Public Interest - As used within these rules and regulations, public interest refers to the beneficial and adverse impacts and effects of a project upon members of the general public, especially residents of South Carolina who are not the owners and/or developers of the project. To the extent that, in the opinion of the Department, the value of such public benefits is greater than the public costs embodied in adverse environmental, economic, and fiscal effects, a proposed project may be credited with net public benefits.

(47) Setback Area - the area located between the setback line and the baseline.

(48) Setback Line - the line landward of the baseline that is established at a distance which is forty (40) times the average annual erosion rate as determined by historical and other scientific means and adopted by the Department in the State Comprehensive Beach Management Plan. However, all setback lines shall be established no less than twenty (20) feet landward of the baseline, even in cases where the shoreline has been stable or has experienced net accretion over the past forty (40) years.

(49) Significant Dune - A dune located completely seaward of the setback line, which because of its size and/or location is necessary to protect the beach/dune system of which it is a part.

(50) Special Geographic Circumstances - physical characteristics and land uses of surrounding uplands and waters may warrant additional consideration toward dock sizes. Special Geographic Circumstances identified by OCRM include: tidal ranges of greater than six (6) feet; lots with greater than five hundred (500) feet of water frontage; and no potential access via dockage from the opposite side of the creek. At the discretion of Department staff, one or more of these circumstances may be applied to dock applications, which may allow up to an additional fifty (50) percent to what is allowed in R.30-12.A(2)(c).

(51) Standard Erosion Zone - a segment of shoreline which is subject to essentially the same set of coastal processes, has a fairly constant range of profiles and sediment characteristics, and is not directly influenced by tidal inlets or associated inlet shoals.

(52) Tidelands - all areas which are at or below mean high tide and coastal wetlands, mudflats, and similar areas that are contiguous or adjacent to coastal waters and are an integral part of the estuarine systems involved. Coastal wetlands include marshes, mudflats, and shallows and means those areas periodically inundated by saline waters whether or not the saline waters reach the area naturally or through artificial water courses and those areas that are normally characterized by the prevalence of saline water vegetation capable of growth and reproduction. Provided, however, nothing in this definition shall apply to wetland areas that are not an integral part of an estuarine system. Further, until such time as the exact geographic extent of this definition can be scientifically determined, the Department shall have the authority to designate its approximate geographic extent.

(53) Transmittal Form - the official form prepared by the agency with subject matter jurisdiction that is filed with the Division notifying it of a request by any person for a contested case hearing.

(54) Water-dependent - a facility which can demonstrate that dependence on, use of, or access to coastal waters is essential to the functioning of its primary activity.

(55) Waterfront property - For purposes of these regulations, waterfront property will generally be defined as upland sites where a straight-line extension of both, generally shore perpendicular, upland property lines reaches a navigable watercourse within one thousand (1,000) feet of the marsh critical line. Waterfront property may also be identified via an approved dock master plan where designated corridors differing from upland property line extensions are delineated.

R.30-2. Applying for a Permit.

Amend R.30-2.B(8) to read:

(8) When considered appropriate by the Department, additional information may be required.

(a) For major development activities this additional information may include, but is not limited to, a Stormwater Management Plan, approved freshwater wetland delineation, and cultural resource and endangered species survey.

(b) The plat or copy of a plat submitted for those activities subject to the Beachfront Management Act (S.C. Code Sections 48-39-270 through 350) shall show the location of the baseline and setback line, applicable to the subject property. The lines shall be derived from information available from the Department. The lines shall be part of the plat and sealed by a South Carolina Registered Land Surveyor and may not be placed on the application by anyone other than a South Carolina Registered Land Surveyor or a member of the Department staff.

(c) For pilot project permit applications, additional required information is set forth in R.30-13.S.

R.30-11. General Guidelines for All Critical Areas.

Amend R.30-11.D to read:

D. General Guidelines for Beaches and the Beach/Dune System: In addition to the provisions of the South Carolina Coastal Management Act of 1977, the policies of the South Carolina Coastal Management Program, and applicable rules and regulations, the Department shall base its decisions on activities in the beaches and beach/dune system critical areas on the findings and policies specified in S.C. Code Section 48-39-250 and Section 48-39-260 of the 1977 Coastal Zone Management Act, as amended, and the following:

(1) The Department shall discourage new construction in the beaches and beach/dune system critical areas and encourage beach preservation to maintain the natural processes and functionality and benefits of the beaches and the beach/dune system critical areas as defined in R.30-1(D) and S.C. Code Section 48-39-10.

(2) According to S.C. Code Section 48-39-250, the use of armoring in the form of hard erosion control devices to protect erosion-threatened structures adjacent to the beach has not proven effective and, in many instances, contributes to the deterioration and loss of the dry sand beach. The Department shall promote soft-solutions to erosion within the context of a policy of beach preservation and prevent the strengthening, enlargement and/or rebuilding of existing erosion control structures. Erosion control structures may be repaired if not destroyed more than the percentage allowed in S.C. Code Section 48-39-290(B)(2)(b)(iii). Repairs must be made with materials similar to those of the structure or device being repaired.

(3) The Department shall promote public access to the beaches of this state.

(4) The Department shall consider state and local comprehensive plans. No permit shall be issued which is inconsistent with the state plan, and all permits issued shall be consistent with local plans to the maximum extent practicable.

(5) The Department shall be guided by the prohibitions against construction contained in S.C. Code Section 48-39-290 and Section 48-39-300 which are based upon the conclusion that ill-planned development, whether habitable structures, recreational amenities, erosion control devices, or other manmade structures, will now and in the future adversely impact the fragile beaches and beach/dune system critical areas. These structures interfere with the natural system and impact the highest and best uses of the system. In order to protect the highest and best uses of the beaches and beach/dune system critical areas, the Department, in its management capacity, shall encourage minimal development therein.

(6) The destruction of beach or dune vegetation within the beaches and/or beach/dune system critical areas is prohibited unless there is no feasible alternative. When there is destruction of vegetation permitted seaward of the setback line, mitigation, in the form of planting new vegetation to rectify the destruction, is required as a permit condition. In no event shall any part of a building be constructed on a primary oceanfront sand dune.

Amend R.30-13. Specific Project Standards for Beaches and the Beach/Dune System, to read:

A. Normal Maintenance and Repair of Habitable Structures: Normal maintenance and repair of habitable structures is allowed without notice to the Department. See R.30-5(A)(10) and R.30-1(D)(33).

B. Construction of New Habitable Structures: If any part of a new habitable structure is constructed within the beaches and/or beach/dune system critical areas, the owner shall certify to the Department that construction meets the following requirements:

(1) The structure is located as far landward on the property as practicable.

(2) That portion(s) of the habitable structure seaward of the setback line is no larger than five thousand (5,000) square feet of heated space.

(3) A drawing has been submitted to the Department showing a footprint of the structure on the property, a cross section of the structure, and the structure’s relation to property lines and setback lines which affect the property.

(4) No erosion control structure or device is incorporated as an integral part of the habitable structure pursuant to S.C. Code Section 48-39-290.

(5) No part of the building is being constructed on the primary oceanfront sand dune or on the active beach. The Department may grant a special permit to construct a habitable structure seaward of the baseline pursuant to R.30-15.

(6) When required, all mitigation meets the standards of the Mitigation Policy adopted as part of the State Beachfront Management Plan.

C. Additions to Habitable Structures: Additions located wholly or partially within the beaches and/or beach/dune system critical areas are only allowed provided the following requirements are met:

(1) The additions together with the existing structure do not exceed five thousand (5,000) square feet of heated space seaward of the setback line.

(2) Additions to habitable structures comply with the conditions of new habitable structures as set forth in R.30-13(B).

(3) The additions must be located no farther seaward than the existing structure, i.e. must be landward or upward of the existing structure. The linear footage of the structure, parallel to the coast, cannot be increased.

(4) Additions constructed totally landward of the beaches and beach/dune system critical areas do not require any notice to the Department.

(5) No part of the building is being constructed on the primary oceanfront sand dune or on active beach. The Department may grant a special permit to make additions to a habitable structure seaward of the baseline pursuant to R.30-15.

D. Repair and Renovation of Habitable Structures: Repair and renovation of a habitable structure located wholly or partially within the beaches and/or beach/dune system critical areas, damaged but not destroyed beyond repair due to natural or man-made causes, is allowed after notice and written documentation to the Department, provided no construction is on active beach.

E. Replacement or Rebuilding of Habitable Structures: A habitable structure located wholly or partially within the beaches and/or beach/dune system critical areas, which has been destroyed beyond repair due to natural causes, may be replaced or rebuilt provided all of the following requirements are met and after notice and written documentation to the Department:

(1) The total square footage of the replaced structure seaward of the setback line does not exceed the total square footage of the original structure seaward of the setback line.

(2) The linear footage of the replaced structure parallel to the coast does not exceed the original linear footage parallel to the coast.

(3) The replaced structure is no farther seaward than the original structure.

(4) Where possible, the replaced structure is moved landward of the setback line or, if not possible, then as far landward as practicable, considering local zoning and parking regulations.

(5) The reconstruction is not seaward of the baseline unless permitted elsewhere in S.C. Code Section 48-39-250 through 48-39-360 and the implementing regulations.

(6) Replacement of a habitable structure destroyed beyond repair due to man-made causes is allowed provided the rebuilt structure is no larger than the original structure it replaces and is constructed as far landward as possible, but the new structure must not be farther seaward than the original structure.

(7) No part of the building is constructed on the primary oceanfront sand dune or on active beach.

F. Landscaping, Earthmoving, and Fill for Landscaping: Within the beaches and/or beach/dune system critical areas, the installation of materials and associated amenities, moving of earth and placing of fill to accomplish these installations are allowed provided all of the following requirements are met:

(1) A comprehensive landscaping plan is submitted to and approved in writing by the Department;

(2) The construction of a retaining wall which extends below existing grade will not be allowed;

(3) No sand from the beach shall be used as backfill;

(4) No native plant material growing on the frontal dunes may be disturbed unless it can be demonstrated that the condition of the dune will be improved;

(5) Only native salt tolerant plant species may be planted on dunes and shall be approved by the Department staff;

(6) Adequate measures shall be taken to contain fill and irrigation runoff;

(7) Construction shall not alter or impact existing primary oceanfront sand dunes and shall not occur on active beach; and

(8) All work shall be in compliance with applicable local ordinances.

G. Fences, Lighting, Trash Receptacles, Sidewalks, and Signs. Within the beaches and/or beach/dune system critical areas, the placement, maintenance and repair, and replacement of fences, lighting, trash receptacles, sidewalks, and signs are allowed provided all of the following requirements are met:

(1) Construction shall not alter or impact existing sand dunes, dune vegetation, or the beach;

(2) New sidewalks may not exceed six (6) feet in width. New residential or private sidewalks must be constructed of wood. Existing concrete sidewalks attendant to public streets may be replaced within their original footprint;

(3) Trash receptacles (not dumpsters) may be attached to access ways or placed on the beach when the local government determines there is a need for such receptacles;

(4) Signs are limited to only those attached to attendant structures or mail receptacles or informational signs deemed necessary by federal, state, or local government for public health and safety. Advertisements are not allowed except on the walls or roofs of commercial structures;

(5) No fence may be used as a retaining wall; and

(6) Any additional lighting within the beaches and/or beach dune system critical areas shall be designed to shield the beach from illumination.

H. Emergency Vehicle Access Ways, Small Wooden Decks, Gazebos, and Other Structures Which Enhance Beach Access. Within the beaches and/or beach/dune system critical areas, the placement, maintenance and repair, and replacement of emergency vehicle access ways, decks, gazebos, and other structures which enhance beach access are allowed provided all of the following requirements are met:

(1) Emergency vehicle access ways shall:

(a) Be constructed at sites which preclude alteration of existing sand dunes and dune vegetation to the maximum extent practicable;

(b) Be constructed above the existing grade except for points of entry and exit;

(c) Be constructed of wood or other approved material;

(d) Be located at least one-half (1/2) mile from any other vehicle access to the beach unless, after review by the Department, this provision is determined to be unreasonable due to site-specific circumstances concerning health and safety needs;

(e) Be approved by the local government with jurisdiction; and

(f) Provide for pedestrian access use.

(2) Small wooden decks are allowed provided the following criteria are met:

(a) These structures must be constructed of wood or other Department-approved material;

(b) Roofs will be allowed;

(c) These structures must not exceed one hundred forty-four (144) square feet inclusive of an associated walkway (this square footage is not included in the five thousand (5,000) square-foot limitation on habitable structures);

(d) Only one of these structures per lot is permitted unless a limit of one would cause an unnecessary hardship as determined by the Department;

(e) These structures may not be constructed on the active beach or over primary oceanfront sand dunes, and if they ever become situated on the active beach they must be removed;

(f) These structures may be attached to the habitable structure provided they are not made an integral part of the habitable structure; and

(g) These structures may not be enclosed or screened.

I. The Construction and/or Repair of Drives and Parking Lots. Within the beaches and/or beach/dune system critical areas, the construction and/or repair of drives and parking lots is allowed provided all of the following requirements are met:

(1) On front row lots, new driveways and/or parking lots shall not extend seaward of habitable structures;

(2) Existing drives and/or parking lots may only be expanded on the landward side;

(3) No sand from the beach may be used during construction and/or repair;

(4) No alteration of the primary oceanfront sand dune or its dune vegetation is allowed;

(5) At the Department’s discretion, a Stormwater Management Plan may be required;

(6) The work shall comply with applicable local ordinances;

(7) Best Management Practices (BMPs) such as hay bales, silt fences, mulches, or other appropriate measures shall be used as necessary during the construction phase to prevent sedimentation reaching adjacent waters and wetlands. Upon project completion the disturbed areas shall be stabilized as soon as possible with grass or other appropriate vegetative cover; and

(8) No new driveway or parking lot shall be constructed seaward of the baseline unless a special permit as provided in S.C. Code Section 48-39-290(D) is obtained. In no circumstance shall a new driveway or parking lot be constructed on active beach.

J. Installation or Repair of Underground and Overhead Water, Sewer, Gas, Electrical, Telephone Lines and Cable Service Lines. Within the beaches and/or beach/dune system critical areas, the placement, maintenance, repair, and replacement of service lines are allowed provided the following requirements are met:

(1) All service lines, with the exception of subsea cables pursuant to R.30-15.F, shall be located as far landward as possible on each individual lot, and shall not be placed within active beach;

(2) Lines, junction boxes, poles, and accessory features will be relocated landward as far as possible in the event there is a need for replacement;

(3) Dunes allowed to be altered during construction shall be reconfigured and revegetated to preconstruction conditions;

(4) All work shall be in compliance with applicable local ordinances; and

(5) A comprehensive plan for new or replacement utilities shall be approved in writing by the Department.

K. Drainage Structures. Within the beaches and/or beach/dune system critical areas the placement, maintenance and repair, and replacement of drainage structures are allowed provided the following requirements are met:

(1) For new construction, the structures shall be part of the Department-approved Stormwater Management Plan or drainage plan which must be submitted either prior to or at the time the permit application is submitted.

(2) The replacement of drainage structures shall not involve an increase in the size of the existing structures, unless this change is approved by the Department as a part of the Stormwater Management Plan or drainage plan.

(3) Any disturbance to the dunes and dune vegetation shall be restored to pre-project conditions as soon as possible, and the restoration shall be approved by the Department staff.

(4) The drainage structure shall comply with the local drainage plan for the area within the beaches and/or beach/dune system critical areas.

(5) New drainage structures may be placed on the beach only if:

(a) Existing structures are eliminated; and

(b) No feasible alternative exists.

(6) All work shall be in compliance with applicable local ordinances.

(7) Areas disturbed during construction shall be revegetated to the Department’s satisfaction.

L. Sand Fences, Minor Beach Renourishment, and Dune Revegetation. In an effort to provide beachfront property owners with passive, low-cost dune stabilization methods, the placement, maintenance and repair, and replacement of sand fencing, dune revegetation, and minor renourishment may be allowed within the beaches and/or beach/dune system critical areas under the following conditions (Note: These steps may not be viewed as being undertaken for erosion control but rather as dune enhancement and stabilization measures. Since a broad beach and a healthy dune provide a storm buffer, these methods should aid the natural processes affecting the beaches and beach/dune system.)

(1) Sand fencing requirements:

(a) The fence material shall be biodegradable.

(b) The fences shall be installed according to plans established by the Department staff.

(c) The fences shall be installed in a manner so as not to impede turtle nesting. The Department may require sand fences be moved or removed entirely if the fences are found to impact turtle nesting activities or, in the Department’s opinion, have the potential to impact turtle nesting activities.

(d) The fence shall be placed above the highest up rush of the waves as determined by the Department staff.

(e) The fencing shall not impede public access.

(f) The fence shall be installed with the understanding that this is a temporary measure.

(g) If fence material is damaged, debris shall be removed expeditiously from the beach area by the owner.

(h) If the Department determines that the fence has a detrimental impact to the beaches and/or beach/dune system, it shall be removed by the owner as directed by the Department.

(2) Revegetation requirements: Property owners are encouraged to plant vegetation as a means of stabilizing oceanfront dunes. The roots of plant material tend to bind sand to dunes, while plant foliage serves to trap wind-blown sand. Suggested plant varieties include, but are not limited to, American beach grass (Ammophila breviligulata), bitter panicum (Panicum amarum), and sea oats (Uniola paniculata).

(a) Vegetation may be planted any time during the year and shall be planted, irrigated and fertilized according to nursery instructions or the Department’s recommendations.

(b) The Department staff shall inspect the site first and determine that there is a need for vegetative stabilization.

(3) Minor renourishment is allowed in an attempt to build and maintain healthy dunes. Minor renourishment requirements are:

(a) Sand shall be compatible in size and grain color, shall be from an upland source, and its use approved in writing by the Department staff.

(b) Minor renourishment shall be performed between November 1 and May 15.

(c) The Department staff shall inspect the site and establish that there is a need for the project.

(d) All projects shall be in compliance with applicable local ordinances.

M. Returning Sand to the Beaches and Beach/Dune System. Within the beaches and/or beach/dune system critical areas, sand that has drifted out of these critical areas may be returned to them if the following requirements are met:

(1) A description of the proposed work shall be submitted to and approved in writing by the Department prior to any initiation of work; and

(2) Work is pursuant to R.30-13(L)(3).

N. Erosion Control.

(1) Jetties and offshore breakwaters interfere with the natural transport of sediment and therefore require special permits. They shall only be permitted after thorough analysis of the project demonstrates that there will be no negative effect on adjacent areas. The following standards shall apply:

(a) A bond may be required to ensure that necessary remedial steps are taken to alleviate any adverse effects on adjacent areas caused by the installation of these structures. These remedial steps may include redesign and reconfiguration of the structures or even complete removal.

(b) A monitoring plan to assess post-project impact on adjacent areas must be approved by the Department prior to the issuance of a permit.

(c) Construction activities shall be scheduled so as not to interfere with nesting and brood-rearing activities of sea birds, sea turtles, or other wildlife species.

(d) Where feasible, jetties shall be designed to provide public recreational fishing opportunities.

(e) The applicant must have written approval from the local government which has jurisdiction in the area where the project is proposed.

(2) Protection of Beaches and Artificial Beach Nourishment: The following requirements apply to the Department’s consideration of projects for the renourishment of beaches:

(a) Careful study shall be given to the type (grain size and quality) of material most suitable for nourishment of a particular beach area;

(b) Borrow areas and sand for artificial nourishment shall be carefully selected to minimize adverse effects. Where possible, artificial beach nourishment shall be performed in concert with inlet stabilization or navigation projects;

(c) Dredging in the borrow areas shall not be in conflict with spawning seasons or migratory movements of significant estuarine or marine species. Nourishment of beach areas shall be scheduled so as not to interfere with nesting and brood-rearing activities of sea birds, sea turtles, or other wildlife species; and

(d) All policies concerning dredging and filling cited at R.30-12(G) shall be applied to beach nourishment proposals.

(3) Erosion Control Structures or Devices.

(a) No new erosion control structures or devices are allowed within the beaches and/or beach/dune system critical areas except to protect a public highway which existed as such on June 25, 1990.

(b) No erosion control structures or devices may be incorporated as an integral part of a habitable structure.

(c) Erosion control structures or devices must not be enlarged, strengthened, or rebuilt but may be maintained in their present condition if not destroyed more than the percentage allowed in S.C. Code Section 48-39-290(B)(2)(b)(i), (ii), and (iii). Repairs must be made with materials similar to those of the structure or device being repaired.

(d) Erosion control structures or devices determined to be destroyed more than the percentage allowed in S.C. Code Section 48-39-290(B)(2)(b)(i), (ii), and (iii) must be removed at the owner’s expense. Nothing in this section requires the removal of an erosion control structure or device which existed on July 1, 1988, that protected a public highway.

(e) Erosion control structures or devices which existed on June 25, 1990, must not be repaired or replaced if destroyed:

(i) more than eighty (80) percent above grade through June 30, 1995;

(ii) more than sixty-six and two-thirds (66 2/3) percent above grade from July 1, 1995, through June 30, 2005.

(iii) more than fifty (50) percent above grade after June 30, 2005. [See R.30-14(D)(3)(c) and (d) for damage assessment.]

O. Sand Dune Management.

(1) Walkways over dunes, as provided in S.C. Code Section 48-39-130(D), shall meet the following requirements:

(a) All components must be constructed of wood or other Department-approved material;

(b) Have a maximum width of six (6) feet;

(c) Conform with the contour of the dunes with a two (2)-foot vertical clearance between the surface of the dune and top of the walkway;

(d) Displace no sand in a critical area;

(e) Be constructed with as little environmental damage as possible;

(f) Not be located within fifty (50) feet of another walkway on the same parcel of property;

(g) Be limited to no more than one (1) of these structures per lot unless a limit of one (1) would cause an unnecessary hardship as determined by the Department; and

(h) Be shore perpendicular, except as necessary for disability access.

(2) Projects to protect, restore, or build dunes shall conform to the following standards:

(a) The use of natural beach vegetation to trap wind-blown sand is encouraged. Where pedestrian traffic has destroyed natural vegetation, the use of temporary sand fencing or its equivalent may be permitted.

(b) The construction of a dune by using beach sand and mechanical equipment shall be permitted only for restoration after unusual damage, such as that caused by a hurricane.

(c) Artificial dunes shall not be constructed seaward of the normal spring high-tide line.

(d) Any artificially constructed dunes shall be aligned to the greatest extent possible with existing dune ridges and shall be of the same general configuration as adjacent dunes.

P. Nonwater-dependent Structures. Nonwater-dependent structures, including, but not limited to, residences, restaurants, motel/hotel facilities, other commercial activities, and parking facilities, have been constructed in the past within the beach/dune system. The siting of new nonwater-dependent structures seaward of the baseline is prohibited unless a special permit is obtained pursuant to S.C. Code Section 48-39-290(D) and R.30-15(F).

Q. Golf Courses.

(1) Golf Courses are allowed seaward of the baseline because they can adjust to a changing shoreline more readily than other types of land uses. The use of sandbags is allowed as temporary protection for golf courses located seaward of the baseline if the golf course existed prior to May 24, 1991, and if the emergency condition conforms with the definition of emergency in S.C. Code Section 48-39-10(U), and in accordance with R.30-15.H(1). Sand scraping or renourishment may be used as temporary protection for golf courses in accordance with R.30-15.H(4) and (5).

(2) Specific standards which shall apply are as follows:

(a) Leveling or damaging of dunes or dune fields is prohibited.

(b) Golf courses should be located as far landward as practicable to minimize encroachment into the setback area.

(c) Any lighting within the beaches and/or beach/dune system critical areas must be low intensity and adequately shielded to prevent impact on sea turtle nesting.

(d) Measures must be taken to protect the integrity of the primary oceanfront sand dune from foot traffic. These measures may include:

(i) courses designed in a manner that will minimize adverse effects on the sand dunes;

(ii) physical barriers such as sand fencing placed at the landward trough of the dune;

(iii) certain types of vegetation that would discourage pedestrian traffic; and/or

(iv) any other measures the Department may deem necessary.

R. Fishing Piers:

(1) New fishing piers are allowed seaward of the baseline in order to provide public access to our coastal resources.

(2) Specific standards which shall apply to the construction of new piers or the alteration of existing piers are as follows:

(a) New piers must be dedicated to public use. A reasonable fee may be charged to the public but the general public may not be excluded from use.

(b) No restaurant, arcade, or other nonwater-dependent structure shall be placed on the pier seaward of the baseline or over the active beach.

(c) The height of the pier stringers and spacing of the piles must be able to accommodate vehicular traffic associated with emergency operations and renourishment projects.

S. Pilot Projects.

(1) Any person wishing to utilize or alter the beaches or beach/dune system critical areas as part of a pilot project qualifying under S.C. Code Section 48-39-320(C) must receive a pilot project permit from the Department. The Department may consider pilot projects that are intended to study the efficacy of any new technology, methodology or structure that has not previously been tested in South Carolina, is not regulated elsewhere under South Carolina statute or regulation, and is reasonably anticipated to be successful in addressing an erosional issue in the beaches or beach/dune system critical areas. According to S.C. Code Section 48-39-250, the use of armoring in the form of hard erosion control devices to protect erosion-threatened structures adjacent to the beach has not proven effective and, in many instances, contributes to the deterioration and loss of the dry sand beach. The installation of new erosion control structures or devices as part of a pilot project will not be permitted.

(2) Because pilot projects may have unknown risks, in addition to the requirements of R.30-2, the applicant for a pilot project must provide the following information, and any other information that the Department determines necessary, before a pilot project permit application is considered complete:

(a) Detailed study design and purpose that includes, but is not limited to:

(i) Documentation of the erosional issue at the study area;

(ii) Study methodology, including how the project is expected to address the erosional issue;

(iii) Location of the study including anticipated boundary of potential impacts;

(iv) Established control site(s);

(v) List of materials and specifications to be deployed or installed;

(vi) Timeline of the project, including end date of the study;

(vii) Anticipated outcomes;

(viii) Previous findings of any existing or similar technology, methodology, or structure that has been implemented in other areas/states; and

(ix) Strategy for removal and restoration, if required, after a storm event, at the conclusion of the study, and/or if the Department requires its termination pursuant to R.30-13.S(3)(g).

(b) Monitoring plans. Applications for pilot projects must be accompanied by a detailed monitoring plan. The monitoring plan must be approved by the Department prior to permit issuance and shall become a condition of the permit. Monitoring shall be required for the life of the pilot project. Monitoring plans must include, but are not limited to, photographs, data, and information necessary to measure and evaluate pre- and post-site conditions at pilot and control sites and adjacent areas.

(c) Applications must include information that demonstrates coordination with federal, state, and local entities. This coordination in no way affects or limits the ability of these entities to comment on the entire permit application before the Department. The applicant must have written acknowledgement from the local government which has jurisdiction in the area where the project is proposed.

(3) The following standards, along with any special conditions that may be appropriate, shall apply to pilot projects:

(a) Any construction activities associated with the pilot project shall be scheduled so as not to interfere with nesting and brood-rearing activities of shorebirds, sea turtles, or other wildlife species;

(b) No part of the project can be constructed upon the primary oceanfront sand dune and construction activities shall not adversely impact the primary oceanfront sand dune;

(c) Installation of seawalls, bulkheads, or revetments is not allowed as part of a pilot project;

(d) No non-native or invasive species shall be allowed as part of a pilot project. Only native beach vegetation may be used as part of a pilot project and must be approved by the Department. Approval may require the applicant to submit a certified letter from the supplier of the source material.

(e) Pilot projects shall avoid adverse effects to flora, fauna, and physical and aesthetic resources to the maximum extent practicable;

(f) Modifications to the project design, materials, or other aspects of the pilot project must be submitted, evaluated, and approved by the Department prior to implementation. The Department may require a formal permit amendment and public notice depending on the scope of the proposed modifications.

(g) The Department may require the termination of the pilot project and/or the removal of pilot project materials, and/or require restoration of impacted critical areas, for reasons that include, but are not limited to:

(i) deployed materials are no longer generally intact and functional;

(ii) deployed materials have resulted in marine debris;

(iii) the pilot project impedes navigation or public use of state lands and waters;

(iv) the pilot project timeline has concluded;

(v) the pilot project has resulted in material harm to flora, fauna, or physical or aesthetic resources; or if

(vi) an adjacent or downdrift community or property owner demonstrates to the Department that the pilot project caused or is causing an adverse impact.

(h) The applicant for a pilot project shall provide a financially binding commitment, such as a performance bond or letter of credit that is reasonably estimated to cover the cost of removal of the project, and/or restoration of the affected beach and/or beach dune system as approved by the Department. The financially binding commitment shall authorize the Department to utilize these financial commitments in the event that the permittee is required to remove the project and/or restore the affected area and fails to do so.

(4) Upon completion of the pilot project, the permittee must submit a detailed final report to the Department.

(a) The final report must be submitted within ninety (90) calendar days of the project end date and include, but is not limited to, photographs, data, analysis, and information pertaining to the following:

(i) pre- and post-site conditions at pilot and control sites and adjacent areas;

(ii) impacts of the project to flora, fauna, or physical or aesthetic resources;

(iii) impacts to adjacent and downdrift properties;

(iv) the pilot project’s effects on the erosional issue in the beaches and/or beach/dune system critical areas; and

(v) any other information the Department determines is necessary to evaluate the success of the pilot project.

(b) The Department will review the final report and shall render a final determination regarding the success of the pilot project in addressing an erosional issue in a beach and/or beach/dune system critical area. The Department’s determination of success will include an assessment of both the beneficial effects and adverse impacts of the project. If the Department determines that the pilot project is successful, the Department may authorize the use of the studied technology, methodology, or structure at the study site or additional locations through a critical area permit or emergency order. If the Department determines that the pilot project is unsuccessful, it must be removed within thirty (30) calendar days of the final decision.

R.30-15. Activities Allowed Seaward of Baseline.

Amend R.30-15.H to read:

H. Emergency Orders: Emergency situations before or after a storm event may prompt the Department, or an appointed official of a county or municipality or of the state to issue emergency orders under R.30-5, allowing property owners to construct temporary barriers against wave uprush. A structure is determined to be in imminent danger when the erosion comes within twenty (20) feet of that structure. If the Department determines that extenuating circumstances related to site conditions exist, the Department, at its discretion, may allow the use of emergency orders when erosion is beyond twenty (20) feet of the structure. In an effort to protect Loggerhead turtle nesting sites, emergency orders issued between April 15th and November 1st must be reviewed by the Department prior to actual performance of the activity authorized by the emergency order. The U.S. Army Corps of Engineers must be notified within seventy-two (72) hours of the issuance of an emergency order by the Department if the Department issued the emergency order. If the emergency order is issued by an appointed official such notification must be accomplished by the issuing official. The property owner or other recipient of the emergency order must obtain any additional permit(s) and agency review(s) that may be required by other local, state or federal agencies. All required permits and reviews must be obtained prior to the commencement of work pursuant to the issued emergency order. Unless otherwise approved by the Department, emergency sandbagging, sand scraping and renourishment shall be performed using the criteria established in this section. The Department may apply any requirements under this section to any Department-approved technology that is authorized under an emergency order.

(1) Emergency orders for sandbags may be issued by the Department, or upon written notification to the Department by an appointed official of a county or municipality or of the state acting to protect public health and safety. Sandbags shall only be used to construct temporary protection for existing habitable structures and critical infrastructure if the Department or appointed official determines a structure to be in imminent danger and emergency conditions conform with the definition of emergency in S.C. Code Section 48-39-10(U), or as allowed in R.30-13.Q(1). In this section, “critical infrastructure” shall mean utilities, roadways and associated infrastructure necessary to provide for public health and safety, communication, and transportation.

(2) Emergency orders for sandbags shall be subject to the following process:

(a) The Department or an appointed official of a county or municipality or of the state may issue emergency orders for areas specifically included under a state emergency declaration or at the request of a local government or property owner.

(b) Within one hundred twenty (120) days of the issuance of an emergency order for sandbags, the property owner may provide the Department with evidence that their community has a feasible and financially viable renourishment plan for the affected area that is consistent with their approved Local Comprehensive Beachfront Management Plan.

(c) If the property owner has not provided the Department with an acceptable plan for renourishment within one hundred twenty (120) days of the issuance of an emergency order for sandbags, then the emergency order shall expire at the end of the one hundred twentieth (120th) day, and the sandbags shall be removed at the property owner’s expense.

(d) If the property owner’s plan is acceptable and calls for renourishment, then a renourishment permit application shall be submitted to the Department within eighteen (18) months of the issuance of the emergency order.

(i) If the Department approves the renourishment permit, sandbags shall be allowed to remain in place for up to twelve (12) months after the permit is issued to allow sufficient time for the project to be completed, but must be removed at the property owner’s expense prior to the placement of renourishment sand at the property, or at the end of the twelve (12)-month period, whichever occurs first.

(ii) If the Department denies the renourishment permit application, the sandbags shall be removed within ninety (90) days of the final agency decision, including all appeals, at the property owner’s expense.

(iii) If a renourishment permit application is not submitted to the Department within eighteen (18) months of the issuance of the emergency order, the emergency order shall expire at the end of the eighteenth (18th) month, and the sandbags shall be removed at the property owner’s expense.

(3) To maintain the temporary nature that is intended for the use of sandbags, the following criteria shall be used when issuing emergency orders for sandbags:

(a) The bags shall be commercially manufactured for the purpose of holding sand. One year from the effective date of these regulations, applicants must provide information demonstrating that all bags are specifically manufactured for use in a coastal environment, considering factors such as resistance to saltwater corrosion, UV degradation, and durability in erosive conditions, among other appropriate factors as determined by the Department. Biodegradable bags may be required if deemed appropriate by the Department.

(b) The bags, when filled, shall be a maximum size of approximately one (1) cubic yard.

(c) The bags may be placed no farther seaward than is necessary to protect the existing habitable structure, critical infrastructure or golf course qualified under R.30-13.Q(1). In no case may sandbags be used to protect a dune. Sandbags may not retard normal shoreline movement unless used to protect an existing habitable structure, critical infrastructure or golf course qualified under R.30-13.Q(1).

(d) All sandbags are to be placed parallel to the shoreline. Excavation shall not be allowed below existing beach grade. The toe of the sandbags shall not be buried during installation. At no time shall the sandbags be buried or covered with sand from non-natural processes, including sand scraping or renourishment.

(e) Sandbags shall generally be limited to a maximum height of six (6) feet above the beach. The sandbags shall be stacked at an angle no steeper than forty-five (45) degrees.

(f) The Department may consider site specific engineering reports which will improve the effectiveness of sandbag placement for site specific situations.

(g) Sandbag fill material must be from an upland source and compatible in grain size and color with the native beach sand and should contain no more than a minimal amount of organic material. Only clean sand may be placed in the bags.

(h) The property owner is responsible for the day-to-day maintenance of the sandbags to ensure that they remain in the location authorized by the emergency order, above grade and in good repair. The Department may allow sandbags which become buried or partially buried through natural processes to remain in place for the duration of the emergency order process as outlined in R.30-15.H, provided that the coordinates and number of bags have been surveyed, and written documentation has been provided to the Department. Failure to maintain the sandbags may result in the Department requiring the removal of the sandbags at the property owner’s expense.

(i) A copy of the issued emergency order shall be in the possession of anyone performing the placement of sandbags.

(4) Emergency orders for sand scraping may be issued by the Department, or upon written notification to the Department by an appointed official of a county or municipality or of the state acting to protect public health and safety. Sand scraping may be used to construct temporary protection if the Department or local official determines a structure to be in imminent danger and emergency conditions conform with the definition of emergency in S.C. Code Section 48-39-10(U). The following criteria shall be used when issuing emergency orders for sand scraping:

(a) Sand scraping may only be ordered and performed to protect existing structures. Sand scraping shall not be allowed in front of erosion control structures unless it can be proven that the erosion control structure is itself in danger of collapsing and is within ten (10) feet of the habitable structure.

(b) Sand scraping may be used to provide temporary protection for golf courses pursuant to the requirements of this subsection.

(c) Sand may only be scraped from the intertidal beach and only between extended property lines of the structure receiving the sand. The depth of scraping may not exceed one (1) foot below the existing beach level.

(d) Sand may be placed against an eroded scarp or to replace an eroded dune that is seaward of a threatened structure. The dune shall not exceed six (6) feet above grade or twenty (20) feet in width as measured from dune toe to dune toe.

(e) No sand may be placed landward of an existing, functional erosion control device.

(f) Sand scraping may be performed one (1) time only per property for each emergency order issued by the local official without prior approval by the Department.

(g) A copy of the issued emergency order shall be in the possession of anyone performing sand scraping.

(h) Sand scraping activities shall generally be accomplished through private or local funding unless a state of emergency is declared, then state funding is not precluded.

(5) Emergency orders for renourishment may be issued by the Department, or upon written notification to the Department by an appointed official of a county or municipality or of the state acting to protect public health and safety. Renourishment may be used to construct temporary protection if the Department or local official determines a structure to be in imminent danger and emergency conditions conform with the definition of emergency in S.C. Code Section 48-39-10(U). The following criteria shall be used when issuing emergency orders for renourishment:

(a) Renourishment sand must originate from an upland source and be approved by the Department as compatible in grain size and color with the native beach sand and should contain no more than a minimal amount of organic material.

(b) Sand placed on the beach must be located between the extended property lines of the property receiving the sand.

(c) Sand may be stabilized with sand fencing and beach vegetation pursuant to the permitting requirements in R.30-13.L.

(d) A copy of the issued emergency order shall be in the possession of anyone performing authorized renourishment activities.

(e) Renourishment activities conducted under an emergency order may be used to provide temporary protection for golf courses pursuant to the requirements of this subsection.

(f) Renourishment activities conducted under an emergency order shall generally be accomplished through private or local funding unless a state of emergency is declared, then state funding is not precluded.

Fiscal Impact Statement:

The Department estimates minimal additional cost incurred by the state or its political subdivisions as a result of the promulgation, approval, and implementation of these proposed amendments. The Department will use existing staff and resources to implement these amendments.

Statement of Need and Reasonableness:

The following presents an analysis of the factors listed in 1976 Code Sections 1-23-115(C)(1)-(3) and (9)-(11):

DESCRIPTION OF REGULATION: R.30-1., Statement of Policy; R.30-2., Applying for a Permit; R.30-11., General Guidelines for All Critical Areas; R.30-13., Specific Project Standards for Beaches and the Beach/Dune System, and R.30-15., Activities Allowed Seaward of the Baseline.

Purpose: These amendments are proposed to improve the Department’s implementation of the state’s policy of beach preservation, as adopted under the 2018 Beachfront Management Reform Act (Act 173 of 2018). Although the state has been utilizing beach preservation approaches, like beach nourishment, to manage the beaches and beach/dune system critical areas since the 1980s, Coastal Division regulations currently lack a definition for beach preservation. Establishing a regulatory definition for beach preservation will provide guidance for regulatory staff to ensure projects within South Carolina beaches and beach/dune system critical areas uphold the state’s policy of beach preservation. The proposed regulatory definition was developed by the Workgroup, convened by the Department in 2022.

As part of the beach preservation discussion, the Workgroup examined pilot projects proposed within the beaches or beach/dune system critical areas. Pursuant to S.C. Code Section 48-39-320(C), pilot projects may be allowed if it is reasonably anticipated that the use will be successful in addressing an erosional issue in a beach or dune area.

Pilot projects may have unknown risks, which could impact species and habitat, the economic benefits associated with the beaches and beach/dune system critical areas, and public use of these resources. Proposed amendments, developed in coordination with the Workgroup, specify requirements and standards for pilot projects proposed within the beaches or beach/dune system critical areas. These requirements and standards will provide clarity for those wanting to utilize or alter the beaches or beach/dune system critical areas as part of a pilot project, improve the Department’s ability to properly evaluate pilot projects, and ensure their use does not conflict with the state’s policy of beach preservation.

South Carolina’s beaches are experiencing an increase in the rate of shoreline change which has resulted in beaches critical area being located landward of the state’s beach/dune system critical area. The Workgroup reviewed the effects of this shoreline change on regulated activities that are allowed within these beachfront critical areas, including habitable structures and erosion control structures. Based on the Workgroup’s recommendations and additional Departmental review, the proposed amendments were developed to provide for consistency in the application of standards for activities across the beaches and beach/dune system critical areas and to support the policy of beach preservation.

During the 2024 legislative session, in coordination with a subcommittee of the Senate Agriculture and Natural Resources Committee, additional amendments were included within R.30-1.D, 30-11.D, 30-13, and 30-15.H. These amendments add a definition for beaches that matches statutory language and further clarify the definition of beach preservation, statutory requirements associated with erosion control structures, specific project standards for beaches and the beach/dune system, and pilot project language. Additionally, the amendments clarify regulations related to emergency orders and the use of sandbags seaward of the baseline.

Legal Authority: 1976 Code Sections 48-39-10 et seq.

Plan for Implementation: The amendments will take legal effect upon General Assembly approval and upon publication in the State Register. Department personnel will then take appropriate steps to inform the regulated community of the amendments. Additionally, a copy of the regulation will be posted on the Department’s website, accessible at www.scdhec.gov/regulations-table. Printed copies may also be requested, for a fee, from the Department’s Freedom of Information Office.

DETERMINATION OF NEED AND REASONABLENESS OF THE REGULATION BASED ON ALL FACTORS HEREIN AND EXPECTED BENEFITS:

The Department is charged with implementing the state’s policy of beach preservation, as established under the 2018 Beachfront Management Reform Act. Coastal Division regulations currently lack a regulatory definition for beach preservation, have insufficient requirements and standards associated with pilot projects proposed within the beaches or beach/dune system critical areas, and need consistency and clarity in implementing activities across beachfront critical areas. The Department proposes amending these regulations to provide a beach preservation definition, define requirements and project standards for pilot projects within the state’s beaches or beach/dune system critical areas, and to provide for consistency in the application of standards for activities across the beaches and beach/dune system critical areas.

The proposed amendments will provide additional guidance for regulatory staff ensuring future projects within the beaches and beach/dune system critical areas align with the state’s policy of beach preservation. The proposed amendments also provide the regulated community with more clarity on the requirements for pilot projects and other activities proposed within the beaches or beach/dune system critical areas. Additionally, the proposed amendments ensure regulatory staff have sufficient information to evaluate pilot projects in these critical areas. The proposed amendments are reasonable and necessary to manage and preserve the long-term health and sustainability of the state’s beaches and beach/dune system critical areas.

DETERMINATION OF COSTS AND BENEFITS:

The Department anticipates minimal additional costs to the state resulting from the administration and processing of these proposed amendments. Benefits to the state would include improved management of coastal resources by creating a regulatory definition for beach preservation, identifying requirements and project standards for pilot projects, and providing consistency for implementing activities across beachfront critical areas. The Department does not anticipate additional cost to the regulated community as a result of the proposed amendments associated with beach preservation and the beaches critical area. The proposed amendments associated with pilot projects within the beaches and beach/dune system critical areas, establishing a permitting pathway for these activities, will be subject to the administrative fee structure outlined in R.61-30.G(13), Environmental Protection Fees.

UNCERTAINTIES OF ESTIMATES:

None.

EFFECT ON THE ENVIRONMENT AND PUBLIC HEALTH:

Implementation of the proposed amendments seeks to benefit the environment by providing more clarity to the Department’s statutory directives to manage the state’s beaches and beach/dune critical areas in a manner that promotes the policy of beach preservation.

DETRIMENTAL EFFECT ON THE ENVIRONMENT AND PUBLIC HEALTH IF THE REGULATION IS NOT IMPLEMENTED:

Not implementing these proposed amendments will result in continued challenges associated with the Department’s ability to properly implement the policy of beach preservation. Further, there will continue to be a lack of guidance for those wishing to utilize or alter the beaches or beach/dune system critical areas through the use of a pilot project, and there will continue to be a lack of project standards necessary for regulatory staff to sufficiently evaluate pilot projects sited within these critical areas. Finally, there will be continued inconsistencies in the application of standards for activities across the beaches and beach/dune system critical areas.

Statement of Rationale:

Here below is the Statement of Rationale pursuant to S.C. Code Section 1-23-110(A)(3)(h):

The Department is charged with implementing the state’s policy of beach preservation, as established under the 2018 Beachfront Management Reform Act. Coastal Division regulations currently lack a regulatory definition for beach preservation, lack requirements and project standards associated with pilot projects proposed within the beaches or beach/dune system critical areas, and need consistency in the application of standards for activities across the beaches and beach/dune system critical areas.

The Department proposes amending these regulations to provide a beach preservation definition, identify requirements and project standards for pilot projects that would utilize or alter the state’s beaches or beach/dune system critical areas, and provide for consistent application of standards within the beaches and beach/dune system critical areas. Recommendations and key findings from the Beach Preservation Stakeholder Workgroup were the primary basis for developing these proposed amendments.