Agency Name: Building Codes Council - Labor, Licensing and Regulation

Statutory Authority: 6-9-40, 6-9-50, and 6-9-55

Document Number: 5416

Proposed in State Register Volume and Issue: 49/10

House Committee: Reg., Admin. Procedures, AI and Cybersecurity

Senate Committee: Labor, Commerce and Industry Committee

110 Day Review Expiration Date for Automatic Approval: 05/03/2026

Final in State Register Volume and Issue: 50/5

Status: Final

Subject: International Plumbing Code

History: 5416

By	Date	Action Description	Jt. Res. No.	Expiration Date

-	10/24/2025	Proposed Reg Published in SR

-	01/13/2026	Received President of the Senate & Speaker		05/03/2026

S	01/13/2026	Referred to Committee

H	01/14/2026	Referred to Committee

H	04/02/2026	Committee Requested Withdrawal

110 Day Period Tolled

-	04/02/2026	Withdrawn and Resubmitted		05/03/2026

S	04/30/2026	Resolution Introduced to Approve	1168

-	05/03/2026	Approved by: Expiration Date

-	05/22/2026	Effective Date unless otherwise

provided for in the Regulation

Document No. 5416

DEPARTMENT OF LABOR, LICENSING AND REGULATION

BUILDING CODES COUNCIL

CHAPTER 8

Statutory Authority: 1976 Code Sections 6-9-40, 6-9-50, and 6-9-55

8-1400. International Plumbing Code.

Synopsis:

The South Carolina Building Codes Council proposes to amend Chapter 8, Article 14, to reflect modifications to the 2024 South Carolina Building Codes, the International Plumbing Code.

A Notice of Drafting was published in the State Register on June 27, 2025.

Instructions:

Print the regulation as shown below. All other items remain unchanged.

Text:

ARTICLE 14

INTERNATIONAL PLUMBING CODE

2024 International Plumbing Code Modification Summary

(Statutory Authority: 1976 Code Sections 6-9-40)

8-1400. International Plumbing Code.

NOTE - This article is based upon the International Plumbing Code, 2024 Edition, in accordance with the statutory amendments to acts governing the Building Codes Council, except for the modifications referenced below.

This code is identical to the 2024 International Plumbing Code, except for the following modifications:

8-1401. IPC Section 202 General Definitions.

Drinking fountain: A plumbing fixture that is connected to the potable water distribution system and the drainage system. The fixture allows the user to obtain a drink directly from a stream of flowing water without the use of any accessories. Such fixtures can be separate from or integral to a bottle-filling station.

Bottle Filling Station: A type of water dispenser that is connected to the potable water distribution system and the drainage system. The fixture is designed and intended for automatically or manually filling personal use drinking water bottles or containers not less than 10 inches (254 mm) in height and is in compliance with the American with Disabilities Act (42 U.S.C. § 12101 et seq.). Such fixtures can be separate from or integral to a drinking fountain and can incorporate a water filter and a cooling system for chilling the drinking water.

Water Cooler: A drinking fountain or bottle filling station that incorporates a means of reducing the temperature of the water supplied to it from the potable water distribution system.

Water Dispenser: A plumbing fixture that is automatic or manually controlled by the user for the purpose of dispensing potable drinking water into a receptacle such as a cup, glass or bottle. Such fixture is connected to the potable water distribution system of the premises. This definition includes a freestanding apparatus for the same purpose that is not connected to the potable water distribution system and that is supplied with potable water from a container, bottle or reservoir.

8-1402. IPC Table 403.1 Minimum Number of Required Plumbing Fixtures.

| No. | Classification | Description | Water Closets (Urinals: See Section 424.2) |  | Lavatories |  | Bathtubs/ Showers | Drinking Fountain See Section 410 |  | Other |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | M | F | M | F |  |  |  |  |
| 3 | Educational | Educational Facilities | 1 per 50 |  | 1 per 50 |  | – |  | 1 per 100 | 1 service sink |

8-1403. IPC 403.1.1 Fixture calculations.

To determine the occupant load of each sex, the total occupant load shall be divided in half. To determine the required number of fixtures, the fixture ratio or ratios for each fixture type shall be applied to the occupant load of each sex in accordance with Table 403.1. Fractional numbers resulting from applying the fixture ratios of Table 403.1 shall be rounded up to the next whole number. For calculations involving multiple occupancies, such fractional numbers for each occupancy shall first be summed and then rounded up to the next whole number.

Exception: The total occupant load shall not be required to be divided in half where approved statistical data indicate a distribution of the sexes of other than 50 percent of each sex.

8-1404. IPC 403.2 Separate facilities.

Where plumbing fixtures are required, separate toilet facilities shall be provided for each sex.

Exceptions:

1.	Separate toilet facilities shall not be required for dwelling units and sleeping units.

2.	Separate toilet facilities shall not be required in structures or tenant spaces with a total occupant load, including both employees and customers, of 15 or fewer.

3.	Separate toilet facilities shall not be required in mercantile occupancies in which the maximum occupant load is 100 or fewer.

4.	Separate toilet facilities shall not be required in business occupancies in which the maximum occupant load is 25 or fewer.

5.	Separate toilet facilities shall not be required to be designated by sex where single-user toilet rooms are provided in accordance with Section 403.1.2.

8-1405. IPC Section 410. 4 Substitution.

Where restaurants provide drinking water in a container free of charge, drinking fountains shall not be required in those restaurants. In other occupancies where three or more drinking fountains are required, water dispensers shall be permitted to be substituted for not more than 50 percent of the required number of drinking fountains.

Fiscal Impact Statement:

There will be no cost incurred by the State or any of its political subdivisions for these regulations.

Statement of Rationale:

The South Carolina Building Codes Council proposes to amend Chapter 8, Article 14, to reflect modifications to the 2024 South Carolina Building Codes, the International Plumbing Code.